3GPP TSG-RAN WG4 Meeting # 112                                                              R4-2411086

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	System parameters for new NR bands n87 and n88

Source: 	CATT

Agenda item:	7.15.2

Document for:	Approval

- Background

During RAN#104 meeting, new WI on introduction of NR bands n87 and n88 was approved in [1]. The core objective is to standardize two new NR FDD bands n87 and n88 in the 410MHz range.

  - E-UTRA band 87 (UL 410 - 415, DL 420 – 425 MHz)

  - E-UTRA band 88 (UL 412 - 417, DL 422 – 427 MHz)

In this paper, we provide system parameters for the two new NR bands.

- System parameters

The system parameters which need to be updated for the two new NR bands are shown as following.

  - Operating bands

Both NR and NB-IoT are supported on bands 87 and 88, so it’s straightforward that NR and NB-IoT are supported by bands n87 and n88

Table 1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- | --- |
| n87 | 410 – 415 | 420 – 425 | FDD |  |
| n88 | 412 – 417 | 422 – 427 | FDD |  |

Proposal 1: Both NR and NB-IoT are supported on bands n87 and n88.

  - BS channel bandwidth per operating band

Table 2: BS channel bandwidths and SCS per operating band in FR1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n87 | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n88 | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - Channel raster entries for each operating band

Table 3: Applicable NR-ARFCN per operating band in FR1

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n87 | 100 | 82000 – <20> – 83000 | 84000 – <20> – 85000 |
| n88 | 100 | 82400 – <20> – 83400 | 84400 – <20> – 85400 |

  - Synchronization raster entries for each operating band

Table 4: Applicable SS raster entries per operating band (FR1) for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n87 | 15 kHz | Case A | 1055 – <1> – 1057 |
| n88 | 15 kHz | Case A | 1061 – <1> – 1062 |

Table 5: Applicable SS raster entries per operating band (FR1) for 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n87 | 15 kHz | Case A | 28743 – <1> – 28754 |
| n88 | 15 kHz | Case A | 28752 – <1> – 28764 |

  - TX–RX frequency separation

Table 6: UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n87 | 10 MHz |
| n88 | 10 MHz |

- Summary

In this paper, we provide system parameters for the two new bands, we have the following proposal.

Proposal 1: Both NR and NB-IoT are supported on bands n87 and n88.

Proposal 2: The system parameters which need to be updated for new NR bands n87 and n88 are shown in Table 1 to Table 6.

- Reference

[1] RP-241663, “New WID on introduction of NR bands n87 and n88”, Nokia, RAN#104

[2] TS 38.101-1