3GPP TSG-RAN WG4 Meeting #112											             R4-2411090

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			8.2.3

Source:	CATT

Title:	Further discussion on IMT parameters for NR in 7125 to 8400MHz

Document for:	Discussion

# Introduction

For the LS from WP5D, RAN WG4 has already agreed the following estimated completion dates for the work:

•	4400 to 4800 MHz	Estimated date for completion: May 2024 (RAN WG4#111)

•	7125 to 8400 MHz	Estimated date for completion: August 2024 (RAN WG4#112)

•	14800 to 15350 MHz	Estimated date for completion: November 2024 (RAN WG4#113)

Following the above plan, the first reply LS on the first frequency range 4400 – 4800 MHz has been agreed in RAN4#111 [1], and in this meeting, the second reply LS on the frequency range 7125 – 8400 MHz is expected to be agreed.

In this contribution, we further discuss the corresponding parmaters on top of the consensus reached in previous meetings for this frequency range [2][3].

# Discussion

## General aspects

In RAN4#111, further way forward was agreed on top of the way forward in RAN4#110bis and for general aspects, the following issues were discussed and agreed [3]:

Sub-topic 2-1 General / system issues Issue 2-1: Duplex mode Agreement: - Do not include SBFD in the LS, but add information in the TR. Add a reference to the TR in the LS Issue 2-3: Typical channel bandwidth Agreement: - Mention 100MHz as typical bandwidth in the LS - FFS whether to mention wider bandwidths may be possible in future in the LS - Describe other possibilities for bandwidth in the TR, mentioning that additional specification effort needed for >100MHz Issue 2-4: Typical signal bandwidth Outcome of discussion (Not agreed): Option 1: 273 RB, 30k SCS as 38.104 Option 2: Quote formula of RBs * SCS without number of RBs Option 3: Directly quote the signal bandwidth in MHz Issue 2-6: Deployment scenarios Agreement: - Confirm Urban macro and urban micro should be considered - This is based on assumed antenna size that is sufficient for 450m or lower ISD for urban macro

Typical channel bandwidth

To confirm a feasibility of a wider channel bandwidth than 100MHz requires further analysis and works, however, due to the limited time as the second reply LS is expected to be agreed in this meeting, it is difficult to conclude the corresponding works, hence, we propose not to mention a channel bandwidth wider than 100MHz in the reply LS.

Proposal 1: Not to mention wider channel bandwidth than 100MHz in the second reply LS.

Typical signal bandwidth

For typical signal bandwidth, the listed three options are more or less equal, just a matter of representation. In our views, it’s better to keep an aligned style in all three reply LS. And in the first reply LS sent out already [1], Option 2 is used, therefore, we propose to quote the formula in the second reply LS on typical signal bandwidth.

Proposal 2: Quote formula of RBs * SCS without number of RBs in the reply LS for typical signal bandwidth.

## BS parameters

Sub-topic 2-2 BS parameters Issue 2-7: Emissions mask Agreement: - AAS BS: - Use n104 unwanted emissions mask - [Aim to use 100MHz for delta_obue, conclude feasibility until RAN4#112] - Non-AAS BS: - [n104 unwanted emissions mask for IMT response] - Double check on whether there are any concerns towards sharing by RAN4#112 and in case concerns are identified, consider n96 if needed to resolve concerns Issue 2-10: Noise figure Agreement: - Follow 38.820, i.e., 6dB WA, 11dB MR, 14dB LA for noise figure of BS Issue 2-11: Sensitivity - Proposals - Option 1: As for n104 in 38.104 (CMCC, Samsung, Nokia) Agreement: - Agree on option 1. - Capture it in the TR only. Issue 2-14: BS antenna array parameters Agreement: Rural macro (If it’s available) Parameters proposed by ZTE; Suburban macro; Urban macro; Urban small cell (outdoor)/Micro cell; Indoor (small cell). 1; Base station antenna characteristics. 1.6; Antenna array configuration (Row × Column) (Note 4); 8 × 8 elements; Option 1: 8 × 16 (Nokia, Ericsson, Huawei, Spark) Option 2: 8 x 8 (ZTE, Samsung); Option 1: 8 × 16 (Nokia, Ericsson, Huawei, Cable Labs) Option 2: 8 x 8 (ZTE, Samsung); 8 × 8; 4 × 4. 1.7a; Number of element rows in sub-array, Msub; Option 1: 4 (Nokia, Spark, Cable Labs, Charter) Option 2: 3 (Ericsson, Huawei, Samsung); Option 1: 4 (Nokia, Spark, Cable Labs, Charter) Option 2: 3 (Ericsson, Huawei, Samsung); N/A; N/A. Agreement: - Remove the rural macro scenario from the table. - non-AAS can be used for Urban small cell/Micro cell scenario - non-AAS can be used for indoor scenario - FFS on where to capture the above two bullets

Emission mask

The following table illustrates the current boundaries for n104, n96 and non-6GHz bands.

|  |  | n104 | Non-6GHz NR bands (larger than 200MHz and less than 900MHz bandwidth) |
| --- | --- | --- | --- |
| ΔfOBUE | 1-C | 40 MHz | 40 MHz |
|  | 1-H | 100 MHz | 40 MHz |
|  | 1-O | 100 MHz | 40 MHz |
| ΔfOOB | 1-C | 60 MHz | 60 MHz |
|  | 1-H | 100 MHz | 60 MHz |
|  | 1-O | 100 MHz | 60 MHz |

Considering the fact that the bandwidth of the frequency range 7125 – 8400 MHz is almost doubled as that for n104 (1275 MHzVs 700 MHz),  ΔfOBUE and ΔfOOB could be revisited for this frequency range assuming that the whole frequency range is to be defined as a single band, in particular for hybrid and OTA requirements. However, considering the limited time for this frequency range, we could accept to re-use n104 if no other issues identified.

Proposal 3:  Accept ΔfOBUE as 100 MHz considering the limited time and if no other issues identified.

Sub-array size

- DL throughput with 8 x 16 configuration

- DL throughput with 8 x 8 configuration

Fig. 1, initial coexistence simulation results for 8GHz with different sub-array sizes and array configurations

We have conduct some initial coexistence simulations on 8GHz with different sub-array sizes and array configurations, and the initial coexistence results are shown in Fig. 1. According to the results, it could be observed that the sub-array size does not have big impacts on DL throughputs, therefore, we could choose Option 2 for the subarray size.

Observation 1: The sub-array size does not have big impacts on DL throughputs with different array configurations.

Proposal 4: Set sub-array size as 3.

Antenna array configuration

For the convenience of comparison, we compare UL/DL throughputs with a sub-arry size of 3 for these two antenna array configurations (8 x 16, 8 x 8). If we look at 30dB ACIR, DL throughput difference is around 10%, while its cost is doubled number of antenna elements.

Fig. 2,  8x16 Vs 8x8 for Msub = 3

Observation 2: For antenna array configurations with 8 x 16 and 8 x 8, the DL throughput difference is around 10% at 30dB ACIR with a sub-array size of 3.

Proposal 5: Set antenna array configuration as 8 x 8.

## UE parameters

Sub-topic 2-3 UE parameters Issue 2-15: Maximum output power Tentative agreement: - Capture 23dBm as typical maximum output power in the LS to ITU. - Other output powers are not precluded and refer to TR - Capture 20dBm, 23dBm, 26dBm, and higher power classes in TR

It is reasonable to reply ITU with a typical UE maximum output power as 23dBm but capture other maximum output powers in TR, i.e., we can confirm the tentative agreements in previous meeting on UE maximum output power.

Proposal 6: Confirm the tentative agreements on UE maximum output power, i.e., 23dBm in the reply LS, and other power classes captured in TR.

# Conclusion

In this contribution, we have the following observations and proposals for replying the LS on the frequency range 7125 – 8400 MHz:

Observation 1: The sub-array size does not have big impacts on DL throughputs with different array configurations.

Observation 2: For antenna array configurations with 8 x 16 and 8 x 8, the DL throughput difference is around 10% at 30dB ACIR with a sub-array size of 3.

Proposal 1: Not to mention wider channel bandwidth than 100MHz in the second reply LS.

Proposal 2: Quote formula of RBs * SCS without number of RBs in the reply LS for typical signal bandwidth.

Proposal 3:  Accept ΔfOBUE as 100 MHz considering the limited time and if no other issues identified.

Proposal 4: Set sub-array size as 3.

Proposal 5: Set antenna array configuration as 8 x 8.

Proposal 6: Confirm the tentative agreements on UE maximum output power, i.e., 23dBm in the reply LS, and other power classes captured in TR.

# Reference

[1] R4-2410576, LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, Ericsson.

[2] R4-2406615, WF on IMT parameters for other frequency ranges, CATT.

[3] R4-2410741, WF on IMT parameters study, Ericsson.

[4]