3GPP TSG-RAN WG4 Meeting #112											             R4-2411091

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			8.2.4

Source:	CATT

Title:	Further discussion on IMT parameters for NR in 14.8GHz to 15.35GHz

Document for:	Discussion

# Introduction

RAN4 has sent out the first reply to ITU-R WP5D’s LS [1] on the frequency range 4400 – 4800 MHz on IMT parameters [2] according to the following agreed work plan:

- 4400 to 4800 MHz	Estimated date for completion: May 2024 (RAN WG4#111)

- 7125 to 8400 MHz	Estimated date for completion: August 2024 (RAN WG4#112)

- 14800 to 15350 MHz	Estimated date for completion: November 2024 (RAN WG4#113)

The frequency 14.8 GHz – 15.35 GHz requires a bit more efforts where coexistence studies and simulations are expected, in order to propose and justify corresponding IMT parameters. In this contribution, we provide some initial simulation results for the highest prioritized scenarios based on the previous WF [3][4] and propose part of parameters accordingly.

# Discussion

According to the latest draft TR 38.922 v0.1.0, the intended scenarios to be studied could be found in the following table [5]:

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Priority |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, TBD MHz | NR, TBD MHz | DL to DL | 15 GHz | Indoor hotspot | Second |
| 2 | eMBB | NR, TBD MHz | NR, TBD MHz | DL to DL | 15 GHz | Urban macro | First |
| 3 | eMBB | NR, TBD MHz | NR, TBD MHz | DL to DL | 15 GHz | Dense urban | Third |
| 4 | eMBB | NR, TBD MHz | NR, TBD MHz | UL to UL | 15 GHz | Indoor hotspot | Second |
| 5 | eMBB | NR, TBD MHz | NR, TBD MHz | UL to UL | 15 GHz | Urban macro | First |
| 6 | eMBB | NR, TBD MHz | NR, TBD MHz | UL to UL | 15 GHz | Dense urban | Third |

Scenario #2 and #5 are urban macro with the first priority. And the corresponding simulation assumptions are highlighted below:

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 450 m (first priority) 350 m (second priority) Other (third priority) |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20/0% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Grid shift |  | 0 / 100 % |  |
| BS antenna array |  | • Option 1: 2048 (Sub Array size 8) • Option 2: 1536 (Sub Array size 4) | 0.7 wave-length for vertical spacing |
| UE antenna configuration |  | - FR1 like: Tx element gain 0dBi, 0 dB diversity gain assuming 4Rx |  |
| Noise figure |  | BS: 8 dB UE: 11 dB |  |
| Maximum transmission power |  | BS: 43 dBm UE: 23dBm, FR1 like, |  |
| System bandwidth |  | 100 MHz |  |

- DL Throughput with different BS array size: 0% grid shift

- DL Throughput with different BS array size: 100% grid shift

Fig. 1, DL throughputs with different BS array size and grid shifts

Fig. 1 illustrates initial simulation results for downlink throughtputs difference between two different BS array configurations for 0% and 100% grid shift. It could be observed that the BS array configuration with 1536 elements and a sub-array size of 4 slightly outperforms the BS array configuration with 2048 elements and a sub-array size of 8, hence, the previous BS array configuration can be selected.

Proposal : Set BS array size as 1536 and the sub-array size as 4.

# Conclusion

In this contribution, we have the following proposal for IMT parameters for the frequency range 14.8 GHz – 15.35 GHz:

Proposal : Set BS array size as 1536 and the sub-array size as 4.

# Reference

[1] R4-2400333 Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz, ITU-R WP5D

[2] ] R4-2410576, LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, Ericsson.

[3] R4-2406615, WF on IMT parameters for other frequency ranges, CATT.

[4] R4-2410741, WF on IMT parameters study, Ericsson.

[5] R4-2410763, TR 38.922 v0.1.0, Ericsson