3GPP TSG-RAN WG4 Meeting #112											              R4-2411095

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			8.22.2.1

Source:	CATT

Title:	Further discussion on system parameters for LP-WUS

Document for:	Discussion

# Introduction

In RAN4#111, some conclusions were agreed on general and system parameters for LP-WUS [1]. In particular, it was agreed that RAN4 will further check whether the channel raster is needed for LP-WUR:

Issue 1-1-7: Channel raster for LP-WUR Agreements: RAN4 further check whether no channel raster is needed for LP-WUR.

In this contribution, we propose to introduce a separate channel raster for LP-WUR in order to ensure an LP-WUS carrier could be flexibly allocated within an NR carrier for those NR bands with 100kHz channel raster.

# Discussion

In order to make most of LP-WUS, at least the following two points should be satisfied from standardization perspective:

- PRB grid of an LP-WUS carrier should be aligned with that of its surrounding wider NR carrier;

- An LP-WUS carrier can be flexibly allocated within the wider NR carrier.

Obviously, for an NR band with SCS based channel raster, both could be fulfilled, hence, no channel raster is needed for LP-WUR for those NR bands.

Proposal 1: For NR bands with SCS based channel raster, no new channel raster is needed for LP-WUR.

However, for NR bands with 100kHz channel raster, it may be a bit more complicated, in particular for the second point above. Fig. 1 illustrates four different cases where an LP-WUS carrier is allocated within the PRB grid of the corresponding NR carrier (with 100kHz channel raster) based on the PRB numbers’ oddity of the two carriers.

- Fig. 1a indicates the case where the PRB number of the NR carrier is even while that of LP-WUS carrier is odd, i.e., Case 1.

- Fig. 1b indicates the case where the PRB number of the NR carrier is even while that of LP-WUS carrier is even, i.e., Case 2.

- Fig. 1c indicates the case where the PRB number of the NR carrier is odd while that of LP-WUS carrier is odd, i.e., Case 3.

- Fig. 1d indicates the case where the PRB number of the NR carrier is odd while that of LP-WUS carrier is even, i.e., Case 4.

Fig. 1, channel raster for WUS carrier

As shown in Fig. 1, the distance D between the LP-WUS carrier and NR carrier can be expressed as:

- q * SCS * 12 + 6 * SCS for Case 1 and 4;

- q * SCS * 12 for Case 2 and 3.

Where q is an integer.

The channel raster of LP-WUR, labelled as cr(t),  can be calculated as:

cr(t) = n*100kHz + q*SCS*12 + m*6*SCS           (1)

where n, q is an integer, and m = 0 or 1.

Let’s take 15kHz SCS as an example. The Eq. (1) is further expressed as:

cr(t) = n*100kHz + q*180kHz + m*90kHz           (2)

Actually, Eq. (2) can be simplified as:

cr(t) = n*100kHz +p * 90kHz           (3)

where p is an integer between -272 and 272.

The final set indicated by Eq. (3) is {10kHz, 20kHz, 30kHz, …, m*10kHz, …}, which means cr(t) = t * 10k. Therefore, a channel raster with 10kHz can ensure the flexible allocation of an LP-WUS carrier within a wider NR carrier with 100kHz channel raster.

Proposal 2: Introduce 10kHz channel raster for LP-WUR for an NR band with 100kHz channel raster to ensure an LP-WUS carrier can be flexibly allocated within an NR carrier in that NR band.

# Conclusion

In this contribution, we have the following proposals on system parameters for LP-WUR:

Proposal 1: For NR bands with SCS based channel raster, no new channel raster is needed for LP-WUR.

Proposal 2: Introduce 10kHz channel raster for LP-WUR for an NR band with 100kHz channel raster to ensure an LP-WUS carrier can be flexibly allocated within an NR carrier in that NR band.

# Reference

[1] R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo