3GPP TSG-RAN WG4 Meeting #112											                 R4-2411098

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			10.2

Source:	CATT

Title:	On solution to enable HPUE maximum transmit power in downlink CA with single UL transmission

Document for:	Discussion

# Introduction

In RAN#104, a task was assigned to RAN4 to investigate a solution for enabling HPUE maximum transmit power in downlink carrier aggregation with single uplink transmission [1], as a final effort with a reduced target to address the long-standing power class issues on which discussions have now been stopped:

Way Forward - RAN tasks RAN4 to investigate a solution to enable HPUE maximum transmit power in downlink CA with single UL transmission. - RAN task shall conclude by RAN#106 - RAN4 online meeting time should be limited to <0.1 TUs

In this contribution, we propose a simple solution to enable HPUE maximum transmission power.

# Discussion

The ultimate origin of the ambiguity in current RAN4 specs on power class for CA is the word ”assigned” for FR1 [2]:

and ”configured” for FR2 [3]:

There could be two different interpretations on these key words:

- Understanding #1: UL configured and scheduled

- Understanding #2: UL configured but not scheduled

Both lead to different applicable requirements specified, e.g., for Understanding#2, even an UL uplink is not scheduled, the power class cap for the band combination still applies, hence HPUE maximum transmission power capability cannot be fully exploited.

One straightforward solution is to explicitly state that the applicable uplink power class requirements depend on the actual uplink transmission for inter-band carrier aggregation. This clarification could of course allow HPUE maximum transmission power to be achieved in scenarios where carrier aggregation occurs only in the downlink direction, with only one carrier being scheduled and transmitted in the uplink.

Proposal 1: RAN4 explicitly specify that the applicable uplink power class requirements depend on the actual uplink transmission for inter-band carrier aggregation.

Our companion draft CRs implement this simple solution [4][5].

Proposal 2: RAN4 to endorse the draft CRs as in [4][5].

# Conclusion

In this contribution, we have the following proposals on the RAN task to investigate a solution for enabling HPUE maximum transmit power in downlink carrier aggregation with single uplink transmission:

Proposal 1: RAN4 explicitly specify that the applicable uplink power class requirements depend on the actual uplink transmission for inter-band carrier aggregation.

Proposal 2: RAN4 to endorse the draft CRs as in [4][5].

# Reference

[1] RP-241625, ” Moderator's summary for offline discussion on CA Power Class Support”, Moderator(AT&T)

[2] 3GPP TS 38.101-1

[3] 3GPP TS 38.101-2

[4] R4-2411099, ” draftCR to TS 38.101-1 on enabling HPUE maximum transmission power for downlink CA with single uplink transmission”, CATT

[5] R4-2411100, ” draftCR to TS 38.101-2 on enabling HPUE maximum transmission power for downlink CA with single uplink transmission”, CATT