3GPP TSG-RAN WG4 Meeting #112											                R4-2411108

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			8.1.1.2.1

Source:	CATT

Title:	Further discussion on power domain enhancements for single carrier

Document for:	Discussion

# Introduction

For the Rel-19 UE RF WID, RAN4 has continued the discussion on power domain enhancements for single carrier and agreed on a way forward in RAN4#111 [1], which captured the following related agreements:

## 1.1 Consideration on relaxation of requirements for identified scenarios Agreement in main session: - No relaxation of level of spurious emission requirements - FFS on whether to change the spurious emission boundary ## 1.2. Mechanisms for enabling MPR reduction and/or power boosting Way forward: - Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW - Other mechanisms are not precluded

In this contribution, we continue our discussion on the aforementioned issues.

# Discussion

2.1 Spurious domain boundary

For the out-of-band (OOB) and spurious domain definition, ITU-R introduces 250% of the necessary bandwidth from the center of transmission as the boundary according to SM.329 [2], while 3GPP has a more strict requirement which sets the boundary as channel bandwidth + 5MHz from the edge of the channel, as illustrated in Fig. 1 (less than 5MHz channel bandwidth not applicable).

Fig. 1, OOB domain definitions

In addition, 3GPP has also specified spectrum emission mask (SEM) in OOB domain, and in-band emission (IBE) within the UE channel bandwidth.

In the targeted scenario to be improved where the UE channel bandwidth is located in a larger BS channel bandwidth, as agreed in last RAN4 meeting, the spurious emission level keeps unchanged, no matter whether the spurious emission boundary is changed or not. If we move the boundary, the emission falls into BS channel bandwidth at the receiver is changed hence the integral of the emission within the BS channel may impact BS receiver performance, and it has to be analyzed sufficiently before we make such a decision.

Proposal 1: Sufficient evaluation of impacts on BS receiver should be conducted before RAN4 makes a decision to change the spurious domain boundary for the target scenario where a UE channel bandwidth is located in a wider BS channel bandwidth.

However, considering the limited time and minimizing specs impacts, we prefer a solution as simple as possible and tend to propose to keep the spurious boundary unchanged.

Proposal 2: A feasible simple solution is to keep the spurious boundary unchanged for the target scenario if considering the limited time and minimized specs impacts.

2.2 Mechanism to enable MPR reduction and/or power boosting

For the mechanism to enable MPR reduction and/or power boosting, RAN4 is mainly focusing on the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW.  In this case, the actual OOB bandwidth is changed from BWchannel+5MHz to ½BWchannel+5MHz. It is noteworthy that the current minimum OOB bandwidth is 5+5 = 10MHz. By extending UE BW, it is reasonable to keep the minimum OOB bandwidth as 10MHz, i.e., ½BWchannel+5MHz should be no less than 10MHz, hence BWchannel should be 10MHz or larger for the target scenario.

Proposal 3: Only consider the approach to enable MPR reduction and/or power boosting for a UE channel bandwidth no less than 10MHz.

Now the issue is how to change SEM in the modified OOB domain if keeping the spurious emission boundary unchanged but extending UE BW at the same time.

Proposal 4: Consider changing SEM level in the modified OOB domain by extending UE BW and keeping spurious emission domain unchanged.

Actually, other requirements such as ACLR is applicable in the OOB domain independent from SEM, therefore, no matter how SEM is changed, ACLR requirement should be the same.

Proposal 5: Keep ACLR unchanged when changing SEM level in the modified OOB domain.

To minimized the impacts on operations among different BS carriers, any change of SEM level in the modified OOB domain shall not relax emissions outside BS channel bandwidth.

Proposal 6: No relaxation outside BS channel bandwdith.

# Conclusion

In this contribution, we have the following proposals on power domain enhancements for single carrier:

Proposal 1: Sufficient evaluation of impacts on BS receiver should be conducted before RAN4 makes a decision to change the spurious domain boundary for the target scenario where a UE channel bandwidth is located in a wider BS channel bandwidth.

Proposal 2: A feasible simple solution is to keep the spurious boundary unchanged for the target scenario if considering the limited time and minimized specs impacts.

Proposal 3: Only consider the approach to enable MPR reduction and/or power boosting for a UE channel bandwidth no less than 10MHz.

Proposal 4: Consider changing SEM level in the modified OOB domain by extending UE BW and keeping spurious emission domain unchanged.

Proposal 5: Keep ACLR unchanged when changing SEM level in the modified OOB domain.

Proposal 6: No relaxation outside BS channel bandwdith.

# Reference

[1] R4-2410749, ”WF on power domain enhancements”, Huawei/HiSilicon

[2] Recommendation  ITU-R  SM.329-12, ” Unwanted emissions in the spurious domain”, ITU-R