3GPP TSG-RAN WG4 Meeting #112											              R4-2411109

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			8.1.1.2.2

Source:	CATT

Title:	Further discussion on MPR applicability for FR1 intra-band UL CA

Document for:	Discussion

# Introduction

For the Rel-19 UE RF WID, RAN4 has continued the discussion on MPR applicability for FR1 intra-band UL CA and agreed on a way forward in RAN4#111 [1], which captured the following related agreements:

## 2.1. MPR applicability for FR1 intra-band contiguous UL CA Way forward: - For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated. - FFS: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. - FFS: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. - FFS: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. - FFS whether spurious emissions/ACLR/SEM are kept for aggregated CBW for CA ## 2.2. MPR applicability for FR1 intra-band UL non-contiguous CA Way forward: Enhancements not aligned with the WID will not be considered. - Further clarify the scope for NC CA in next meeting based on the WID objectives.

In this contribution, we continue our discussion on the aforementioned issues.

# Discussion

Currently RAN4 discussion is focused on the case where a single uplink CC is activated or transmitted even with uplink configured with intra-band UL CA (bandwidth class B and C). RAN4 has specified MPR requirements for intra-band UL CA with both contiguous and non-contiguous RB allocation for different power classes, e.g., the below table specifies MPR requirements for contigous RB allocation for power class 3:

| It is obvious that these MPR values assume that RB allocation in the two contiguous CCs are also contiguous, equivalent to a larger “virtual” carrier with a larger “virtual” BWP compared with the case of single carrier only, hence there are a couple dBs of MPR difference between the two cases. However, if there is only a single uplink carrier transmission, applying the MPR requirements for intra-band contiguous UL CA would be excessive. Therefore, it is more reasonable to adhere to the MPR requirements, e.g., Table 6.2.2-1 for PC3, for a single carrier instead. Proposal 1: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C for PC3 contiguous UL CA when only 1 CC is transmitted. |  |  |  |
| --- | --- | --- | --- |

Similar consideration may also apply for PC2 with or without TxD.

Proposal 2: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated for PC2 contiguous UL CA when only 1 CC is transmitted.

Proposal 3: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent for PC2 contiguous UL CA when only 1 CC is transmitted.

For spurious emissions, ACLR, and SEM, the same logic applies for uplink contiguous CA (bandwidth class B and C), e.g., for ACLR, tables below specify CA ACLR, where aggregated channel bandwidth acts as a ”virtual” larger channel bandwidth.

It is reasonable to apply spurious emission, ACLR and SEM for single UL CC transmission for CA bandwidth class B and C.

Proposal 4: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

# Conclusion

In this contribution, we have the following proposals on MPR applicability for FR1 intra-band UL CA:

Proposal 1: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C for PC3 contiguous UL CA when only 1 CC is transmitted.

Proposal 2: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated for PC2 contiguous UL CA when only 1 CC is transmitted.

Proposal 3: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent for PC2 contiguous UL CA when only 1 CC is transmitted.

Proposal 4: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

# Reference

[1] R4-2410749, ”WF on power domain enhancements”, Huawei/HiSilicon