3GPP TSG-RAN WG4 Meeting #112											                R4-2411110

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			8.1.1.2.3

Source:	CATT

Title:	Further discussion on MPR applicability for FR2

Document for:	Discussion

# Introduction

In RAN4#111, RAN4 has reached some agreements regarding MPR applicability for FR2 [1]:

## 2.3. Applicable MPR for FR2 single carrier UL with DL intra band CA Way forward: - For the case of single carrier UL with DL intra band CA, the MPR requirements of single carrier case in clause 6.2.2 of TS 38.101-2 applies with UE indication of new capability for MPR improvement. ## 2.4. Applicable MPR for FR2 UL CA with DL intra band CA Way forward: - MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) with UE indication of new capability for MPR improvement - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - FFS whether new MPR requirement could be defined for CABW < 400 MHz (e.g., 200 MHz).

In this contribution, we continue discussing these issues and propose not to introduce new UE capability indicating MPR improvement for FR2 single UL carrier with DL intra-band CA, and hold on discussion on specifying new MPR requirement for CABW < 400MHz unless the WID is updated.

# Discussion

2.1 MPR applicability for FR2 single UL carrier with DL intra-band CA

RAN4 has agreed that for the case of single carrier UL with DL intra-band CA, the MPR requirements of single carrier case in clause 6.2.2 of TS 38.101-2 applies with UE indication of new capability for MPR improvement. However, in the context of FR2 intra-band CA, such a capable UE is usually designed to manage its transmit power efficiently across the aggregated carriers without requiring additional network intervention for MPR adjustments. The UE's built-in capabilities allow it to meet the necessary power limits and out-of-band emission requirements autonomously. This design ensures that the UE can maintain optimal performance and compliance with regulatory standards while minimizing interference with adjacent channels.

As a result, network-side configurations to enable MPR improvement become redundant for these UEs. The network does not need to implement specific measures to control the UE's transmission power, as the UE can handle these adjustments independently, and can already be reflected by PHR reporting etc. This autonomy simplifies the network's management of UEs and helps in maintaining overall system efficiency.

Proposal 1: Network does not need to enable the MPR improvement originating from the application of MPR requirements for single UL carrier if there is no other particular reason, hence, there is no need to introduce a new UE capability for such MPR improvement.

2.2 MPR applicability for FR2 UL intra-band CA with DL intra-band CA

Similar consideration can be applicable for MPR improvement by changing BW basis from CABW to UL BWchannel_CA.

Proposal 2: Network does not need to enable the MPR improvement originating from changing the BW basis from CABW to UL BWchannel_CA if there is no other particular reason, hence, there is no need to introduce a new UE capability for such MPR improvement.

According to the most recent revision of the WID for this Rel-19 Work Item, the objective concerning MPR applicability for FR2 intra-band CA is outlined as follows [2]:

- Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated

As also pointed out by Moderator in previous discussions, the objective only involves MPR applicability, not new MPR requirements, therefore, we propose to hold on discussions on whether new MPR requirement is defined for CABW < 400MHz unless the WID can be updated accordingly.

Proposal 3: Hold on discussions on whether new MPR requirement is defined for CABW < 400MHz unless the WID can be updated accordingly.

# Conclusion

In this contribution, we have the following proposals for MPR applicability for FR2 intra-band CA:

Proposal 1: Network does not need to enable the MPR improvement originating from the application of MPR requirements for single UL carrier if there is no other particular reason, hence, there is no need to introduce a new UE capability for such MPR improvement.

Proposal 2: Network does not need to enable the MPR improvement originating from changing the BW basis from CABW to UL BWchannel_CA if there is no other particular reason, hence, there is no need to introduce a new UE capability for such MPR improvement.

Proposal 3: Hold on discussions on whether new MPR requirement is defined for CABW < 400MHz unless the WID can be updated accordingly.

# Reference

[1] R4-2410749, ”WF on power domain enhancements”, Huawei/HiSilicon

[2] RP-241656, ” Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei/HiSilicon