3GPP TSG-RAN WG4 Meeting #112											                  R4-2411111

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			10.1.1.1

Source:	CATT

Title:	On technical wording ambiguities and table modifications for UE RF specs improvement

Document for:	Discussion

# Introduction

For the RAN task on UE RF specs quality improvement, RAN4 has made some further agreements on table modifications as following [1]:

### Sub-topic 1-1 Specification table simplification Issue 1-1-1: Simplification of Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands) (from R4-2404448) <Way forward/Agreement>: Agreement: - Do not consider the simplification for the tables of combinations of operating bands for 5G - Further discuss the simplification for configuration tables for EN-DC - Reducing the redundancy of band combination tables with the readability being ensured can be done for the future generation ### Sub-topic 1-2 MSD simplification Issue 1-2-1: Simplification receiver relaxations tables <Way forward/Agreement>: Consider following Options to simplify ΔTIB,c and ΔRIB,c tables - Option 1: List CA band combinations per ΔTIB,c or ΔRIB,c row (a specific example can be seen in R4-2407688) - Option 2: Use following generic values for ΔRIB,c and ΔTIB,c depending on the number of bands for band combination [R4-2407225] the number of bands; ΔRIB,c; ΔTIB,c. 2; 0.0 dB; 0.1 dB. 3; 0.2 dB. 4; 0.3 dB. 5; 0.4 dB. 6; 0.5 dB. - Option 3: Other options are not precluded

And we keep advocating to use template-based approach to remove redundancy by showing the example on operating band combination tables, however, as discussed, there is a concern on its readibility with the proposed approach.

In this contribution, we propose to modify configuration tables for EN-DC and also introduce the template-based approach ΔTIB,c and ΔRIB,c tables for NR CA but at the same time still keeping readibility.

# Discussion

## Modifying configuration tables for EN-DC

For intra-band EN-DC configuration tables, e.g., the below table indicates channel bandwidths for E-UTRA and NR carrier respectively:

The table inherits LTE’s style to indicate component carriers in order of increasing carrier frequency, hence, to allow all permutation of LTE and NR carriers, the table has two symmetric table cells for LTE carrier as illustrated in red circles and blue circles as two examples. The redundant symmetric table cells actually invalidate the need of order of increasing carrier frequency, therefore, we propose to remove the columns/rows associated with the symmetric table cell and clarify without consideration of order of increasing carrier frequency. The below table is the one under this proposal:

Table – 1: Simplification of configuration tables of EN-DC

| E-UTRA – NR configuration /Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers without considering order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) |  |  |  |
| DC_(n)3AA | DC_(n)3AA4 | 5, 10, 15, 20 | 5,10,15,20, 25, 30 |  | 50 | 0 |
| DC_(n)5AA | DC_(n)5AA4 | 5, 10 | 5, 10, 15, 20 |  | 25 | 0 |
| DC_(n)7AA | DC_(n)7AA4 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_(n)12AA | DC_(n)12AA4 | 5, 10 | 5, 10 |  | 15 | 0 |
| DC_(n)38AA | DC_(n)38AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
| DC_(n)41AA |  | DC_(n)41AA |  | 20 |  | 40, 60, 80,100 |  |  |  | 120 |  | 0 |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |

Proposal 1: For configuration EN-DC tables, do not consider order of increasing carrier frequency for LTE and NR carriers and remove redundant symmetric table cells for each BCS as illustrated in Table - 1.

## Template-based approach for ΔTIB,c and ΔRIB,c tables for NR CA

For ΔTIB,c and ΔRIB,c tables for NR CA, Option 1 tries to group band combinations which have the identical values for both, however, this is not ideal for finding a specific band combination, as the lexicographic order is lost, which reduces readability.

Observation 1: Option 1 for simplifying ΔTIB,c and ΔRIB,c tables reduces readability since the lexicographic order of the band combinations is lost.

Option 2 appears ideal in terms of aesthetics, but it involves altering core requirements, which is not feasible at this late stage.

Observation 2: Option 2 for simplifying ΔTIB,c and ΔRIB,c tables involves altering core requirements which is not feasible at this late stage.

As our previous paper points out, the existing redundancy removal is one of the keys to improve UE RF specs, and the template-based approach may be helpful for this purpose. And according to the discussion, readability should not be reduced. In our views, the template-based approach can be applied perfectly for simplifying ΔTIB,c and ΔRIB,c tables resulting in removal of redundancy but unimpacted readability at the same time.

Take two-band tables as one example for ΔTIB,c :

And we can introduce a template: {a, b} representing ΔTIB,c =a for band nX and ΔTIB,c = b for band nY for CA band combination CA_nX-nY, then the table can have two or more band combinations for each row:

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 | Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |
| --- | --- | --- | --- |
|  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |  | {a,b} representing a for nX and b for nY for CA_nX-nY10 |
| CA_n1-n3 | {0.3, 0.3} | CA_n1-n5 | {0.3, 0.3} |
| CA_n1-n7 | {0.5, 0.6} | CA_n1-n8 | {0.3, 0.3} |

And the lexicographic order is still kept row by row. Considering there are many pages for such tables, the improvement is not trivial. And this can be treated as a new Option 3.

Proposal 2: RAN4 to introduce the template-based approach for simplifying ΔTIB,c and ΔRIB,c tables as Option 3, which keeps both lexicographic order of band combinations and readability.

# Conclusion

In this contribution, we have the following observations and proposals for table modifications for UE RF specs improvement:

Proposal 1: For configuration EN-DC tables, do not consider order of increasing carrier frequency for LTE and NR carriers and remove redundant symmetric table cells for each BCS as illustrated in Table - 1.

Table – 1: Simplification of configuration tables of EN-DC

| E-UTRA – NR configuration /Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers without considering order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) |  |  |  |
| DC_(n)3AA | DC_(n)3AA4 | 5, 10, 15, 20 | 5,10,15,20, 25, 30 |  | 50 | 0 |
| DC_(n)5AA | DC_(n)5AA4 | 5, 10 | 5, 10, 15, 20 |  | 25 | 0 |
| DC_(n)7AA | DC_(n)7AA4 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_(n)12AA | DC_(n)12AA4 | 5, 10 | 5, 10 |  | 15 | 0 |
| DC_(n)38AA | DC_(n)38AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
| DC_(n)41AA |  | DC_(n)41AA |  | 20 |  | 40, 60, 80,100 |  |  |  | 120 |  | 0 |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |

Observation 1: Option 1 for simplifying ΔTIB,c and ΔRIB,c tables reduces readability since the lexicographic order of the band combinations is lost.

Observation 2: Option 2 for simplifying ΔTIB,c and ΔRIB,c tables involves altering core requirements which is not feasible at this late stage.

Proposal 2: RAN4 to introduce the template-based approach for simplifying ΔTIB,c and ΔRIB,c tables as Opton 3, which keeps both lexicographic order of band combinations and readability.

# Reference

[1] R4-2410714, ” WF on for [111][139] UERF_Spec_Improvement”, Qualcomm

[2] R4-2404448, ” On UE RF specifications improvement”, CATT