3GPP TSG-RAN WG4 Meeting #112											             R4-2411114

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			8.6.3

Source:	CATT

Title:	Discussion on impacts on UE RF requirements and DL performance for fragmented carriers

Document for:	Discussion

# Introduction

In RANP#104, a study item on fragmented carriers was approved with the following objectives [1]:

- Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering: - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - Other requirements are not precluded, if identified - The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2]; - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4]; - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4] - Impacts on DL performance [RAN4] ; - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2]. NOTE 1: No RAN1 impact is foreseen NOTE 2: RAN2 work, if necessary, will be triggered by RAN4 LS NOTE 3: This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers. - Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In this contribution, we seek to identify the necessary clarification on the scope and scenarios in this SID, and further raise some essential questions for the study which are expected to be answered from the proponents.

# Discussion

Clarification on the work scope

By reading the original proposal [2], the demand for this study comes from a collocated deployment scenario where two operators owning two non-contiguous frequency carriers support intra-band non-contiguous CA operation for these two CCs in an enhanced manner while there is a third carrier owning by a third operator, and the enhancement is enabled by the reception of these two CCs like a “virtual DL CC” resulting in a reduced number of DL Rx chains. Fig.1 illustrates the deployment scenario, named “inter-operator collocated scenario” in the SID.

Fig. 1, Deployment scenario for SID

However, the description above is not reflected in the SID, except the term of “inter-operator collocated scenario” as the target scenario. Hence, it is necessary to clarify the term and accordingly revise the SID to facilitate the study.

Proposal 1: RAN4 to clarify the exact meaning of inter-operator collocated scenario and update the SID accordingly to facilitate the study.

Furthermore, considering to minimize the specs impacts, the “enhanced” reception of the two non-contiguous CCs is still under the current DL non-contiguous CA framework, i.e., the two non-contiguous belong to two different cells, but received by a shared Rx chain, although some RF requirements may be impacted by this enhancement.

Proposal 2: RAN4 to clarify that the reception of the two non-contiguous CCs is still under the current DL non-contiguous CA framework, i.e., the two non-contiguous belong to two different cells, but received by a shared Rx chain, although some RF requirements may be impacted by this enhancement.

And for the sake of simplicity, DL MIMO should not be considered, and RAN4 can focus on non-DL-MIMO at this stage.

Proposal 3: At this stage RAN4 does not take DL MIMO into account.

Moreover, the objective only focuses on DL reception, and it is not clear what is the uplink configuration in this study which requires clarification as well. And it is important to conduct Rx requirement impacts.

Proposal 4: RAN4 to clarify the intended uplink configuration in this study.

Finally, there are two essential questions which require confirmation from proponents.

The first one is on the frequency span. Now it is the maximum channel bandwidth, i.e., 100MHz for FR1, however, for SCS 15kHz, the maximum channel bandwidth is 50MHz instead, therefore, a confirmation from proponents is required for the intended frequency span.

Question 1: According to the original intention, some clarification is required on the DL frequency span, i.e., DL frequency span≤100MHz is only for SCS 30kHz, and ≤  50MHz for SCS 15kHz?

Secondly, for the band with 100kHz channel raster, literally the subcarrier grid of the third CC (from the third operator) located in the gap between two intended CCs may not be aligned with subcarriers of the two CCs according to the current specs, however it would make different if they can be alighed.

Question 2: Is there any regulation that could guarantee the subcarrier grid alignment among all operators in a single band with 100kHz channel raster?

Proposal 5: The answers to the following questions from proponents are needed:

- Question 1: According to the original intention, some clarification is required on the DL frequency span, i.e., DL frequency span≤100MHz is only for SCS 30kHz, and ≤  50MHz for SCS 15kHz?

- Question 2: Is there any regulation that could guarantee the subcarrier grid alignment among all operators in a single band with 100kHz channel raster?

# Conclusion

In this contribution, we have the following proposals on the scope for this study item on the fragmented DL carriers:

Proposal 1: RAN4 to clarify the exact meaning of inter-operator collocated scenario and update the SID accordingly to facilitate the study.

Proposal 2: RAN4 to clarify that the reception of the two non-contiguous CCs is still under the current DL non-contiguous CA framework, i.e., the two non-contiguous belong to two different cells, but received by a shared Rx chain, although some RF requirements may be impacted by this enhancement.

Proposal 3: At this stage RAN4 does not take DL MIMO into account.

Proposal 4: RAN4 to clarify the intended uplink configuration in this study.

Proposal 5: The answers to the following questions from proponents are needed:

- Question 1: According to the original intention, some clarification is required on the DL frequency span, i.e., DL frequency span≤100MHz is only for SCS 30kHz, and ≤  50MHz for SCS 15kHz?

- Question 2: Is there any regulation that could guarantee the subcarrier grid alignment among all operators in a single band with 100kHz channel raster?

# Reference

[1] RP-241669, ” New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek

[2] RP-233374, ” Fragmented carriers in the DL”, TELUS et, al.