3GPP TSG-RAN WG4 Meeting #112											               R4-2411115

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			8.7.2

Source:	CATT

Title:	On UE RF requirements for PC2 RedCap

Document for:	Discussion

# Introduction

With the increasing demands for enhanced RedCap devices, a Rel-19 work item was agreed in RAN#104 for PC2 RedCap with the following objectives[1]:

- Apply the PC2 RF requirements for non-RedCap UE to RedCap UE, based on 1Tx architecture with 26 dBm PA. - The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed. - For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. - Note: Not differentiate the applicable form factors for TDD bands in the specification. - Check at RAN #106 on whether to define PC2 RF requirements for FDD bands and the applicable form factors for FDD bands (if defined). Note 1: Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE. Note 2: PC2 for Rel-17 Redcap and Rel-18 eRedcap are considered for this WI. Table: Requested example frequency bands for PC2 RedCap UE NR band; contact name, company; Contact email; other supporting companies (min. 3); status (new, ongoing, completed, stopped). n77, n78; Bo Liu, China Telecom; liubo1@chinatelecom.cn; MediaTek, ZTE, Sanechip, Vodafone, Verizon; completed for non-RedCap PC2, new for RedCap PC2. n41; Xiaoran ZHANG, CMCC; zhangxiaoran@chinamobile.com; MediaTek, ZTE, Sanechip; completed for non-RedCap PC2, new for RedCap PC2. n79; Xiaoran ZHANG, CMCC; zhangxiaoran@chinamobile.com; MediaTek, ZTE, Sanechip; completed for non-RedCap PC2, new for RedCap PC2.

In this contribution, we discuss some generic aspects on RedCap PC2 specifications.

# Discussion

The topic of operating bands and requirements for RedCap was once highly controversial. As a compromise, the current RAN4 specifications include only one Tx RF requirement, with the suffix "I" indicating RedCap. This approach allows for flexibility to accommodate different interpretations and implementations [2].

The current specs structure for Tx and Rx RF requirements are shown in the following:

## 6.1I (Reserved) ## ------ ## 6.2I Transmitter power for RedCap ### 6.2I.1 Maximum output power for RedCap For Redcap UE, the requirements for power class 3 specified in clause 6.2.1 apply.

## 7.1I General For a Redcap UE the requirements in Section 7 shall be verified with the channel bandwidth up to 20MHz and REFSENS specified in clause 7.3I. ---- ## 7.3I Reference sensitivity for RedCap ### 7.3I.1 General The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. ### 7.3I.2 Reference sensitivity power level … ### 7.3I.3 Reference sensitivity power level for eRedCap ...

Thanks to subclause 7.1I, the whole set of Rx RF requirements are defined for a RedCap UE. However, strictly speaking, there is no other explicit other Tx RF requirements for a RedCap UE, except power class 3. Therefore, with the addition of RedCap PC2, RAN4 should make it very clear and complete the whole Tx RF requirements set. One possible way is to add a general sub-clause like 7.1I:

6.1I General For a Redcap UE the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

Proposal 1: To complete the missing piece in current specification, RAN4 should add one sub-clause as following:

6.1I General For a Redcap UE the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

For the operating bands supporting RedCap PC2, it can be added into sub-clause 6.2I.1. The below shows a possible way to add example bands in this WID:

### 6.2I.1 Maximum output power for RedCap For Redcap UE, the requirements for power class 3 specified in clause 6.2.1 apply, and the requirements for power class 2 are specified in the following table for any transmission bandwidth configuration within the channel bandwidth of NR carrier up to 20MHz unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). NR band; Class 2 (dBm); Tolerance (dB). n41; 26. n77; 26. n78; 26. n79; 26.

Proposal 2: Revise sub-clause 6.2I.1 to capture maximum output power for RedCap:

### 6.2I.1 Maximum output power for RedCap For Redcap UE, the requirements for power class 3 specified in clause 6.2.1 apply, and the requirements for power class 2 are specified in the following table for any transmission bandwidth configuration within the channel bandwidth of NR carrier up to 20MHz unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). NR band; Class 2 (dBm); Tolerance (dB). n41; 26. n77; 26. n78; 26. n79; 26.

# Conclusion

In this contribution, we have the following proposals for UE RF requirements for PC2 RedCap:

Proposal 1: To complete the missing piece in current specification, RAN4 should add one sub-clause as following:

6.1I General For a Redcap UE the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

Proposal 2: Revise sub-clause 6.2I.1 to capture maximum output power for RedCap:

### 6.2I.1 Maximum output power for RedCap For Redcap UE, the requirements for power class 3 specified in clause 6.2.1 apply, and the requirements for power class 2 are specified in the following table for any transmission bandwidth configuration within the channel bandwidth of NR carrier up to 20MHz unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). NR band; Class 2 (dBm); Tolerance (dB). n41; 26. n77; 26. n78; 26. n79; 26.

# Reference

[1] RP-241665, ” New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1”, China Telecom, MediaTek

[2] 3GPP, TS 38.101-1 v18.6.0