3GPP TSG-RAN WG4 Meeting #112											                R4-2411116

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			7.20.2

Source:	CATT

Title:	On UE RF requirements for PC2 with UL-MIMO for n28

Document for:	Discussion

# Introduction

In RAN#104, a Rel-19 spectrum-related WID on enhancing n28 support including PC2 and 40MHz UE channel bandwidth was approved with two major objectives, of which the first objective is seen below [1]:

- Introduce requirements for PC2 and those associated with following bullets - Both 1Tx and 2Tx PC2 are supported - UL-MIMO is supported. - Specify A-MPR requirements for PC2 - No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

The second objective is excerpted as following and discussed in our companion paper [2]:

- Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2. - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz. - Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated.

In this contribution, we mainly focus on some general aspects on the required standardization work.

# Discussion

As per objectives stated in WID, the second objective is for introducing 40MHz UE channel bandwidth for both PC2 and PC3 implying a single duplexer architecture, hence it is quite natural that the first objective of introducing PC2 requirements is for UE channel bandwidths no larger than 30MHz in order to avoid overlapping between two objectives.

Proposal 1: To prevent overlap between the two objectives in the WID, RAN4 should focus on scenarios where the UE channel bandwidth is no greater than 30MHz for the first objective of the WID regarding PC2 requirements.

With the introduction of a new duplexer architecture, there are potentially different set of requirements for different power classes (PC3, PC2) and duplexer architectures (dual and single duplexer) as illustrated in Table – 1 below:

Table – 1 Objective #1: Requirements specified and to be specified (UE CBW <= 30MHz)

|  | Dual duplexer | Single duplexer |
| --- | --- | --- |
| PC3 | Specified | Same as that for PC3 dual-duplexer |
| PC2 | To be specified | To be specified |

As stated in the second note of the WID, there is a strong preference to avoid changes to the existing requirements (38.101-1 v18.5.0) in order to prevent any impact on existing PC3 networks. Consequently, it is reasonable to apply the current PC3 requirements with dual duplexer to PC3 with a single duplexer.

Proposal 2: Apply the same UE RF requirements for PC3 with dual duplexer to PC3 with single duplexer for UE channel bandwidth up to 30MHz.

Since PC2 requirements for band n28 have not yet been specified, RAN4 will need to consider both duplexer architectures. To accommodate flexible implementations, two sets of UE RF requirements for PC2 can be established: one for dual duplexer and the other for single duplexer. Additionally, a new UE capability can be introduced to allow the network to differentiate between the two types of UEs.

Proposal 3: RAN4 to introduce two sets of UE RF requirements for PC2 where one is associated with dual duplexer architecture and the other is with single duplexer for UE channel bandwidth up to 30MHz.

Proposal 4: Introduce a new UE capability [singleDuplexer-n28] to indicate the support of single duplexer architecture for band n28 for UE channel bandwidth up to 30MHz.

# Conclusion

In this contribution, we have the following proposals for UE RF requirements for PC2 for n28:

Proposal 1: To prevent overlap between the two objectives in the WID, RAN4 should focus on scenarios where the UE channel bandwidth is no greater than 30MHz for the first objective of the WID regarding PC2 requirements.

Proposal 2: Apply the same UE RF requirements for PC3 with dual duplexer to PC3 with single duplexer for UE channel bandwidth up to 30MHz.

Proposal 3: RAN4 to introduce two sets of UE RF requirements for PC2 where one is associated with dual duplexer architecture and the other is with single duplexer for UE channel bandwidth up to 30MHz.

Proposal 4: Introduce a new UE capability [singleDuplexer-n28] to indicate the support of single duplexer architecture for band n28 for UE channel bandwidth up to 30MHz.

# Reference

[1] RP-241660, ” New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28”, CMCC, CBN

[2] R4-2411117, ” on UE RF requirements for introducing 40MHz channel bandwidth for n28”, CATT