3GPP TSG-RAN WG4 Meeting #112											                R4-2411117

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			7.20.3

Source:	CATT

Title:	on UE RF requirements for introducing 40MHz channel bandwidth for n28

Document for:	Discussion

# Introduction

In RAN#104, a spectrum-related WID was approved to satisfy the strong market demand for a larger channel bandwidth, i.e., 40MHz for band n28, which necessitates the usage of a single duplexer for this band, and the corresponding objective is shown as following [1]:

• Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2.  Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz. • Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated.

In this contribution, we discuss some generic aspects on the required standardization works.

# Discussion

Relative channel bandwidth

For 40MHz channel bandwidth located in 703 – 743 MHz, the relative channel bandwidth is calculated as following:

Relative channel bandwidth = 2*BWChannel / (FUL_low + FUL_high)=2*40/(703+743)=5.5%

It is much larger than the FDD threshold 3%, hence new ΔMPR values for PC3 and PC2 should be specified for the channel bandwdith which requires new MPR/A-MPR evaluation with a single duplexer architecture.

Proposal 1: Relative channel bandwidth of 40MHz at the requested UL carrier location is 5.5%, hence new MPR/A-MPR evaluation is required for specifying corresponding ΔMPR values for PC3 and PC2 with a single duplexer architecture.

MPR/A-MPR simulation assumptions

As stated above, new MPR/A-MPR evaluation/simulation is required for both PC3 and PC2 with a single duplexer architecture. To facilitate and consolidate evaluation results from different companies, a PA calibration point should be agreed at first for PC2 and PC3 respectively.

Proposal 2: PA calibration point should be agreed for PC3 and PC2 respectively for further MPR/A-MPR simulation consolidation.

Exceptional channel raster point (UL: 144608/DL: 155608)

Previously when the exceptional channel raster entry (UL: 144608/DL: 155608) is added to 40MHz BS CBW is added to TS 38.104, the same channel raster point is not added to TS 38.101-1 due to lack of 40MHz UE CBW, however, there is an agreement indicating that it should be added to TS 38.101-1 when UE 40MHz CBW is specified [2]:

- In the future, if UE 40MHz CBW is specified, a new channel raster for n28 40MHz will also be added in TS38.101-1

Hence, it comes to that moment to add it to TS 38.101-1.

Proposal 3: Add the exceptional channel raster point of n28 to UE RF specification TS 38.101-1 for UE CBW 40MHz.

# Conclusion

In this contribution, we have the following proposals for UE RF requirements for introducing 40MHz CBW for n28:

Proposal 1: Relative channel bandwidth of 40MHz at the requested UL carrier location is 5.5%, hence new MPR/A-MPR evaluation is required for specifying corresponding ΔMPR values for PC3 and PC2 with a single duplexer architecture.

Proposal 2: PA calibration point should be agreed for PC3 and PC2 respectively for further MPR/A-MPR simulation consolidation.

Proposal 3: Add the exceptional channel raster point of n28 to UE RF specification TS 38.101-1 for UE CBW 40MHz.

# Reference

[1] RP-241660, ” New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28”, CMCC, CBN

[2] R4-2220816, ” Way forward on operation with different channel BWs for n28”, CMCC