3GPP TSG-RAN WG4 Meeting #112                                                                                                      R4-2411120

Maastricht, Netherlands, 19th –23th August, 2024

Source: 	CATT

Title: 	Discussion on coexistence evaluation for Ku-band for NR NTN

Agenda item:	8.9.3

Document for:	Approval

- Introduction

In this contribution, we provide our views on coexistence evaluation for Ku-band for NR NTN.

- Discussion

For objective of introduction of Ku Band for NR NTN in WID, we discuss coexistence evaluation for Ku-band for NR-NTN as below:

Frequency range designation for Ku-band

As per the WID [1], the objectives concerning coexistence evaluation for Ku-band is shown as below:

- Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services.

As above objectives of coexistence evaluation for Ku-band, since the DL and UL frequency of Ku-band are different from those of Ka-band. So it is needed to update Satellite Parameters and VSAT Parameters for Ku-band.

According to above frequency for ITU regions for Ku-band, it is sufficient to use 12GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation.

Proposal 1: Use 12GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation for Ku-band.

The Ka-band satellite antenna parameters for 20GHz DL and 30GHz UL in Table 6.1.1.1-1 of TR 38.821 are copied as following table:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

Like Ka-band coexistence simulation, the Satellite TX max Gain, TX  3dB beamwidth, Satellite RX max Gain in Table 6.1.1.1-1 of TR 38.821 can be reused for Ku-band SAN antenna, but the TX equivalent satellite antenna aperture  and RX Equivalent satellite antenna aperture need to be modified for 12GHz DL frequency and 14GHz UL frequency.  For example, the TX equivalent satellite antenna aperture for 12GHz DL can be calculated with 5m*20GHz/12GHz, which is equal to 8.33m.  So the Satellite antenna parameter in following table can be used for Ku-band coexistence simulation.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6a.2.3.1 in TR 38.863 | Section 6a.2.3.1 in TR 38.863 | Section 6a.2.3.1 in TR 38.863 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (12 GHz for DL) | 8.33 m | 0.83 m | 0.83 m |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (14 GHz for UL) | 7.14 m | 0.71 m | 0.71 m |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

Base on above analysis, we propose,

Proposal 2: Use the Satellite antenna parameter in following table for Ku-band coexistence simulation.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6a.2.3.1 in TR 38.863 | Section 6a.2.3.1 in TR 38.863 | Section 6a.2.3.1 in TR 38.863 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (12 GHz for DL) | 8.33 m | 0.83 m | 0.83 m |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (14 GHz for UL) | 7.14 m | 0.71 m | 0.71 m |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

The Ka-band VSAT antenna parameters for 20GHz DL and 30GHz UL in Table 6.1.1.1-3 of TR 38.821 are copied as following table:

| Characteristics | VSAT |
| --- | --- |
| Frequency band | Ka band(30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of TR 38.811 with 60 cm equivalent aperture diameter |
| Polarisation | circular |
| Rx antenna gain | 39.7 dBi |
| Tx antenna gain | 43.2 dBi |

Like Ka-band coexistence simulation, the above Rx antenna gain and Tx antenna gain for VSAT can be reused for VSAT for Ku-band, but the 60 cm equivalent aperture diameter for 30 GHz UL and 20 GHz DL need to be modified for 14 GHz UL and 12 GHz DL for Ku-band. For example, the equivalent aperture diameter for VSAT for 14 GHz UL and 12 GHz DL for Ku-band can be calculated with 60cm*30GHz/14GHz, which is equal to 129cm. So the VSAT antenna parameter in following table can be used for Ku-band coexistence simulation.

| Characteristics | VSAT |
| --- | --- |
| Frequency band | Ku band(14 GHz UL and 12 GHz DL) |
| Antenna type and configuration | Directional Section 6a.2.3.1 in TR 38.863 with129 cm equivalent aperture diameter |
| Polarisation | circular |
| Rx antenna gain | 39.7 dBi |
| Tx antenna gain | 43.2 dBi |

Proposal 3: Use the VSAT antenna parameter in following table for Ku-band coexistence simulation.

| Characteristics | VSAT |
| --- | --- |
| Frequency band | Ku band(14 GHz UL and 12 GHz DL) |
| Antenna type and configuration | Directional Section 6a.2.3.1 in TR 38.863 with129 cm equivalent aperture diameter |
| Polarisation | circular |
| Rx antenna gain | 39.7 dBi |
| Tx antenna gain | 43.2 dBi |

- Conclusion

This contribution provides analysis on coexistence evaluation parameters for Ku-band for NR NTN, and the following proposals and observations are concluded as follows:

Proposal 1: Use 12GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation for Ku-band.

Proposal 2: Use the Satellite antenna parameter in following table for Ku-band coexistence simulation.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6a.2.3.1 in TR 38.863 | Section 6a.2.3.1 in TR 38.863 | Section 6a.2.3.1 in TR 38.863 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (12 GHz for DL) | 8.33 m | 0.83 m | 0.83 m |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (14 GHz for UL) | 7.14 m | 0.71 m | 0.71 m |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

Proposal 3: Use the VSAT antenna parameter in following table for Ku-band coexistence simulation.

| Characteristics | VSAT |
| --- | --- |
| Frequency band | Ku band(14 GHz UL and 12 GHz DL) |
| Antenna type and configuration | Directional Section 6a.2.3.1 in TR 38.863 with129 cm equivalent aperture diameter |
| Polarisation | circular |
| Rx antenna gain | 39.7 dBi |
| Tx antenna gain | 43.2 dBi |

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104