3GPP TSG-RAN WG4 Meeting #112                                                                                       R4-2411121

Maastricht, Netherlands, 19th –23th August, 2024

Source: 	CATT

Title: 	Discussion on system parameters for Ku-band for NR NTN

Agenda item:	8.9.3

Document for:	Approval

- Introduction

In this contribution, we provide our views on system parameters for Ku-band for NR NTN.

- Discussion

For objective of introduction of Ku Band for NR NTN in WID, we discuss system parameters for Ku-band for NR-NTN as below:

Frequency range designation for Ku-band

As per the WID [1], the objectives concerning frequency range designation for Ku-band is shown as below:

- Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths.

In TS 38.108, the NTN frequency ranges were defined as FR1-NTN and FR2-NTN, frequency range of FR1-NTN and FR2-NTN is 410-7125MHz and 17300-30000MHz respectively. The frequency range of Ku-band is 10.70-14.5GHz, which is not in FR1-NTN and FR2-NTN. This Ku-band is located between FR1-NTN and FR2-NTN, so it is reasonable to extend FR1-NTN upwards or extend FR2-NTN downwards. In TS38.104, 60 kHz and120 kHz SCS were specified for FR2-NTN,  and 15 kHz,30 kHz,60kHz SCS were specified for FR1-NTN. From numerology point of view, large subcarrier spacing (SCS) can ensure that no large inter-carrier interference (ICI) caused by large Doppler frequency shift. For satellite operating in Ku-band, the large subcarrier spacing (SCS) is needed. So we prefer to extend FR2-NTN downwards for Ku-band as following table 2-1.

Table2-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 7125 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

Proposal 1: Extend FR2-NTN downwards for Ku-band as following table 2-1:

Table2-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 7125 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

Operating bands for Ku-band

As above proposal, we propose to extend FR2-NTN downwards for Ku-band, so it is reasonable to define operating bands for Ku-band in FR2-NTN as following table 2-2.

Table 2-2: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512 (NOTE 1) | 27500 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n511 (NOTE 2) | 28350 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n510 (NOTE 3) | 27500 - 28350 MHz | 17300 - 20200 MHz | FDD |
| n509 | 10700 - 12750 MHz | 13750 - 14500 MHz | FDD |
| n508 | 10700 - 12700 MHz | 13750 - 14500 MHz | FDD |
| n507 | 10700 - 12750 MHz | 12750 - 13250 MHz | FDD |
| n506 (NOTE 4) | 10700 - 12700 MHz | 12700 - 13250 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). NOTE 4: This band is applicable excluding US. |  |  |  |

Proposal 2: Define n509, n508, n507, and n506 operating bands in FR2-NTN for Ku-band as following table 2-2.

Table 2-2: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512 (NOTE 1) | 27500 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n511 (NOTE 2) | 28350 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n510 (NOTE 3) | 27500 - 28350 MHz | 17300 - 20200 MHz | FDD |
| n509 | 10700 - 12750 MHz | 13750 - 14500 MHz | FDD |
| n508 | 10700 - 12700 MHz | 13750 - 14500 MHz | FDD |
| n507 | 10700 - 12750 MHz | 12750 - 13250 MHz | FDD |
| n506 (NOTE 4) | 10700 - 12700 MHz | 12700 - 13250 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). NOTE 4: This band is applicable excluding US. |  |  |  |

Transmission bandwidth configuration for Ku-band

For extending FR2-NTN downwards for Ku-band, Transmission bandwidth configuration NRB for FR2-NTN in Table 5.3.2-2 of TS 38.108 can be reused for Ku-band.

Observation 1: For extending FR2-NTN downwards for Ku-band, Transmission bandwidth configuration NRB for FR2-NTN in Table 5.3.2-2 of TS 38.108 can be reused for Ku-band.

Minimum guardband and transmission bandwidth configuration for Ku-band

For extending FR2-NTN downwards for Ku-band, Minimum guardband (kHz) (FR2-NTN) in Table 5.3.3-2 of TS 38.108 can be reused for Ku-band.

Observation 2: For extending FR2-NTN downwards for Ku-band, Minimum guardband (kHz) (FR2-NTN) in Table 5.3.3-2 of TS 38.108 can be reused for Ku-band.

SAN channel bandwidth per operating band for Ku-band

As above analysis, for extending FR2-NTN downwards for Ku-band, the SAN channel bandwidths and SCS per operating band in FR2-NTN for Ku-band can be defined as following table 2-3. It is acceptable to supplement some additional SAN channel bandwidth, such as 125MHz, 250MHz.

Table 2-3: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n509 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n508 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n507 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n506 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |

Observation 3: For extending FR2-NTN downwards for Ku-band, the SAN channel bandwidth and SCS per operating band in FR2-NTN for Ku-band in following table 2-3 can be as starting point.

Table 2-3: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n509 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n508 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n507 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n506 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |

Channel spacing for adjacent carriers for Ku-band

For extending FR2-NTN downwards for Ku-band, channel spacing for adjacent carriers for SAN FR2-NTN operating bands with 60 kHz channel raster as below is applicable for Ku-band.

- For SAN FR2-NTN operating bands with 60 kHz channel raster, - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 + {-20 kHz, 0 kHz, 20 kHz} for ∆FRaster equals to 60 kHz - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 + {-40 kHz, 0 kHz, 40 kHz} for ∆FRaster equals to 120 kHz

Observation 4: For extending FR2-NTN downwards for Ku-band, channel spacing for adjacent carriers for SAN FR2-NTN operating bands with 60 kHz channel raster is applicable for Ku-band.

NR-ARFCN and channel raster for Ku-band

The NR-ARFCN parameters for the global frequency raster in Table 5.4.2.1-1 of TS 38.108 as below were defined for Range of frequencies 0-30000MHz, the frequency of Ku-band is in Range of frequencies 0-30000MHz, so it is applicable for Ku-band.

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| 24250 – 30000 | 60 | 24250.08 | 2016667 | 2016667 – 2112499 |

Observation 5: The NR-ARFCN parameters for the global frequency raster in Table 5.4.2.1-1 of TS 38.108 is applicable for Ku-band.

Channel raster entries for each operating band for Ku-band

For extending FR2-NTN downwards for Ku-band, same step size <1> for 60KHz and <2> for 120kHz for Uplink, <4> for 60KHz and <8> for 120kHz for Downlink can be reused for Ku-band, the range of NREF for operating band in Ku-band can be calculated by same method for Ka-band.

Observation 6: For extending FR2-NTN downwards for Ku-band, same step size <1> for 60KHz and <2> for 120kHz for Uplink, <4> for 60KHz and <8> for 120kHz for Downlink can be reused for Ku-band.

Synchronization raster and numbering for Ku-band

The GSCN parameters for the global frequency raster in Table 5.4.3.1-1 of TS 38.108 as below were defined for Range of frequencies 0-24250MHz, the frequency of Ku-band is in Range of frequencies 0-24250MHz, so it is applicable for Ku-band.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 1200 kHz + M * 50 kHz, N = 1:2499, M ϵ {1,3,5} (Note) | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 | 3000 MHz + N * 1.44 MHz, N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE: The default value for operating bands which only support SCS spaced channel raster(s) is M=3. |  |  |  |

Observation 7: The GSCN parameters for the global frequency raster in Table 5.4.3.1-1 of TS 38.108 is applicable for Ku-band.

Synchronization raster entries for each operating band for Ku-band

For extending FR2-NTN downwards for Ku-band, same SS Block pattern and step size for 120kHz and 240kHz can be reused for Ku-band, the Range of GSCN for operating band in Ku-band can be calculated by same method for Ka-band.

Observation 8: For extending FR2-NTN downwards for Ku-band, same SS Block pattern and step size for 120kHz and 240kHz can be reused for Ku-band.

- Conclusion

This contribution provides analysis on system parameters for Ku-band for NR NTN, and the following proposals and observations are concluded as follows:

Proposal 1: Extend FR2-NTN downwards for Ku-band as following table 2-1:

Table2-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 7125 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

Proposal 2: Define n509, n508, n507, and n506 operating bands in FR2-NTN for Ku-band as following table 2-2.

Table 2-2: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512 (NOTE 1) | 27500 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n511 (NOTE 2) | 28350 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n510 (NOTE 3) | 27500 - 28350 MHz | 17300 - 20200 MHz | FDD |
| n509 | 10700 - 12750 MHz | 13750 - 14500 MHz | FDD |
| n508 | 10700 - 12700 MHz | 13750 - 14500 MHz | FDD |
| n507 | 10700 - 12750 MHz | 12750 - 13250 MHz | FDD |
| n506 (NOTE 4) | 10700 - 12700 MHz | 12700 - 13250 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). NOTE 4: This band is applicable excluding US. |  |  |  |

Observation 1: For extending FR2-NTN downwards for Ku-band, Transmission bandwidth configuration NRB for FR2-NTN in Table 5.3.2-2 of TS 38.108 can be reused for Ku-band.

Observation 2: For extending FR2-NTN downwards for Ku-band, Minimum guardband (kHz) (FR2-NTN) in Table 5.3.3-2 of TS 38.108 can be reused for Ku-band.

Observation 3: For extending FR2-NTN downwards for Ku-band, the SAN channel bandwidth and SCS per operating band in FR2-NTN for Ku-band in following table 2-3 can be as starting point.

Table 2-3: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n509 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n508 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n507 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n506 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |

Observation 4: For extending FR2-NTN downwards for Ku-band, channel spacing for adjacent carriers for SAN FR2-NTN operating bands with 60 kHz channel raster is applicable for Ku-band.

Observation 5: The NR-ARFCN parameters for the global frequency raster in Table 5.4.2.1-1 of TS 38.108  is applicable for Ku-band.

Observation 6: For extending FR2-NTN downwards for Ku-band, same step size <1> for 60KHz and <2> for 120kHz for Uplink, <4> for 60KHz and <8> for 120kHz for Downlink can be reused for Ku-band.

Observation 7: The GSCN parameters for the global frequency raster in Table 5.4.3.1-1 of TS 38.108 is applicable for Ku-band.

Observation 8: For extending FR2-NTN downwards for Ku-band, same SS Block pattern and step size for 120kHz and 240kHz can be reused for Ku-band.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104