3GPP TSG-RAN WG4 Meeting #112                                                                                                      R4-2411123

Maastricht, Netherlands, 19th –23th August, 2024

Source: 	CATT

Title: 	Discussion on co-existence evaluations for Ambient IoT in NR

Agenda item:	8.20.2.2

Document for:	Approval

- Introduction

In RAN4#111 meeting, a WF [1] on Ambient IoT in NR was agreed and the remaining issues on co-existence elevation in WF require further discussion.

This contribution, we provide our views on co-existence elevation for ambient-IoT in NR.

- Discussion

2.1 Evaluation methodology

Performance metric for AIOT

As per the WF [1], the remaining open issue concerning performance metric for AIoT is shown as below:

Issue 2-4-2: Performance metric for AIOT Agreement in RAN4#110bis: - For NR system, use 5% throughput loss as performance metric as legacy. - For AIOT system, including reader, device, intermediate UE, further discuss the performance metric: - Option 1: [10%] BLER, [Rx power] - Option 2: SINR degradation - Other options are precluded. Agreement in RAN4#111: - Use SINR for calibration purpose - FFS on performance metric for co-existence evaluation and requirements definition.

For BLER and Rx power in option 1, we think that it is hard to find appropriate BLER degradation or Rx power degradation threshold value as performance metric for A-IoT like 5% throughput loss threshold value for legacy NR.

For SINR degradation in option 2, let’s first analyze SINR degradation corresponding to the 5% throughput loss for legacy NR. Referring to the link level performance for 5G NR existence in sub-clause 5.2.7 of TR 38.803, the SINR degradation  can be achieved using the following equations:

(Equation 1)

Where,

(Equation 2)

According to the Table 5.2.7-1 of TR 38.803, the value for DL and UL is 0.6 and 0.4, respectively, and  for DL and UL is 30dB and 20dB, respectively. Using equation 1, we can get that SINR degradation is 1.4959dB and 0.9589dB, respectively.

Observation 1: SINR degradation for DL and UL corresponding to 5% throughput loss for legacy NR coexistence evaluation is 1.4959dB and 0.9589dB, respectively.

Referring to Table 6.4.13-1 of TR 36.802, the Criteria for NB-IoT coexistence evaluation is that average of SNR value loss is less or equal to 1 dB, the SNR degradation (SNR loss) is as performance metric for NB-IoT existence evaluation.

Observation 2: 1 dB SNR degradation (SNR loss) was used as performance metric for NB-IoT existence evaluation in TR 36.802.

Referring to above analysis for SNR degradation in legacy NR and NB-IoT coexistence evaluation, it is reasonable to adopt SINR degradation as performance metric for A-IoT co-existence evaluation. For A-IoT, SINR degradation can directly indicate that the coverage distance is decreased, and coverage distance loss is more important performance requirement for A-IoT, so we propose to adopt SINR degradation (Option 2) as performance metric for A-IoT coexistence evaluation.

From our understanding, the 10% coverage distance loss is coverage performance loss threshold for A-IoT, so SINR degradation is corresponding to degradation of path loss due to 10% coverage distance (d3D) loss. As per WF[1], the NR indoor industrial scenario (InF-xx) and NR InH-Office are defined for A-IoT co-existence scenario, according to Table 7.4.1-1 of TR 38.901, degradation of path loss  due to 10% coverage distance (d3D) loss can be calculated with following equation:

(Equation 3)

Where,

is from scenarios (InH-Office, InF-SL, InF-DL, InF-SH, and InF-DH) in Table 7.4.1-1 of TR 38.901.

Using above Equation 3, we can get:

The in dB for InH-Office is 1.7525dB, which is calculated with.

The in dB for InF-SL is 1.1668dB, which is calculated with.

The in dB for InF-DL is 1.6335dB, which is calculated with.

The in dB for InF-SH is 1.0524dB, which is calculated with.

The in dB for InF-DH is 1.0021dB, which is calculated with.

So, the SINR degradation  corresponding to degradation of path loss due to 10% coverage distance (d3D) loss for InH-Office, InF-SL, InF-DL, InF-SH, and InF-DH is 1.7525dB, 1.1668dB, 1.6335dB, 1.0524dB, and 1.0021dB, respectively. The minimum 1.0021dB SNR degradation for InF-DH can apply to all scenarios (InH-Office and InF-xx).

Based on above analysis, for legacy NR coexistence evaluation, SINR degradation for DL and UL corresponding to 5% throughput loss performance metric is 1.4959dB and 0.9589dB, respectively. For NB-IoT, performance metric is 1dB SNR loss (SNR degradation), so it is reasonable to adopt 1dB SNR degradation as performance metric for A-IoT coexistence evaluation.

From our understanding, the 5% CDF SINR distribution means that devices are located at cell edge, and the 50% CDF SINR distribution means that devices are located at cell center. So we propose that it is reasonable to define 1dB SNR degradation for 5% and 50% CDF SINR point as performance metric for A-IoT co-existence evaluation.

Proposal 1: Use 1dB SINR degradation for 5% and 50% CDF SINR point as performance metric for A-IoT co-existence evaluation for DL and UL.

Evaluation methodology

As per the WF [1], the remaining open issue concerning evaluation methodology is shown as below:

Issue 2-4-1: Evaluation methodology Agreement in RAN4#110bis: - Use the Monte-Carlo method as baseline for co-existence evaluation, i.e. Section 5.3 in TR38.803 - Depending on the discussion on deployment scenarios, for some cases, calculation for the worst interference link may be enough. - FFS on whether RAN4 needs to perform link level simulation. Agreement in RAN4#111: - Use the Monte-Carlo method as baseline for co-existence evaluation, i.e. Section 5.3 in TR38.803 - FFS on whether RAN4 needs to perform link level simulation

Based above discussion, we propose to define 1dB SNR degradation for 5% and 50% CDF SINR point as performance metric for A-IoT co-existence evaluation, the simulation steps using SNR degradation for Monte-Carlo method for A-IoT coexistence evaluation is shown as below:

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	Devices are distributed randomly across the network.

2.	Device associations: Devices are associated to Reader based on coupling loss.

3.	SINRICI is computed in the victim systems without considering ACI as below:

-	SINRICI=SN+IICI, where IICI is the inter-cell interference.

-    Calculate CDF of SINRICI, then get SINRICI,@5%CDF and SINRICI,@50%CDF, where SINRICI,@5%CDF is SINRICI at 5% CDF point, and SINRICI,@50%CDF is SINRICI at 50% CDF point.

4.	SINRICI+ACI is computed in the victim systems considering ACI as below:

-	SINRICI+ACI=SN+IICI+IACI, where IACI is the adjacent channel interference.

-    Calculate CDF of SINRICI+ACI, then get SINRICI+ACI,@5%CDF and SINRICI+ACI,@50%CDF, where SINRICI+ACI,@5%CDF is SINRICI+ACI at 50% CDF point, and SINRICI+ACI,@50%CDF is SINRICI+ACI  at 50% CDF point.

5.	RF parameters are determined based on the SNR degradation cause by ACI as below:

-	SNRdegradationACI,@%5CDF=SINRICI,@5%CDF-SINRICI+ACI,@5%CDF

-	SNRdegradationACI,@%50CDF=SINRICI,@50%CDF-SINRICI+ACI,@50%CDF

Proposal 2: Adopt following simulation steps using SNR degradation for Monte-Carlo method for A-IoT coexistence evaluation:

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	Devices are distributed randomly across the network.

2.	Device associations: Devices are associated to Reader based on coupling loss.

3.	SINRICI is computed in the victim systems without considering ACI as below:

-	SINRICI=SN+IICI, where IICI is the inter-cell interference.

-    Calculate CDF of SINRICI, then get SINRICI,@5%CDF and SINRICI,@50%CDF, where SINRICI,@5%CDF is SINRICI at 5% CDF point, and SINRICI,@50%CDF is SINRICI at 50% CDF point.

4.	SINRICI+ACI is computed in the victim systems considering ACI as below:

-	SINRICI+ACI=SN+IICI+IACI, where IACI is the adjacent channel interference.

-    Calculate CDF of SINRICI+ACI, then get SINRICI+ACI,@5%CDF and SINRICI+ACI,@50%CDF, where SINRICI+ACI,@5%CDF is SINRICI+ACI at 50% CDF point, and SINRICI+ACI,@50%CDF is SINRICI+ACI  at 50% CDF point.

5.	RF parameters are determined based on the SNR degradation cause by ACI as below:

-	SNRdegradationACI,@%5CDF=SINRICI,@5%CDF-SINRICI+ACI,@5%CDF

-	SNRdegradationACI,@%50CDF=SINRICI,@50%CDF-SINRICI+ACI,@50%CDF

As above discussion, the 1dB SNR degradation as performance metric for A-IoT coexistence evaluation is reasonable, BLER is not appropriate as performance metric A-IoT coexistence evaluation, the BLER vs SNR curve is not needed for A-IoT coexistence evaluation, so the link level simulation for BLER vs SNR curve is not needed.  Only ICI from co-channel adjacent cell was considered in Monte-Carlo method for A-IoT coexistence evaluation, the ICI between A-IoT carriers and NR carriers in same cell for A-IoT in-band or guard band operation in NR spectrum is unknown and is not needed to be considered in Monte-Carlo method for A-IoT coexistence evaluation.  So link level simulation is needed to determine guard RB size to cancel ICI between A-IoT carriers and NR carriers in same cell for A-IoT in-band or guard band operation in NR spectrum and ensure A-IoT performance.

Observation 3: Link level simulation is needed to determine guard RB size to cancel ICI between A-IoT carriers and NR carriers in same cell for A-IoT in-band or guard band operation in NR spectrum and ensure A-IoT performance.

- Conclusion

This contribution provides analysis on coexistence evaluation for A-IoT. The following proposals and observations are concluded as follows:

Observation 1: SINR degradation for DL and UL corresponding to 5% throughput loss for legacy NR coexistence evaluation is 1.4959dB and 0.9589dB, respectively.

Observation 2: 1 dB SNR degradation (SNR loss) was used as performance metric for NB-IoT existence evaluation in TR 36.802.

Proposal 1: Use 1dB SINR degradation for 5% and 50% CDF SINR point as performance metric for A-IoT co-existence evaluation for DL and UL.

Proposal 2: Adopt following simulation steps using SNR degradation for Monte-Carlo method for A-IoT coexistence evaluation:

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	Devices are distributed randomly across the network.

2.	Device associations: Devices are associated to Reader based on coupling loss.

3.	SINRICI is computed in the victim systems without considering ACI as below:

-	SINRICI=SN+IICI, where IICI is the inter-cell interference.

-    Calculate CDF of SINRICI, then get SINRICI,@5%CDF and SINRICI,@50%CDF, where SINRICI,@5%CDF is SINRICI at 5% CDF point, and SINRICI,@50%CDF is SINRICI at 50% CDF point.

4.	SINRICI+ACI is computed in the victim systems considering ACI as below:

-	SINRICI+ACI=SN+IICI+IACI, where IACI is the adjacent channel interference.

-    Calculate CDF of SINRICI+ACI, then get SINRICI+ACI,@5%CDF and SINRICI+ACI,@50%CDF, where SINRICI+ACI,@5%CDF is SINRICI+ACI at 50% CDF point, and SINRICI+ACI,@50%CDF is SINRICI+ACI  at 50% CDF point.

5.	RF parameters are determined based on the SNR degradation cause by ACI as below:

-	SNRdegradationACI,@%5CDF=SINRICI,@5%CDF-SINRICI+ACI,@5%CDF

-	SNRdegradationACI,@%50CDF=SINRICI,@50%CDF-SINRICI+ACI,@50%CDF

Observation 3: Link level simulation is needed to determine guard RB size to cancel ICI between A-IoT carriers and NR carriers in same cell for A-IoT in-band or guard band operation in NR spectrum and ensure A-IoT performance.

- References

- R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC, RAN4#111

- R4-2411124,Co-existence calibration results for Ambient IoT in NR, CATT, RAN4#112