3GPP TSG-RAN WG4 Meeting #112                                                                                                      R4-2411125

Maastricht, Netherlands, 19th –23th August, 2024

Source: 	CATT

Title: 	Discussion on network controlled repeater classes for NCR

Agenda item:	5.31.1

Document for:	Approval

- Introduction

In this contribution, we provide our views on network controlled repeater classes for NCR.

- Discussion

NCR classes

Since the “Local Area NCR-MT” and “Wide Area NCR-MT” were used in TS 38.106, TS 38.115-1 and TS 38.115-2, but they were not defined in these specs, only network controlled repeater class were defined.

For example, in sub-clause 6.5.2.4.2.1 of TS 38.106, the WA NCR-MT and LA NCR-MT were used, but the definition of  NCR-MT class were missed in TS 38.106.

6.5.2.4.2.1 Minimum requirements for NCR-MT type 1-C For WA NCR-MT type 1-C, the BS ACLR basic requirements specified in clause 6.6.3 and 6.6.4 in TS 38.104 apply. For LA NCR-MT type 1-C, the UE ACLR requirements specified in clause 6.5.2.5 in TS 38.101-1 apply. For simultaneous transmission the limits apply for sum of NCR-MT transmission and NCR-Fwd transmission

For example, in Table 4.6-2 of TS 38.115-1, the NCR-Fwd and NCR-MT are declared separately, but definition of NCR-Fwd class and NCR-MT class were missed in TS 38.106, TS 38.115-1 and TS 38.115-2.

Table 4.6-2: Manufacturer declarations for NCR type 1-C and NCR type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | NCR-Fwd | NCR-MT |
| D.1 | NCR class | NCR class of the NCR, declared as Wide Area NCR, Medium Range NCR, or Local Area NCR. | x | x |

Based on above analysis, it is reasonable to add definition of NCR-MT class, NCR Fwd class for downlink, and NCR Fwd class for uplink, and remove NCR class for downlink and NCR class for uplink as following.

## 4.3A Network controlled repeater classes ### 4.3A.1 Network controlled repeater Fwd class for downlink The requirements in this specification apply to downlink Wide Area NCR-Fwd, downlink Medium Range NCR-Fwd and downlink Local Area NCR-Fwd unless otherwise stated. The associated deployment scenarios for each class are exactly the same for NCR-Fwd with and without connectors. For NCR type 1-C, type 1-H and type 2-O, NCR-Fwd downlink classes are defined as indicated below: - Wide Area NCR-Fwd are characterised by requirements derived from Macro Cell scenarios with a NCR to UE minimum distance along the ground equal to 35 m. - Medium Range NCR-Fwd are characterised by requirements derived from Micro Cell scenarios with a NCR to UE minimum distance along the ground equal to 5 m. - Local Area NCR-Fwd are characterised by requirements derived from Pico Cell scenarios with a NCR to UE minimum distance along the ground equal to 2 m or from Femto Cell scenarios. - Note: The requirements in this specification for LA NCR-Fwd type 1-C apply to NCR-Fwd type 1-C with declared output power less than or equal to LA rated output power limits as in table 6.2.1-1. ### 4.3A.2 Network controlled repeater Fwd class for uplink The requirements in this specification apply to uplink Wide Area NCR-Fwd and uplink Local Area NCR-Fwd unless otherwise stated. The associated deployment scenarios for each class are exactly the same for NCR-Fwd with and without connectors. For NCR type 1-C, type 1-H and type 2-O, NCR-Fwd uplink classes are defined as indicated below: - Wide Area NCR-Fwd are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios. - Local Area NCR-Fwd are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios. ### 4.3A.3 Network controlled repeater MT class The requirements in this specification apply to Wide Area NCR-MT and Local Area NCR-MT unless otherwise stated. The associated deployment scenarios for each class are exactly the same for NCR-MT with and without connectors. For NCR type 1-C, type 1-H and type 2-O, NCR-MT classes are defined as indicated below: - Wide Area NCR-MT are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios. - Local Area NCR-MT are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios.

Proposal 1: Add definition of NCR-MT class, NCR-Fwd class for downlink, and NCR-Fwd class for uplink.

Based on above proposals, three CRs [1,2,3] for NCR classes are provided for approval.

- Conclusion

This contribution provides analysis on network controlled repeater classes for NCR. The following proposals and observations are concluded as follows:

Proposal 1: Add definition of NCR-MT class, NCR-Fwd class for downlink, and NCR-Fwd class for uplink.

Based on above proposals, three CRs [1,2,3] for NCR classes are provided for approval.

- References

- R4-2411126,(NR_netcon_repeater-Core)CR for TS 38.106, Correction on network controlled repeater classes for NCR, CATT, RAN4#112

- R4-2411127,(NR_netcon_repeater-Perf)CR for TS 38.115-1, Correction on network controlled repeater classes for NCR, CATT, RAN4#112

- R4-2411128,(NR_netcon_repeater-Perf)CR for TS 38.115-2, Correction on network controlled repeater classes for NCR,CATT, RAN4#112