3GPP TSG-RAN4 Meeting #112	R4-2411128

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-2 | CR | 0019 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_netcon_repeater-Perf)CR for TS 38.115-2, Correction on network controlled repeater classes for NCR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Definition of NCR MTclass, NCR-Fwd downlink class, and NCR-Fwd uplink class were missed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Change NCR class for downlink to NCR-Fwd class for downlink. 2) Change NCR class for uplink to NCR-Fwd class for uplink. 3) Add definition of NCR-MT class. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NCR classes would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3A.1, 4.3A.2, new 4.3A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 4.3A	NCR classes

4.3A.1	NCR Fwd class for downlink

The requirements in this specification apply to downlink Wide Area NCR-Fwd, downlink Medium Range NCR-Fwd and downlink Local Area NCR-Fwd unless otherwise stated. The associated deployment scenarios for each class are exactly the same for NCR-Fwd with and without connectors.

For NCR type 2-O, NCR-Fwd downlink classes are defined as indicated below:

-	Wide Area NCR-Fwd are characterised by requirements derived from Macro Cell scenarios with a NCR to UE minimum distance along the ground equal to 35 m.

-	Medium Range NCR-Fwd are characterised by requirements derived from Micro Cell scenarios with a NCR to UE minimum distance along the ground equal to 5 m.

-	Local Area NCR-Fwd are characterised by requirements derived from Pico Cell scenarios with a NCR to UE minimum distance along the ground equal to 2 m or from Femto Cell scenarios.

4.3A.2	NCR Fwd class for uplink

The requirements in this specification apply to uplink Wide Area NCR-Fwd and uplink Local Area NCR-Fwd unless otherwise stated. The associated deployment scenarios for each class are exactly the same for NCR-Fwd with and without connectors.

For NCR type 2-O, NCR-Fwd uplink classes are defined as indicated below:

-	Wide Area NCR-Fwd are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

-	Local Area NCR-Fwd are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios.

4.3A.3 Network controlled repeater MT class

The requirements in this specification apply to Wide Area NCR-MT and Local Area NCR-MT unless otherwise stated. The associated deployment scenarios for each class are exactly the same for NCR-MT with and without connectors.

For NCR type 2-O, NCR-MT classes are defined as indicated below:

-	Wide Area NCR-MT are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

-	Local Area NCR-MT are characterised by requirements derived from Pico Cell and/or Micro Cell scenarios.

<End of Change 1>