3GPP TSG-RAN WG4 Meeting #112                                                                                                      R4-2411129

Maastricht, Netherlands, 19th –23th August, 2024

Source: 	CATT

Title: 	Discussion on 5MHz channel bandwidth for NCR-MT

Agenda item:	5.31.1

Document for:	Approval

- Introduction

In this contribution, we provide our views on 5MHz channel bandwidth for NCR-MT.

- Discussion

5MHz channel bandwidth for NCR-MT

RAN4 was agreed to define 15kHz SCS and its related CBWs such as 5MHz for NCR-MT, reference sensitivity level for Wide Area NCR-MT type 1-C and 1-H in sub-clause 6.16.1 of TS 38.106 was copied as below:

### 6.16.1 NCR-MT reference sensitivity level #### 6.16.1.1 General The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H at which a throughput requirement shall be met for a specified reference measurement channel. #### 6.16.1.2 Minimum requirements for NCR-MT type 1-C and 1-H The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex B.1.5. The Wide Area NCR-MT reference sensitivity level is specified is table 6.16.1.2-1. Table 6.16.1.2-1: Wide Area NCR-MT reference sensitivity levels NCR-MT channel bandwidth (MHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 5,10,15; 15; G-FR1-A1-27 (Note 1); -101.5. 10, 15; 30; G-FR1-A1-22 (Note 1); -102.0. 10, 15; 60; G-FR1-A1-23 (Note 1); -99.0. 20, 25, 30, 35, 40, 45, 50; 15; G-FR1-A1-24 (Note 1); -95.2. 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; 30; G-FR1-A1-25 (Note 1); -95.4. 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; 60; G-FR1-A1-26 (Note 1); -95.6. NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full NCR-MT channel bandwidth.. The Local Area NCR-MT reference sensitivity level is specified the same as specified in TS 38.101-1 [13] clause 7.3. Referenced requirements applying to NB IoT are not applicable to the NCR-MT.

From above reference sensitivity level for Wide Area NCR-MT type 1-C and 1-H, compared with that for IAB-MT type 1-H in TS 38.174, the 15kHz SCS and its CBWs was not defined in TS 38.174, and compared with that for BS type 1-C and 1-H in TS 38.104, the reference sensitivity level for Wide Area NCR-MT type 1-C and 1-H are different from that for BS type 1-C and 1-H in TS 38.104. So for reference sensitivity level for Wide Area NCR-MT type 1-C and 1-H, defining to refer to TS 38.174 or TS 38.104 is not reasonable, so RAN4 define reference sensitivity level for Wide Area NCR-MT type 1-C and 1-H as stand-alone requirement.

## 6.18 Adjacent channel selectivity for NCR-MT ### 6.18.1 Adjacent Channel Selectivity (ACS) #### 6.18.1.1 General Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system. #### 6.18.1.2 Minimum requirement for NCR-MT type 1-C The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex B.1.5. For Wide Area NCR-MT type 1-C, minimum requirement is the same as specified for BS type 1-C in TS 38.104 [2], clause 7.4.1.2. For Local Area NCR-MT type 1-C, minimum requirement is the same as specified in TS 38.101-1 [13], clause 7.3.2. #### 6.18.1.3 Minimum requirement for NCR-MT type 1-H For Wide Area NCR-MT type 1-H, minimum requirement is the same as specified for IAB-MT type 1-H in TS 38.174 [22], clause 7.4.1.3. For Local Area NCR-MT type 1-H, minimum requirement is the same as specified in TS 38.101-1 [13], clause 7.3.2.

But for ACS, in-band blocking, and receiver intermodulation for NCR-MT in TS 38.106 as above, these requirement was defined to refer to TS 38.104 or 38.174. this method of referencing for these requirement is incorrect, Since the PREFSENS for wanted signal mean power for ACS, in-band blocking, and receiver intermodulation for NCR-MT should be refer to Table 6.16.1.2-1 of TS 38.106 instead of that in TS 38.104 or TS 38.174.

To solve above inconsistencies, it is reasonable to define ACS, in-band blocking, and receiver intermodulation for NCR-MT in TS 38.106 as stand-alone requirement instead of using reference method.

Proposal 1: Define ACS, in-band blocking, and receiver intermodulation for NCR-MT in TS 38.106 as stand-alone requirement instead of using reference method.

Based on above proposals, three CRs [1,2] for NCR classes are provided for approval.

- Conclusion

This contribution provides analysis on 5MHz channel bandwidth for NCR-MT. The following proposals and observations are concluded as follows:

Proposal 1: Define ACS, in-band blocking, and receiver intermodulation for NCR-MT in TS 38.106 as stand-alone requirement instead of using reference method.

Based on above proposals, three CRs [1,2] for NCR classes are provided for approval.

- References

- R4-2411130, (NR_netcon_repeater-Core)CR for TS 38.106, Correction on requirement for 5MHz channel bandwidth for NCR MT, CATT, RAN4#112

- R4-2411131, (NR_netcon_repeater-Perf)CR for TS 38.115-1, Correction on requirement for 5MHz channel bandwidth  for NCR MT,CATT, RAN4#112