3GPP TSG-RAN4 Meeting #112	R4-2411130

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | 0086 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_netcon_repeater-Core)CR for TS 38.106, Correction on requirement for 5MHz channel bandwidth for NCR MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Since the PREFSENS for wanted signal mean power for ACS, in-band blocking, and receiver intermodulation for NCR-MT should be refer to Table 6.16.1.2-1 of TS 38.106 instead of that in TS 38.104 or TS 38.174. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Add ACS requirement for Wide Area NCR-MT type 1-C and Type 1-H. 2) Add Blocking characteristics for Wide Area NCR-MT type 1-C and Type 1-H. 3) Add Receiver intermodulation characteristics for Wide Area NCR-MT type 1-C and Type 1-H. 4) Add Characteristics of the interfering signals for NCR-MT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ACS, blocking, and receiver intermodulation requirement would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.1.2, 6.18.1.3, 6.19.2, 6.19.3, 6.21.2, 6.21.3, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.115-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.18	Adjacent channel selectivity for NCR-MT

6.18.1	Adjacent Channel Selectivity (ACS)

6.18.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

6.18.1.2	Minimum requirement for NCR-MT type 1-C and 1-H

For Wide Area NCR-MT type 1-C and Type 1-H, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For NCR-MT, the wanted and the interfering signal coupled to the NCR-MT type 1-C antenna connector or NCR-MT type 1-H TAB connector are specified in table 6.18.1.2-1 and the frequency offset between the wanted and interfering signal in table 6.18.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 6.16.1.2-1 for each NCR-MT channel bandwidth and further specified in annex B.2. The characteristics of the interfering signal is further specified in annex F.

The ACS requirement is applicable outside the NCR-MT RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the NCR-MT RF Bandwidth edges or Radio Bandwidth edges.

For NCR-MT operating in non-contiguous spectrum within any operating band, the ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 6.18.1.2-2. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the ACS requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 6.18.1.2-2. The interfering signal offset is defined relative to the NCR-MT RF Bandwidth edges inside the Inter RF Bandwidth gap.

Minimum conducted requirement is defined at the antenna connector for NCR-MT type 1-C or the TAB connector for NCR-MT type 1-H.

Table 6.18.1.2-1: ACS requirement for NCR-MT

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area NCR-MT: -52 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the NCR-MT for that bandwidth. NOTE 2: PREFSENS depends on the NCR-MT channel bandwidth as specified in table 6.16.1.2-1. |  |  |

Table 6.18.1.2-2: NCR-MT ACS interferer frequency offset values

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5 MHz CP-OFDM NR signal 15 kHz SCS, 25 RBs |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 | 20 MHz CP-OFDM NR signal 15 kHz SCS, 100 RBs |
| 30 | ±9.4725 |  |
| 35 | ±9.4625 |  |
| 40 | ±9.4675 |  |
| 45 | ±9.4725 |  |
| 50 | ±9.4625 |  |
| 60 | ±9.4725 |  |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

For Local Area NCR-MT type 1-C and type 1-H, minimum requirement is the same as specified in TS 38.101-1 [13], clause 7.3.2.

6.18.1.3	Void

<End of Change 1>

<Start of Change 2>

6.19	Blocking characteristics for NCR-MT

6.19.1	General

The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the TAB connector for NCR-MT type 1-C and NCR-MT type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

6.19.2	Minimum requirement for NCR-MT type 1-C and 1-H

For Wide Area NCR-MT type 1-C and Type 1-H, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to NCR-MT type 1-C antenna connector or NCR-MT type 1-H TAB connector using the parameters in tables 6.19.2-1, 6.19.2-2 and 6.19.2-3 for general blocking and narrowband blocking requirements. The reference measurement channel for the wanted signal is identified in clause 6.16.1.2 for each NCR-MT channel bandwidth and further specified in annex B.2. The characteristics of the interfering signal is further specified in annex F.

The in-band blocking requirements apply outside the NCR-MT RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the NCR-MT RF Bandwidth edges or Radio Bandwidth edges.

The in-band blocking requirement shall apply from FDL,low - ΔfOOB to FDL,high + ΔfOOB. The ΔfOOB for wide area NCR-MT type 1-C and wide area NCR-MT type 1-H is defined in table 6.19.2-0.

Minimum conducted requirement is defined at the antenna connector for NCR-MT type 1-C or the TAB connector for NCR-MT type 1-H.

Table 6.19.2-0: ΔfOOB offset for NR operating bands

| NCR-MT type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
|  | FUL,high – FUL,low ≤ 200 MHz | 20 |
| NCR-MT type 1-C | 200 MHz < FUL,high – FUL,low ≤ 900 MHz | 60 |
|  | FUL,high – FUL,low < 100 MHz | 20 |
| NCR-MT type 1-H | 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz | 60 |

For an NCR-MT operating in non-contiguous spectrum within any operating band, the in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in tables 6.19.2-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 6.19.2-1.

For an NCR-MT operating in non-contiguous spectrum within any operating band, the narrowband blocking requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the channel bandwidth of the NR interfering signal in Table 6.19.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the narrowband blocking requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in Table 6.19.2-3. The interfering signal offset is defined relative to the NCR-MT RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 6.19.2-1: NCR-MT general blocking requirement

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | PREFSENS + 6 dB | Wide Area NCR-MT: -43 | ±7.5 | 5 MHz CP-OFDM NR signal 15 kHz SCS, 25 RBs |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area NCR-MT: -43 | ±30 | 20 MHz CP-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the NCR-MT channel bandwidth as specified in tables 6.16.1.2-1. |  |  |  |  |

Table 6.19.2-2: NCR-MT narrowband blocking requirement

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area NCR-MT: -49 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the NCR-MT for that NCR-MT channel bandwidth NOTE 2: PREFSENS depends on the NCR-MT channel bandwidth as specified in table 6.16.1.2-1. NOTE 3: 7.5 kHz shift is not applied to the wanted signal. |  |  |

Table 6.19.2-3: NCR-MT narrowband blocking interferer frequency offsets

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz CP-OFDM NR signal, 15 kHz SCS, 1 RB |
| 10 | ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | ±(360+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 | 20 MHz CP-OFDM NR signal, 15 kHz SCS, 1 RB |
| 30 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 35 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 40 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 45 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 50 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 60 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 70 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 80 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 90 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 100 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

For Local Area NCR-MT type 1-C and Type 1-H, minimum requirement is the same as specified in TS 38.101-1 [13] clause 7.6.2.

6.19.3	Void

<End of Change 2>

<Start of Change 3>

6.21	Receiver intermodulation characteristics for NCR-MT

6.21.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

6.21.2	Minimum requirement for NCR-MT type 1-C and 1-H

For Wide Area NCR-MT type 1-C and Type 1-H, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals coupled to the NCR-MT type 1-C antenna connector or NCR-MT type 1-H TAB connector, with the conditions specified in tables 6.21.2-1 and 6.21.2-2 for intermodulation performance and in tables 6.21.2-3, and 6.21.2-4 for narrowband intermodulation performance. The reference measurement channel for the wanted signal is identified in tables 6.16.1.2-1 for each NCR-MT channel bandwidth and further specified in annex B.2. The characteristics of the interfering signal is further specified in annex F.

The subcarrier spacing for the modulated interfering signal shall in general be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60 kHz and NCR-MT channel bandwidth <=20MHz, for which the subcarrier spacing of the interfering signal shall be 30 kHz.

The receiver intermodulation requirement is applicable outside the NCR-MT RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the NCR-MT RF Bandwidth edges or Radio Bandwidth edges.

For a NCR-MT operating in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement shall apply in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the NR interfering signal in table 6.21.2-2 or 6.21.2-4. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band connector, the intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the NR interfering signal centre frequency offset from the NCR-MT RF Bandwidth edge.

For a multi-band connector, the narrowband intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the NR interfering signal in tables 6.21.2-2 and 6.21.2-4. The interfering signal offset is defined relative to the NCR-MT RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 6.21.2-1: General intermodulation requirement for NCR-MT

| NCR-MT Type | Wanted Signal mean power (dBm) | Mean power of interfering signals (dBm) | Type of interfering signals |
| --- | --- | --- | --- |
| Wide Area NCR-MT | PREFSENS +6 dB (Note 1) | -52 | See Table 6.21.2-2 |
| NOTE 1: PREFSENS depends on the NCR-MT channel bandwidth as specified in table 6.16.1.2-1. |  |  |  |

Table 6.21.2-2: Interfering signals for intermodulation requirement for NCR-MT

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper NCR-MT RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 10 | ±7.465 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 15 | ±7.43 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 20 | ±7.395 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 25 | ±7.465 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 30 | ±7.43 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 40 | ±7.45 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 50 | ±7.35 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 60 | ±7.49 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 70 | ±7.42 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 80 | ±7.44 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 90 | ±7.46 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 100 | ±7.48 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| NOTE 1: Number of RBs is 25 for 15 kHz subcarrier spacing and 10 for 30 kHz subcarrier spacing. NOTE 2: Number of RBs is 100 for 15 kHz subcarrier spacing, 50 for 30 kHz subcarrier spacing and 24 for 60 kHz subcarrier spacing. NOTE 3: The RBs shall be placed adjacent to the transmission bandwidth configuration edge which is closer to the NCR-MT RF Bandwidth edge. |  |  |

Table 6.21.2-3: Narrowband intermodulation performance requirement in FR1 for NCR-MT

| NCR-MT type | Wanted signal mean power (dBm) | Interfering signalmean power (dBm) | Type of interfering signals |
| --- | --- | --- | --- |
| Wide Area NCR-MT | PREFSENS + 6dB (Note 1) | -52 | See Table 6.21.2-4 |
| NOTE 1: PREFSENS depends on the NCR-MT channel bandwidth as specified in table 6.16.1.2-1. |  |  |  |

Table 6.21.2-4: Interfering signals for narrowband intermodulation requirement in FR1 for NCR-MT

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±360 | CW |
|  | ±1420 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 10 | ±370 | CW |
|  | ±1960 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 15 (Note 2) | ±380 | CW |
|  | ±1960 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 20 (Note 2) | ±390 | CW |
|  | ±2320 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 25 (Note 2) | ±325 | CW |
|  | ±2350 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 30 (Note 2) | ±335 | CW |
|  | ±2350 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 40 (Note 2) | ±355 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 50 (Note 2) | ±375 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 60 (Note 2) | ±395 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 70 (Note 2) | ±415 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 80 (Note 2) | ±435 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 90 (Note 2) | ±365 | CW |
|  | ±2530 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 100 (Note 2) | ±385 | CW |
|  | ±2530 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the NCR-MT channel bandwidth of the interfering signal is located adjacently to the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 3: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

For Local Area NCR-MT type 1-C and Type 1-H, minimum requirement at antenna connector is the same as specified in TS 38.101-1 [13], clause 7.8.2.

6.21.3	Void

<End of Change 3>

<Start of Change 4>

Annex F (normative):

Characteristics of the interfering signals for NCR-MT

The interfering signal shall be configured with PDSCH and PDCCH containing data and DM-RS symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 7 of TS38.211 [8]. Mapping of PDSCH modulation to receiver requirement are specified in table F-1.

Table F-1: Modulation of the interfering signal

| Receiver requirement | Modulation |
| --- | --- |
| Adjacent channel selectivity and narrow-band blocking | QPSK |
| General blocking | QPSK |
| Receiver intermodulation | QPSK |

<En of Change 4>