3GPP RAN WG4 Meeting #112		R4-2411143

Maastricht, Netherlands, 19th – 23th August 2024

Agenda item:	8.2.4.2

Source:	Apple

Title:	TP on UE antenna parameters for 14800-15350MHz

WI/SI:	FS_NR_IMT_4400_7125_14800MHz

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

The WRC-23 conference identified three frequency ranges as potential IMT candidates and tasked ITU-R WP5D to conduct the corresponding sharing and compatibility studies. In turn, the ITU-R WP5D sent the LS to a number of bodies, including 3GPP, asking to provide technical parameters for the candidate frequencies [1]. In response to that LS from WP5D and based on the technical input from RAN WG4 [2], the 3GPP RAN#103 meeting approved a new SI [3], purpose of which will be to study and provide the corresponding IMT technical parameters.

During the RAN4#111 meeting an initial discussion took place and RAN WG4 concluded that for this frequency range both FR1- and FR2-like approaches for the UE RF architecture will be studied further. Thus, in this document we present a text proposal to capture considerations on the UE antenna design trade-offs.

# 2	Text proposal

-------------------------------------------- TP BEGIN --------------------------------------------

# 6	14800 - 15350 MHz frequency range

## 6.5	Antenna characteristics

### 6.5.1	BS antenna characteristics

#### 6.5.1.1		Antenna model

#### 6.5.1.2	Antenna parameters

### 6.5.2	UE antenna characteristics

One of the key issues for this range is the UE RF architecture. While existing FR1 system parameters are defined for frequencies up to 7.1GHz, same FR1 principles are already assumed for e.g. 7.1-8.4GHz, which is however below the considered range of 14.8-15.3GHz. At the same time, the lower bound for the FR2 starts at 24.25GHz, which is notably higher than 15.3GHz. A choice for a particular UE RF architecture at these frequency ranges is a trade-off between implementation feasibility for the handheld devices and anticipated performance and/or coverage. As an example, Figure 6.5.2-1 below (taken from TR 38.820) presents FR2 antenna size versus path loss as a function of the given frequency range. The capabilities of PA technology in the FR2 frequency range motivated 3GPP to consider a UE architecture which relies on panels performing analog beam-forming, which was also considered as a feasible option from the handheld device implementation perspective. On the contrary to it, panels and analog beam-forming is simply not feasible for lower FR1 frequencies.

Figure 6.5.2-1: FR2 type antenna size (free space) vs. operating frequency

In the context of the UE RF architecture, better UL coverage at FR2 frequencies was one of the key driving factors to consider the UE RF design with panels performing analog beam-forming, which was also considered as a feasible option from the handheld device implementation perspective. On the contrary to it, panels and analog beam-forming is not feasible for lower FR1 frequencies. At least when 3GPP was contemplating the 7.1-24GHz frequency range, preliminary conclusions and findings were captured in TR 38.820 with some concerns raised with regards to implementation feasibility of FR2 principles in hand-held devices operating frequency ranges around 10-15GHz. The main obstacle is the size of potential FR2 panel and, as noted in TR 38.820, "… we can see that the antenna module doubles in size going from 28 GHz to 20 GHz and further doubles at 14 GHz". Figure 6.5.2-2 presents the patch width and ground plane extension based on the microstrip patch array design assuming four elements with half-wavelength spacing.

Figure 6.5.2-2: Patch array dimensions vs. frequency

When analog beamforming is prescribed, control of the element radiation patterns is required, therefore dedicated antenna area is required with less opportunity for space sharing or miniaturization.  Use of the bezel is not generally feasible.  Figure 6.5.2-3 below illustrates a survey of the internal area used by four different commercially available 5G devices for FR1.  The FR2 area is calculated based a single 4x1 reference design, the configuration used as the basis for current FR2 spherical coverage requirements.  Additionally, an estimate of the required FR3 antenna array area is provided for comparison, similarly based on a 4x1 microstrip patch array with half-wavelength spacing. So, assuming dedicated apertures, an estimated FR3 4x1 array at 15 GHz would represent an additional area growth on the order of 30-70% relative to antenna area usage currently needed to support FR1. This would mean support of analog beamforming in FR3 would impose a substantial growth and volume requirements dedicated for cellular antennas.  Furthermore, the nature of such an array constrains the dimensions of the implementation, making it substantially more challenging to implement in densely integrated consumer devices dedicated for cellular antennas.

Figure 6.5.2-3: A survey of internal area usage for FR1, FR2 and estimated FR3 antennas

So, from the practical perspective, it is not a straightforward task to put the FR2-like panel supporting the 15GHz range into the hand-held device, which is likely to support all the existing features. In other words, it is difficult to assume that there will be extra space in all physical dimensions. From that perspective the FR1-like approach is more practical and provides more flexibility in deciding how and where the antennas will be placed.

-------------------------------------------- TP END --------------------------------------------

# 3	References

RP-240024, "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz", ITU-R WP5D

R4-2102840, "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)", RAN WG4.

RP-240765, "Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz"

3GPP TR 38.921, "Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz", v17.1.0