3GPP RAN WG4 Meeting #112																	R4-2411144

Maastricht, Netherlands, 19th – 23th August 2024

Agenda item:	8.8.2.2

Source:	Apple

Title:	On restricted set of A-MPR simulations for the satellite band UE Tx requirements

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI.

So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band where the UL is on the L-band and the DL is on the S-band [2]. And a so-called extended L-band was also completed in Rel-18. After the RAN#104 meeting another spectrum WI was agreed for a satellite band operating in the S-band frequency range [3]. In fact, it is anticipated that more satellite bands will be defined by 3GPP soon as the outcome of the corresponding regulatory decisions, especially the ones that allow usage of the satellite services in the existing terrestrial band ranges.

Every time 3GPP decides to standardise a new band, all the regulatory requirements are carefully checked. And if there is a need to protect other services running in the neighbour frequencies, then 3GPP also conducts a set of A-MPR simulations to define whether additional power back-off is needed. In fact, the regulatory requirements have to be checked not only for new bands, but also for existing ones if 3GPP decides to adopt higher power classes; for instance, PC2 has been agreed a new high power class for all FR1 NTN bands that will trigger the corresponding work in RAN WG4 [4].  However, running A-MPR simulations and defining the corresponding back-off values is a non-trivial task that requires a lot of offline efforts and online discussions to calibrate results, agree on a common baseline from which a harmonized set of the power back-off values is derived. However, some of the A-MPR simulations and/or scenarios are not really needed for the satellite bands. Thus, in this discussion paper we present a set of simplifications that can be applied to all satellite bands thus saving a lot of time and efforts on A-MPR simulations.

# 2	A-MPR simulations for satellite bands

## 2.1	General considerations

While running A-MPR simulations for any given (satellite) band, usually all the possible combinations are tested that usually involve the following key aspects:

-	Channel bandwidth(s). If a band is larger than 5MHz, then A-MPR simulations are run for every standard channel bandwidth.

-	Channel bandwidth placement. If a band is larger than the considered channel bandwidth, then companies usually simulate several different channel bandwidth locations within the band because inner channel usually need smaller A-MPR.

-	RB start and number of RBs. For any given channel bandwidth at a particular location within the band, RAN4 usually simulates all possible combinations of the RB allocations and where they start.

-	Waveforms. While running simulations RAN4 checks performance of all the existing waveforms, i.e. DFT-s-OFDM and CP-OFDM.

-	Higher-order modulations. While running initial A-MPR simulations RAN4 usually focuses first on more important modulations, such as PI/2 BPSK and QPSK. However, also higher order modulations should be explicitly simulated instead of applying some fixed relaxation.

-	Power class. Since the amount of power back-off depends on the maximum UE Tx power and the PA assumptions, the corresponding A-MPR simulations are conducted for PC3 as a baseline and, if applicable, also for higher power classes. In Rel-19 3GPP agreed to enable higher power classes also for NTN, and thus A-MPR simulations will have to repeated for existing bands and will have to be conducted for new bands.

So as can be seen, it is a non-trivial number of A-MPR simulations that must be run to test all possible combinations of the aforementioned parameters. And since some of the combinations/parameters are not strictly needed for the satellite bands, in the next section we elaborate more on which enhancements can be applied to alleviate that burden.

## 2.2	A-MPR simplifications for the satellite bands

Unlike terrestrial bands, the satellite bands are characterised by a much more challenging link budget, especially in the UL direction. And to achieve a proper link budget in UL, the network ensures scheduling in such a way that the corresponding modulation schemes are used in conjunction with a limited number of scheduled RBs that provides better PSD at the satellite node receiver. Based on these observations it is possible to reduce considerably number of A-MPR simulations by not treating some scenarios and combinations which to our understanding are anyway not practical for satellite systems.

-	High-order modulations. Because of the stringent UL budget, it is almost impossible that 64QAM or 256QAM will be used with handheld devices. In fact, it can be argued that even 16QAM will not be widely activated for a handheld like devices. That will simplify A-MPR simulations as RAN4 will not waste time on simulating them.

-	CP-OFDM. According to a number of the A-MPR simulations, CP-OFDM performance is much worse comparing to DFT-s-OFDM and PI/2 BPSK when the emission requirements are concerned. So to avoid unnecessary power back-off, an operator would naturally choose either DFT-s-OFDM with PI/2 BPSK or QPSK, not CP-OFDM. Thus, by removing CP-OFDM from A-MPR simulations will reduce noticeably simulations efforts because for every combination of RB start and number of RBs RAN4 can focus simulations on DFT-s-OFDM and PI/2 BPSK / QPSK. Referring to Figure 2.2-1 below as an example, while the DFT-s-OFDM QPSK and PI/2 BPSK has the maximum back-off values of 2.5..3.5dB, CP-OFDM maximum power back-off can be up to 6dB.

-	RB allocations. As mentioned earlier, while running A-MPR simulations RAN4 usually tests all possible combinations of where the allocation starts within the channel and its size. However, it is not likely the case that large allocations will be used for the satellite bands because otherwise the resulting PSD will be quite low. Thus, RAN4 can limit the maximum RB allocation size that will reduce a lot number of A-MPR simulations.  As can be seen from Figure 2.2-1, if a band has strict out-of-band emission requirements, then large RB allocations are the ones that will have highest back-off values, not mentioning the fact that large RB allocations will have very low PSD not suitable for the satellite channels. From that perspective we can limit the maximum RB size, i.e. LCRB, to a more reasonable value such as 4..6 RBs.

Figure 2.2-1: Power back-off values for band n254 the 5MHz channel at Fc=1612.5MHz with PC3.

Table 2.2-1 gives a numerical overview of how many A-MPR simulations we need to run just a single case of the given channel bandwidth if the maximum UL allocations are not restricted, i.e. it can be as large as the channel bandwidth, and if both waveforms (CP-OFDM, DFT-s-OPFDM) and all modulations have to be tested. The table also shows that if we limit the maximum allocation size to e.g. 6RB and test only DFT-s-OFDM with QPSK and PI/2 BPSK, then the total number of simulations reduces dramatically, especially for those NTN bands that can accommodate large channels. And it is worth noting that numbers in Table 2.2-1 are for a single case of the given channel residing at the given center frequency. Depending on the band and requirements, the same channel bandwidth is simulated several times for different center frequencies.

Table 2.2-1: Summary of A-MPR simulations needed for a single case of the given channel bandwidth.

| Channel |  | N_RB |  | Unrestricted UL allocations (maximum allocation = N_RB) |  |  | Restricted UL allocations (maximum allocation = 6 RB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DFT-s-OFDM | CP-OFDM |  | DFT-s-OFDM / CP-OFDM | DFT-s-OFDM with PI/2 BPSK and QPSK |
| 5MHz |  | 25 |  | 238 | 325 |  | 135 | 270 |
| 10MHz |  | 52 |  | 786 | 1378 |  | 297 | 594 |
| 15MHz |  | 79 |  | 1509 | 3160 |  | 459 | 918 |

Proposal:	To reduce number of A-MPR simulations for the FR1 satellite bands, we suggest not considering scenarios/combinations that are not expected for the satellite bands: large RB allocations, CP-OFDM, higher-order modulations.

# 3	Conclusions

In this discussion paper we have presented our initial considerations on a potential way to reduce the number of A-MPR simulations that RAN WG4 runs to determine and define required power back-off values for the satellite bands. As explained in the discussion paper, since the satellite bands have quite tight link budget, especially in the UL direction, there is no practical need to simulate and define A-MPR values for large RB allocations, higher order modulations and some waveforms such as CP-OFDM.

Proposal:	To reduce number of A-MPR simulations for the FR1 satellite bands, we suggest not considering scenarios/combinations that are not expected for the satellite bands: large RB allocations, CP-OFDM, higher-order modulations.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

- RP-241661, "New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)", DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"