3GPP RAN WG4 Meeting #112		R4-2411146

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	10.1.1 UE RF specifications TS 38.101-1/-2/-3

Source:	Apple

Title:	Simplifying ΔRIB,c and ΔTIB,c tables

WI/SI:	-

Release:	Rel-19

Document for:	Approval

# 1	Introduction

In RAN4#111 we proposed to simplify the ΔRIB,c and ΔTIB,c specifications by replacing the large tables with the ΔRIB,c and ΔTIB,c values for each DL band combination with a generic number. In this contribution we want to indicate how this would change the specs and discuss how this can be implemented in the specs.

# Discussion

## 2.1	Discussion results in the last meeting

When presenting R4-2407225, we got a very positive feedback on the proposal to simplify the ΔRIB,c and ΔTIB,c tables by replacing them by generic values, since we described in detail that the the ΔRIB,c and ΔTIB,c values have developed over time towards not being useful anymore, as they are not based on physics. Basically everyone seemed to be positive about the proposal. Unfortunately the protocol of this discussion was lost in the crash of the chairman minutes. Some companies proposed we should do this change to the Rel. 18 specs already, not waiting for Rel. 19. Therefore we need agree in RAN4, if we want to continue to remove the ΔRIB,c and ΔTIB,c tables and replace them with a table with generic values and for which release.

## 2.2	Proposals to agree

As already proposed in R4-2407225, RAN4 should agree to replace the ΔRIB,c and ΔTIB,c tables by the tables below (shown for 38.101-1):

Table 7.3A.3.2-1: ΔRIB,c due to NR CA

| Number of bands in DL configuration | ΔTIB,c [dB] |
| --- | --- |
| 2 | 0.0 |
| 3 | 0.2 |
| 4 | 0.3 |
| 5 | 0.4 |
| 6 | 0.5 |

Table 6.2A.4.2-1: ΔTIB,c due to NR CA

| Number of bands in DL configuration | ΔTIB,c [dB] |
| --- | --- |
| 2 | 0.1 |
| 3 | 0.2 |
| 4 | 0.3 |
| 5 | 0.4 |
| 6 | 0.5 |

The annex includes an exemplary draft CR for ΔRIB,c in 38.101-1, the other CRs, for ΔTIB,c in 38.101-1 and those for 38.101-3 will be very similar. If RAN4 can agree to the replacement of the tables, the real CRs will be brought to the next meeting.

Proposal 1: RAN4 to agree to remove the ΔRIB,c and ΔTIB,c tables in 38.101-1 and 38.101-3 and replace them by generic values based on the number of bands in the DL configuration as exemplarily shown below:

Table 7.3A.3.2-1: ΔRIB,c due to NR CA

| Number of bands in DL configuration | ΔTIB,c [dB] |
| --- | --- |
| 2 | 0.0 |
| 3 | 0.2 |
| 4 | 0.3 |
| 5 | 0.4 |
| 6 | 0.5 |

Table 6.2A.4.2-1: ΔTIB,c due to NR CA

| Number of bands in DL configuration | ΔTIB,c [dB] |
| --- | --- |
| 2 | 0.1 |
| 3 | 0.2 |
| 4 | 0.3 |
| 5 | 0.4 |
| 6 | 0.5 |

Additionally RAN4 needs to decide to which release we do the change. If, as proposed by some companies in the last meeting, we want to do this to Rel. 18, we can just bring Rel. 18 maintenance CRs to the next meeting. In case of Rel. 19, we need to do CRs against the upcoming Rel. 19 spec.

Proposal 2: RAN4 to agree to replace the ΔRIB,c and ΔTIB,c tables in 38.101-1 and 38.101-3 in Rel. 18 as maintenance CRs.

Proposal 3: If Proposal 2 cannot be agreed, RAN4 to agree to replace the ΔRIB,c and ΔTIB,c tables in 38.101-1 and 38.101-3 in Rel. 19.

# 3	Conclusion

In this contribution we have several proposals to implement the replacement of the ΔRIB,c and ΔTIB,c tables.

Proposal 1: RAN4 to agree to remove the ΔRIB,c and ΔTIB,c tables in 38.101-1 and 38.101-3 and replace them by generic values based on the number of bands in the DL configuration as exemplarily shown below:

Table 7.3A.3.2-1: ΔRIB,c due to NR CA

| Number of bands in DL configuration | ΔTIB,c [dB] |
| --- | --- |
| 2 | 0.0 |
| 3 | 0.2 |
| 4 | 0.3 |
| 5 | 0.4 |
| 6 | 0.5 |

Table 6.2A.4.2-1: ΔTIB,c due to NR CA

| Number of bands in DL configuration | ΔTIB,c [dB] |
| --- | --- |
| 2 | 0.1 |
| 3 | 0.2 |
| 4 | 0.3 |
| 5 | 0.4 |
| 6 | 0.5 |

Proposal 2: RAN4 to agree to replace the ΔRIB,c and ΔTIB,c tables in 38.101-1 and 38.101-3 in Rel. 18 as maintenance CRs.

Proposal 3: If Proposal 2 cannot be agreed, RAN4 to agree to replace the ΔRIB,c and ΔTIB,c tables in 38.101-1 and 38.101-3 in Rel. 19.

# 4	References

R4-2407225 “Simplifying RX/TX RF Refsens/Pout tables”, Apple, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

# 5	Annex

## 5.1	Draft CR for 38.101-1 to replace  ΔRIB,c tables

#### 7.3A.3.2	ΔRIB,c for Inter-band CA

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in clause 7.3A.2 shall be increased by the amount given by ΔRIB,c defined in table 7.3A.3.2-1 for the applicable number of operating bands.

Table 7.3A.3.2-1: ΔRIB,c due to NR CA

| Number of bands in DL configuration | ΔTIB,c [dB] |
| --- | --- |
| 2 | 0.0 |
| 3 | 0.2 |
| 4 | 0.3 |
| 5 | 0.4 |
| 6 | 0.5 |

##### 7.3A.3.2.1	Void

##### 7.3A.3.2.2	Void

##### 7.3A.3.2.3	Void

##### 7.3A.3.2.4	Void

##### 7.3A.3.2.5	Void

##### 7.3A.3.2.6	Void