3GPP RAN WG4 Meeting #112		R4-2411147

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	10.1.1	UE RF specifications TS 38.101-1/-2/-3

Source:	Apple

Title:	Simplifying or removing MSD tables

WI/SI:	-

Release:	Rel-19

Document for:	Approval

# 1	Introduction

Since Rel. 10, when inter-band LTE CA was introduced, 3GPP has introduced the concept of exceptions to prevent UEs from failing the CA Refsens test in case a harmonic or IMD of the TX falls into the RX channel and disturbs the reception. Finally this led to the Maximum Sensitivity Degradation (MSD) concept, that has been introduced into the NR specs from the beginning for all CA and DC cases. However, meanwhile the effort has been exploding with the explosive growth of the number of band combinations, more and more carriers and additional features like combining LTE and NR (EN-DC). This results in an excessive work load in RAN4 and also in development and test of UEs. So it is time to re-think how useful the MSD concept is and how to reduce the work load.

# Discussion

## 2.1	Status of the current MSD concept

Currently the MSD definition in RAN4 specs is needed to define exceptions to prevent Refsens tests for CA or DC combinations to be done on frequencies where harmonics or IMD of the own TX will impact the measurement. We have multiple of these tables defining exceptions from the normal Refsens tests for specific use cases, in which such Refsens degradations could result in a fail of the CA/DC Refsens test.

Most of the cases with MSD are defined for inter-band NR-CA and EN-DC within FR1, the following discussions are not intended to change anything for FR1+FR2 or intra-band CA/DC combinations. This means we are discussing 38.101-1 clauses 7.3A.2.3, 7.3A.4, 7.3A.5, 7.3A.6, 7.3A.7 and 38.101-3 clauses 7.3B.1 and 7.3B.2.3.

Observation 1: MSD tables are used to define exceptions from the normal CA/DC Refsens test in case of harmonic/IMD/cross-band isolation issues.

Additionally to defining the exceptions, the MSD tables in most cases also define how much Maximum Sensitivity Degradation (MSD) is allowed in the worst case, however, the worst case is usually not the use case of the network operator, simply because this is the worst case for the network performance of the CA/DC configuration. Instead the MSD values and their scenarios should be avoided by intelligent network scheduling for a better network performance.

Observation 2: MSD tables describe the worst case scenario, which is usually not used in the network due to the worst case network performance. The MSD information in the spec is not helpful in this case

The specified MSD case is just one worst case scenario, but there will be many more configurations of carrier frequencies and bandwidths that will result in large sensitivity degradation, although may be not as much as the specified MSD case. However, the MSD is only specified for the single MSD use case, not for the use case deployed in the network, so for the really existing use case there is no indication how much sensitivity degradation to expect in the deployment scenario.

Observation 3: The network cannot use the MSD values from the spec, as there is no information available for the exact use case in the network (frequencies, bandwidths, RB allocations, output power, RX performance etc.)

## 2.2	Testing MSD

In the RAN5 spec 38.521-1 the Refsens is tested for combinations without exceptions, the combinations with exceptions are measured separately.

Observation 4: As specified in 38.101-1, in the RAN5 specs (38.521-1) the NR-CA Refsens value is measured for the cases without exceptions. The cases with Refsens exceptions using the worst case values from the MSD tables is tested separately.

In the RAN5 spec 38.521-3 the EN-DC Refsens is not tested for combinations without exceptions, there is just a reference to the 36.101 and 38.101-1 specs, however, the combinations with exceptions are measured separately.

Observation 5: Although specified in 38.101-3, in the RAN5 specs (38.521-3) the EN-DC Refsens value without exceptions is not measured. Instead the test spec is pointing to the E-UTRA and NR specs. RAN5 only tests the Refsens exceptions with the worst case values from the MSD tables. There cannot be a fail when the MSD spec is missing, as there is no EN-DC test.

The tests in 38.101-1 and 38.101-3 are conducted tests, where the antenna ports are connected to the UE with cables. However, most of the effects resulting in MSD are radiated effects, for example two TX antennas coupling with each other generating reverse IMD or IMD products leaking from one TX/RX antenna to the other. Also coupling on the PCB plays a significant role here. Therefore in many cases a conducted measurement is not resulting in realistic measurement results.

Observation 6: Sensitivity degradation in many cases happens due to TX power leaking between the TX antenna(s) and the RX antenna(s) due to limited antenna isolation, but MSD is only specified and tested in conducted mode, not taking into account the radiated effects.

The Reference Sensitivity measurement is intended to determine the lowest signal level the RX can detect, this is usually limited by the noise figure of the RF frontend. But the noise figure of the frontend is already measured with the single carrier Refsens, and it varies by just a few 10ths of a dB when switching to CA or DC. Therefore it is questionable, why we need to measure Refsens in NR-CA mode, especially as we already skip that measurement for the EN-DC mode in the RAN5 tests.

Observation 7: Refsens is usually a measurement to determine the noise figure of the receiver, but the NF is already measured in single carrier mode. Therefore Refsens tests for NR-CA could be skipped, similarly as Refsens for EN-DC is already skipped.

## 2.3	Network usage of MSD specs

If the network could use the MSD information in the specs to optimize the network scheduling to avoid the cases with the maximum sensitivity degradation and instead use configurations with less sensitivity degradation, the MSD information in the specs would be helpful. However, the real use case (carrier frequencies, bandwidths, RB allocations, output power, RX performance etc.) deviates from the values specified as worst case sensitivity degradation (MSD) specified in the specs. This limits the usability of the MSD information in the specs making it quite useless. Additionally the tables are so complex that they cannot really be used to optimize network scheduling.

Observation 8: The MSD tables do not contain the use cases needed in the network and are so huge and complex that the information from the MSD tables cannot be used by the network to optimize the network scheduling to avoid MSD issues.

In the networks a sensitivity degradation problem can only be detected with the failure of ACK/NACK or indications in the measurement reports. This can take quite a while. Then the BS needs to modify the band combinations, the RB allocations or reduce output power to overcome the issue

Observation 9: If the connection fails due to harmonic or IMD issues, the BS will become aware of the failure by ACK/NACK or the measurement reports, this may take a while.

Observation 10: Once the BS detects the failure, the BS needs to react and try another CA combination, RB allocation or lower TX power to reduce the IMD/harmonic issues.

It may take some time until the BS can detect the harmonic/IMD issue using measurement reports or by ACK/NACK failures. In the meantime there will be some slots that contain corrupt data, before the BS changes the carrier frequency, RB allocation, TX Power or modulation. This can be improved, if the UE does a real time reporting of harmonic/IMD issues and signals the issue, once it is scheduled with a critical configuration. Such a real time reporting of harmonic/IMD issues is proposed in [1].

Observation 11: Using the measurement reports to detect harmonic/IMD issues is quite slow. To increase the speed of the network to react on harmonic/IMD issues, it would be better to implement a real time measurement reporting of harmonic/IMD issues as proposed in [1].

## 2.4	Improvements and simplifications of MSD specs

Summarizing the status with the currently specified MSD values we see the following issues:

- The currently specified MSD values do not represent the use cases in the network

- The currently specified MSD values cannot be used by the network

- The MSD values are specified conducted, but MSD is mainly a radiated phenomenon

- The Refsens for EN-DC combinations is mentioned in RAN4 specs but not tested in RAN5 specs, therefore no exceptions with MSD values are needed

- The Refsens for NR-CA combinations  is specified in RAN4 and tested in RAN5 specs, however, Refsens is intended to be a noise figure test, and this is already tested with single carriers, as EN-DC Refsens is not tested, NR-CA Refsens may also be removed, then no exceptions with MSD values are needed anymore

- Specifying MSD for all the band combinations creates a huge work load in 3GPP, but the result is not used anywhere

Considering the situation as described above, that the MSD values in the specification are basically useless, it seems the Specs can be improved by simplifying or even removing the MSD tables.

We could remove the Refsens specification for the FR1 inter-band NR-CA and EN-DC combinations in 38.101-1 clauses 7.3A.2.3, 7.3A.4, 7.3A.5, 7.3A.6, 7.3A.7 and 38.101-3 clauses 7.3B.1 and 7.3B.2.3 and refer to the single carrier measurements for the Refsens/Noise figure measurements. In case we remove the CA/DC Refsens spec, IMD and/or harmonic issues will not result in failed Refsens measurements anymore and we will not need the exceptions anymore. nThis would also reduce the effort for new band combinations development in RAN4 significantly, as no IMD/harmonics calculations would be needed anymore.

Proposal 1: Remove the Refsens specifications in 38.101-1 clauses 7.3A.2.3, 7.3A.4, 7.3A.5, 7.3A.6, 7.3A.7 and 38.101-3 clauses 7.3B.1 and 7.3B.2.3 including the MSD tables for CA/DC combinations replacing them with a reference to the single band Refsens measurements

In case removing the Refsens specs is not agreeable, we need to keep the Refsens specs. However, for EN-DC combinations the plain Refsens measurements without exceptions are not specified in 38.521-3, but for NR-DC they are specified in 38.521-1, so at least for NR-DC we then still need the exceptions. Alternatively we may add a note to the Refsens specification that Refsens is not defined for combinations for which harmonics/IMD are falling onto the receive channel or there is low cross-band isolation.

Proposal 2: If Proposal 1 is not agreeable, we can keep the general Refsens specification but with a normative note that the Refsens values are only applicable in case there is no IMD or harmonic or cross-band isolation issue. The MSD tables can be removed.

If the note excluding the IMD/harmonic/cross-band isolation cases is not agreeable, too, we need to keep the exceptions. However, since the values for the MSD definition are not used or useful, we can simplify the tables by replacing them with a table just listing the combinations as exceptions to exempt them from Refsens measurements. So we can replace the MSD tables by a list of combinations with exceptions.

Proposal 3: If both, Proposal 1 and Proposal 2, are not agreeable, we can simplify the exception tables by removing the MSD tables and replacing them with tables just listing the combinations having an exception, but without specifying not needed MSD values nor specifying MSD test conditions.

In case Proposal 3 is not agreeable, it would be possible to add the simplified exception table as described for Proposal 3, but, instead of removing the MSD tables, keep some exemplary band combinations to check the IMD/harmonics/cross-band isolation performance of the RF frontend and remove all but the exemplary combinations fro the MSD tables to significantly shrink the MSD tables and the effort in 3GPP.

Proposal 4: If  Proposal 1, 2 and 3, are not agreeable, we can simplify the MSD tables by removing most of the MSD combinations keeping only some exemplary combinations and add a separate table listing all the combinations having an exception, but without specifying MSD values or MSD test conditions.

# 3	Conclusion

In this contribution we summarize the issues of the current MSD specifications. It is shown that the currently specified MSD values are not useful at all, they are defined in a way that the values cannot be used for any purpose. We propose ways to simplify the specs by removing the MSD tables.

Observation 1: MSD tables are used to define exceptions from the normal CA/DC Refsens test in case of harmonic/IMD/cross-band isolation issues.

Observation 2: MSD tables describe the worst case scenario, which is usually not used in the network due to the worst case network performance. The MSD information in the spec is not helpful in this case

Observation 3: The network cannot use the MSD values from the spec, as there is no information available for the exact use case in the network (frequencies, bandwidths, RB allocations, output power, RX performance etc.)

Observation 4: As specified in 38.101-1, in the RAN5 specs (38.521-1) the NR-CA Refsens value is measured for the cases without exceptions. The cases with Refsens exceptions using the worst case values from the MSD tables is tested separately.

Observation 5: Although specified in 38.101-3, in the RAN5 specs (38.521-3) the EN-DC Refsens value without exceptions is not measured. Instead the test spec is pointing to the E-UTRA and NR specs. RAN5 only tests the Refsens exceptions with the worst case values from the MSD tables. There cannot be a fail when the MSD spec is missing, as there is no EN-DC test.

Observation 6: Sensitivity degradation in many cases happens due to TX power leaking between the TX antenna(s) and the RX antenna(s) due to limited antenna isolation, but MSD is only specified and tested in conducted mode, not taking into account the radiated effects.

Observation 7: Refsens is usually a measurement to determine the noise figure of the receiver, but the NF is already measured in single carrier mode. Therefore Refsens for NR-CA could be skipped, similarly as Refsens for EN-DC is already skipped.

Observation 8: The MSD tables do not contain the use cases needed in the network and are so huge and complex that the information from the MSD tables cannot be used by the network to optimize the network scheduling to avoid MSD issues.

Observation 9: If the connection fails due to harmonic or IMD issues, the BS will become aware of the failure by ACK/NACK or the measurement reports, this may take a while.

Observation 10: Once the BS detects the failure, the BS needs to react and try another CA combination, RB allocation or lower TX power to reduce the IMD/harmonic issues.

Proposal 1: Remove the Refsens specifications in 38.101-1 clauses 7.3A.2.3, 7.3A.4, 7.3A.5, 7.3A.6, 7.3A.7 and 38.101-3 clauses 7.3B.1 and 7.3B.2.3 including the MSD tables for CA/DC combinations replacing them with a reference to the single band Refsens measurements

Proposal 2: If Proposal 1 is not agreeable, we can keep the general Refsens specification but with a normative note that the Refsens values are only applicable in case there is no IMD or harmonic or cross-band isolation issue. The MSD tables can be removed.

Proposal 3: If both, Proposal 1 and Proposal 2, are not agreeable, we can simplify the exception tables by removing the MSD tables and replacing them with tables just listing the combinations having an exception, but without specifying not needed MSD values nor specifying MSD test conditions.

Proposal 4: If  Proposal 1, 2 and 3, are not agreeable, we can simplify the MSD tables by removing most of the MSD combinations keeping only some exemplary combinations and add a separate table listing all the combinations having an exception, but without specifying MSD values or MSD test conditions.

# 4	References

R4-2411174 “Motivation for MSD reporting enhancement in Rel. 19”, Apple, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024