3GPP RAN WG4 Meeting #112		R4-2411148

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	7.12.2	UE RF requirements

Source:	Apple

Title:	n48 100MHz UL

WI/SI:	NR_bands_CA_ENDC_R19_BWs-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

In RAN#104 a basket WI was agreed adding 50, 60, 70, 80, 90 and 100MHz UL Channel Bandwidth [1]. In this contribution we have a closer look at the RF issues with this addition.

# Discussion

## 2.1	Current situation for n48 UL

When LTE band 48 and NR band n48 were introduced, the UL BW was intentionally limited to 40MHz. The main reason is that there is a stringent RF emissions requirement from the FCC for the band. This is reflected in 38.101-1 in an additional spectrum emissions mask as well as an additional spurious emissions requirement. This is the emissions mask requirement in 38.101-1:

##### 6.5.2.3.8	Requirements for network signalling value "NS_27"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_27" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.8-1.

Table 6.5.2.3.8-1: Additional requirements for "NS_27"

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 30 | 40 |  |
| ± 0 - 1 | -13 |  |  |  |  |  | 1 % channel bandwidth |
| ± 1 - X | -13 |  |  |  |  |  | 1 MHz |
| < – X or > X | -25 |  |  |  |  |  |  |
| NOTE 1: X is occupied channel bandwidth as defined in Table 6.5.1-1. NOTE 2: The requirements apply only at the frequency range from 3540 MHz to 3710 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

This is the additional spurious emissions requirement for n48:

##### 6.5.3.3.14	Requirement for network signalling value "NS_27"

When "NS_27" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.14-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.14-1: Additional requirements for "NS_27"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 30, 40 |  |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

To be able to fulfill the additional emissions requirements by the FCC, the UE needs to reduce its output power to reduce the intermodulation products falling into the protected band. The output power reduction can be quite large. This is the A-MPR that is specified in 38.101-1:

#### 6.2.3.16	A-MPR for NS_27

Table 6.2.3.16-1: A-MPR for NS_27

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  | Region B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS | A-MPR | LCRB*12*SCS | A-MPR |
| 15 MHz | 3557.5 ≤ FC < 3562.5 | <1.8 MHz |  |  | A3 | ≥10.8 MHz | A3 |
|  | 3687.5 < FC ≤ 3692.5 | >11.52 MHz |  |  |  |  |  |
| 15 MHz | 3562.5 ≤ FC < 3567.5 | ≤1.08 MHz |  | <1.44 MHz | A4 | ≥11.52 MHz | 2 |
|  | 3682.5 < FC ≤ 3687.5 |  | ≥13.22 MHz |  |  |  |  |
| 20 MHz | 3560 ≤ FC < 3570 | <3.6 MHz |  |  | A5 | ≥10.8 MHz | A5 |
|  | 3680 < FC ≤ 3690 | >12.96 MHz |  |  |  |  |  |
| 20 MHz | 3570 ≤ FC < 3580 | ≤2.16 MHz |  | <1.44 MHz | A6 | ≥14.4 MHz | 2 |
|  | 3670 < FC ≤ 3680 |  | ≥16.92 |  |  |  |  |
| 30 MHz | 3565 ≤ FC < 3585 | < 7.38MHz |  |  | A7 |  |  |
|  |  | ≥ 7.38MHz ≤24.48MHz |  | ≥ 15.3MHz | A2 |  |  |
|  |  |  |  | < 15.3 MHz | A1 |  |  |
|  |  | ≥24.48MHz |  | < 2.7 MHz | A7 |  |  |
|  |  |  | > 19.44 MHz |  | A7 |  |  |
|  | 3665 < FC ≤ 3685 |  | ≤19.44MHz ≥3.24 MHz | ≥ 15.3 MHz | A2 |  |  |
|  |  |  |  | < 15.3 MHz | A1 |  |  |
|  |  |  | <3.24 MHz | < 2.7MHz | A7 |  |  |
|  | 3585 ≤ FC ≤ 3665 | ≤[3.96] MHz |  | < 1.44MHz | A8 | ≥19.44 MHz | 4 |
|  |  |  | ≥24.48MHz |  | A8 |  |  |
| 40 MHz | 3570 ≤ FC < 3600 | <11.34 MHz |  |  | A7 |  |  |
|  |  | ≥11.34 MH, ≤31.0 MHz |  | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  | >31.0 MHz |  | <3.6 MHz | A7 |  |  |
|  | 3650 < FC ≤ 3680 |  | >24.48 MHz |  | A7 |  |  |
|  |  |  | ≤24.48 MHz, ≥6.48 MHz | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  |  | <6.48 MHz | <3.6 MHz | A7 |  |  |
| 40 MHz | 3600 ≤ FC ≤ 3650 | ≤6.12 MHz |  | <1.44 MHz | A8 | >20 MHz | 4.5 |
|  |  |  | ≥ 32.76 |  |  |  |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

Table 6.2.3.16-2: A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | QPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | 16 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 64 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 256 QAM |  | 6 |  |  |  |  | 11 |  |
| CP-OFDM | QPSK | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 16 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 64 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 256 QAM |  | 7 |  |  |  |  | 11.5 |  |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |  |

As can be seen, the needed A-MPR can be up to 11.5dB in many cases. The problem is that for larger bandwidths the IMD3 is directly falling into the frequency range protected by the FCC resulting in the need of very large A-MPR. This is why the UL Channel Bandwidth CBW as well as the UL CA Aggregated Bandwidth ABW for BW class B have been limited to 40MHz.

The UE is responsible to always fulfill the regulatory FCC emissions requirements, therefore even if the network doesn’t signal NS_27, the UE will always apply A-MPR to be able to fulfill the FCC requirements.

Observation 1: There is NS_27 defined for n48 UL due to a stringent FCC emissions requirement. For 40MHz UL carriers the A-MPR is up to 11.5dBm.

## 2.2	Adding 50…100MHz UL to n48

When adding 50…100MHz UL CBW or up to 100MHz UL Aggregated Bandwidth ABW to n48, there will be the need to extend the A-MPR to the new bandwidths. Since the BW will be larger and the IMD3 of the transmitter will fall directly into the FCC protected band, the A-MPR will increase significantly, first indications are up to 15dB. Additionally the number of use cases when this A-MPR is needed will be increasing, since the IMD will almost always fall into the protected frequency range for a 100MHz CBW UL. So effectively in most cases the output power will be reduced from 23 dBm to approximately 8dBm significantly reducing the coverage.

Observation 2: For 100MHz UL carriers in n48 the NS_27 definition needs to be extended to cover the additional CBWs and ABWs resulting in up to 15dBm A-MPR in most of the use cases. This significantly reduces the coverage of the cell.

## 2.3	Proposals for agreement

Since the A-MPR will be quite high in most of the use cases, the coverage of a n48 cell will be significantly degraded. This results in the question how useful it is to implement an UL CBW of 100MHz for n48 in the 3GPP specs.

Proposal 1: Due to the high power reduction needed to fulfill the FCC emissions requirements for a 50…100MHz UL bandwidth on band n48 reducing the coverage of the cell, it is proposed RAN4 does not specify 50…100MHz UL CBW or ABW for n48.

# 3	Conclusion

In this contribution we point out significant issues due to the larger n48 UL bandwidth up to 100MHz.We propose not to specify such large UL bandwidths as the usability seems quite low.

Observation 1: There is NS_27 defined for n48 UL due to a stringent FCC emissions requirement. For 40MHz UL carriers the A-MPR is up to 11.5dBm.

Observation 2: For 100MHz UL carriers in n48 the NS_27 definition needs to be extended to cover the additional CBWs and ABWs resulting in up to 15dBm A-MPR in most of the use cases. This significantly reduces the coverage of the cell.

Proposal 1: Due to the high power reduction needed to fulfill the FCC emissions requirements for a 50…100MHz UL bandwidth on band n48 reducing the coverage of the cell, it is proposed RAN4 does not specify 50…100MHz UL CBW or ABW for n48.

# 4	References

RP-2407225 “New WID: Basket WID on adding channel bandwidth support to existing NR bands”, RAN4 VC, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17-20, 2024