3GPP RAN TSG-RAN4 Meeting # 112		R4-2411150

Maastricht, NL, August 19 – 23, 2024

Agenda item:	7.10.2

Source:	Apple

Title:	On DL interruption for Tx switching across 3 and 4 bands

WI/SI:	DL_intrpt_combos_TxSW_R19-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

During the most recent RAN plenary meeting ( 3GPP TSG RAN Meeting #104, Shanghai, China, 17th – 20th June 2024), a new WID titled “Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink” was approved and added on the list of Release 19 work items [1] .  As captured in the highlighted box below,  the main objective of the WID is to specify the downlink (DL) interruption applicability for inter-band CA combinations when 3 or 4 bands are involved in dynamic UL Tx switching.

## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The objectives of the core part are as follows: - Discuss and specify inter-band uplink CA and EN-DC combinations for which DL interruption is not allowed when dynamic switching between two uplink carriers is conducted, or when dynamic switching with 3 or 4 uplink bands is conducted for inter-band uplink CA. - Based on the requested band combinations from operators, analyze if it is feasible to specify no DL interruption in RAN4 specification. - Based on the feasibility analysis for specific combination, decide to introduce no DL interruption in RAN4 specification. - Specify REL-independent way for CA combinations of this WI. - Introduce the CA with 3 or 4 uplink bands uplink switching in a REL-independent way starting from release-18. - Note: the CA and EN-DC combinations with two bands uplink switching have been introduced in a REL-independent way starting from release 16/17, no changes to 38.307 are needed. The band combinations requested for mandating no DL interruption to conduct dynamic Tx switching are listed in table 1: Table 1: Band combinations requested for mandating no DL interruption Configuration; Uplink band; contact name, company; Contact email; other supporting companies (min. 3). CA_n1-n5-n78; n1, n5, n78; Bo Liu, China Telecom; liubo1@chinatelecom.cn; Huawei, HiSilicon, CATT. CA_n3-n5-n78; n3, n5, n78; Bo Liu, China Telecom; liubo1@chinatelecom.cn; Huawei, HiSilicon, CATT. CA_n1-n3-n5-n78; n1, n3, n5, n78; Bo Liu, China Telecom; liubo1@chinatelecom.cn; Huawei, HiSilicon, CATT.

In this contribution, we will share our views on how the DL interruption can be applied for 3-band or 4-band  inter-band CA combinations subjected to UL TX switching. We will review the previously defined DL interruption applicability rules for 2-band CA combos and explore how to apply those rules to the existing and future 3 and 4 bands CA inter-band CA combinations.

# 2	Discussions

As stated in the specifications (TS 38.133), when dynamic switching between two uplink carriers is conducted, the UE is allowed to cause DL interruption of a number of OFDM symbols in NR downlink carrier(s) as indicated by uplinkTxSwitching-DL-Interruption signaling capability. No DL interruption is allowed in the NR downlink carrier(s) which is not indicated by uplinkTxSwitching-DL-Interruption. The DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL Tx switching period located in uplink NR band(s) as indicated in RRC signaling. The interruption lengths of X for single TAG and two TAGs are defined in Table 8.2.2.2.10D-1 and Table 8.2.2.2.10D-2 as shown in Table 2-1 and Table2-2 respectively.

|  | NR Slot length (ms) | Uplink Tx switching periodNote1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 7 |
| 2 | 0.25 | 4 | 10 | 14 |
| Note 1: Uplink Tx switching period depends on UE capability [TBD]. Note 2: RTD=3us is assumed to derive the DL interruption length. |  |  |  |  |

Table 2-1: DL interruption length for single-TAG

|  | NR Slot length (ms) | Uplink Tx switching periodNote1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | [8] |
| 2 | 0.25 | 4 | 10 | 14 |
| Note 1: Uplink Tx switching period depends on UE capability [TBD]. Note 2: RTD=9us is assumed to derive the DL interruption length. |  |  |  |  |

Table 2-2: DL interruption length for dual-TAG

In the current specifications (TS 38.101-1) for UL, the bands where the DL interruption for UL Tx switching is applicable are indicated for the 2-band and 3-band inter-band CA combinations in Table 5.2A.2.1-1 and Table 5.2A.2.2-1 respectively.

## 2.1	DL interruptions applicability for 2-band NR CA and EN-DC combinations  (Rel-16/Rel-17)

The UL Tx switching feature across two bands was introduced in Rel-16. RAN4 agreed that for cases where the  receivers for the 2 bands are not impacted by the switching, DL interruption would not be allowed. As highlighted in [2], DL interruption for Tx switching between carrier 1 and carrier 2 is not allowed for the following scenarios:

RAN4 reached the following agreements related to DL interruption due to UL switching:

- The following duplex mode combinations (carrier 1 + carrier 2) do not require DL reception interruption:

  - SUL+TDD

  - TDD+TDD CA with the same UL-DL pattern

  - TDD+TDD EN-DC with the same UL-DL pattern

- For other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

  - UE capability is reported per DL band in each band combination for each UL band pair supporting Tx switching.

    - For example, if one UE supports a band combination with DL bands A+B+C+D and UL bands A+B+C, and supports Tx switching between UL band A and B as well as UL band A and C (i.e., not supports Tx switching between UL band B and C), the capability for DL interruption should be reported as follows:

      - For UL band pair A and B, the UE reports whether there is interruption on each of the DL bands, i.e., DL band A, B, C and D.

      - For UL band pair A and C, the UE reports whether there is interruption on each of the DL bands, i.e., DL band A, B, C and D.

  - For UE reporting DL interruption, the RRM interruption requirements defined in RAN4 shall be applied.

From the criterias shown above, we can make the following observation for DL interruption with UL Tx switching across two bands (Rel-16/Rel-17):

Observation #1:

- For TDD+TDD band combinations with the same UL-DL duplex mode, DL interreuption is not required

- For FDD+TDD combos, DL interruption applicabilty is per DL band. For example, interruption is not required when there is a wide frequecny separation between the DL band and the UL bands involved in the switching.

## 2.3	DL interruptions applicability for 3-band and 4-band NR and EN-DC combinations

In RAN4 Meeting #105 ( Toulouse, France, November 14th – 18th , 2022 ), an agreement was made on DL interruption applicability for UL Tx switching across 3 or 4 bands with the WF highlighted below [4] :

### Sub-topic 1-4: Applicability of DL interruption #### Issue 1-4-1: DL interruption for SUL+TDD+FDD combinations Way forward: - For SUL+TDD+FDD combinations, when Tx switching is performed between SUL and TDD bands, whether or not to mandate no DL interruption on the SUL and TDD bands can be discussed in a case by case (i.e., per band combination basis) manner. #### Issue 1-4-2: Issues of different band pairs having different capabilities of DL interruption Way forward: - Further discuss in the next meeting that: - RAN4 clarify applicability of DL interruption when different band pairs have different capability of DL interruption and Tx switching is performed for both band pairs simultaneously. #### Issue 1-4-3: On previous agreements for combinations of SUL+TDD and TDD+TDD CA band combinations Way forward: - For synchronized CA between SUL configuration (with a SUL band and a TDD band) and TDD band(s) with the same UL-DL pattern across all TDD bands: - Option 1: DL interruption is not required - Option 2: whether to mandate no DL interruption can be discussed in a case by case (i.e., per band combination basis) manner. - For synchronized CA among 3 or 4 TDD band(s) [‘with the same UL-DL pattern’ or ‘without mandatory simultaneous Rx/Tx requirement’] across all TDD bands, DL interruption is not required.

Based on the WF highlighted above, RAN4 agreed to extend the DL interruption applicability of Tx switching between 2 bands to DL applicability for Tx switching between 3 and 4 bands. It’s important to observe that, for TX switching scenarios across 3 or 4 bands, whether DL interruption is applied to the unaffected band(s) is case by case dependent. Applying DL interruption on the unaffected band(s) for these switching scenarios will depend on the frequency separations, the UL-DL duplex modes, and whether or not there is a mandatory simultaneous RX/Tx between one of the bands involved in the switching and the unaffected band.

Observation #2: For Tx switching scenario across 3 or 4 bands, applying DL interruption on the unaffected band(s) depends on the frequency separations, the UL-DL frame patterns, and whether or not there is a mandatory simultaneous RX/Tx between one of the bands involved in the switching and the unaffected band.

Proposal #1:

Based on the already defined RAN4 rules, DL interruption for TX switching among 3 or 4 bands is DL band-dependent and the following criteria should be used to decide on its applicability:

- Whether the same UL-DL duplex mode is used for the DL band(s) and the UL band(s)

- Frequency separation between the DL band and the bands involved in the Tx switching

- Mandatory simultaneous RX/TX capability between the DL band(s) and the UL band(s)

Following the rules highlighted above DL applicability for the proposed combos are shown in Table 2.3-1.

| CA combination | Configured Uplink Combos | DL interruption allowed? |
| --- | --- | --- |
| CA_n1-n5-n78 | CA_n1-n5 |  |
|  | CA_n1-n78 | No |
|  | CA_n5-n78 | No |
| CA_n3-n5-n78 | CA_n3-n5 |  |
|  | CA_n3-n78 | No |
|  | CA_n5-n78 | No |
| CA_n1-n3-n5-n78 | CA_n1-n3 |  |
|  | CA_n1-n5 |  |
|  | CA_n1-n78 | No |
|  | CA_n3-n5 |  |
|  | CA_n3-n78 | No |
|  | CA_n5-n78 | No |

Table 2.3-1: DL interruption applicability for the proposed CA combos

# 3	Conclusions

Observation #1:

- For TDD+TDD band combinations with the same UL-DL duplex mode, DL interreuption is not required

- For FDD+TDD combos, DL interruption applicabilty is per DL band. For example, interruption is not required when there is a wide frequecny separation between the DL band and the UL bands involved in the switching.

Observation #2: For Tx switching scenario across 3 or 4 bands, applying DL interruption on the unaffected band(s) depends on the frequency separations, the UL-DL frame patterns, and whether or not there is a mandatory simultaneous RX/Tx between one of the bands involved in the switching and the unaffected band.

Proposal #1:

Based on the already defined RAN4 rules, DL interruption for TX switching among 3 or 4 bands is DL band-dependent and the following criteria should be used to decide on its applicability:

- Whether the same UL-DL duplex mode is used for the DL band(s) and the UL band(s)

- Frequency separation between the DL band and the bands involved in the Tx switching

- Mandatory simultaneous RX/TX capability between the DL band(s) and the UL band(s)

| CA combination | Configured Uplink Combos | DL interruption allowed? |
| --- | --- | --- |
| CA_n1-n5-n78 | CA_n1-n5 |  |
|  | CA_n1-n78 | No |
|  | CA_n5-n78 | No |
| CA_n3-n5-n78 | CA_n3-n5 |  |
|  | CA_n3-n78 | No |
|  | CA_n5-n78 | No |
| CA_n1-n3-n5-n78 | CA_n1-n3 |  |
|  | CA_n1-n5 |  |
|  | CA_n1-n78 | No |
|  | CA_n3-n5 |  |
|  | CA_n3-n78 | No |
|  | CA_n5-n78 | No |

# 4	References

RP-24xxxx, “ New WID: Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink”, by China Telecom, 3GPP TSG-RAN Meeting #104,  Shanghai, China, June 17-20, 2024

R4-2004895, “LS on DL interruption due to Tx switching”, by China Telecom, 3GPP TSG-RAN WG4 Meeting #94bis-e, Electronic Meeting, April 20-30, 2020

R4-2220546, “WF on UE Tx switching requirement for single TAG”, by China Telecom, 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, 14th November 2024 - 18th November 2022

R4-2220546, “WF on UE Tx switching requirement for single TAG”, by China Telecom, 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, 14th November 2024 - 18th November 2022