3GPP RAN TSG-RAN4 Meeting # 112		R4-2411151

Maastricht, NL, August 19 – 23, 2024

Agenda item:	8.1.1.3.3

Source:	Apple

Title:	On the values of ΔTRxSRS for 6Rx SRS antenna switching

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting (Meeting #111, Fukuoka, Japan, May 20-24, 2024), discussions were held on the requirements for 6Rx for handheld and FWA, which are parts of the new  Release 19 WID on RF enhancements for NR FR1/FR2 and EN-DC [1]. As captured in the meeting WF [2], one of the 6Rx aspects that  were greatly debated is the SRS antenna switching and the values of ΔTRxSRS, with the discussion summary highlighted below:

# Topic 2: SRS antenna switching and ΔTRxSRS # Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: SRS antenna switching configurations Way forward: RAN4 to focus on discussing ∆TRxSRS for 1tr6 and t2r6 until RAN1 concludes work on 3t6r and 4t6r. Pending RAN1 feedback, RAN4 can consider any necessary updates to the work scope in a Revised WID for the SRS antenna configurations subject to RAN Plenary approval. Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency Way forward: RAN4 defines different ∆TRxSRS values based on operating frequency. Further discuss whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. ## Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way forward: Further discuss the following options for the ∆TRxSRS requirements. - Option 1: Consider average value of ∆TRxSRS for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; 4.0 dB; 3.4 dB; 4.4 dB. Band n79, [n104]; 5.3 dB; 4.5 dB; 5.7 dB. - Option 2: Consider range of ∆TRxSRS in brackets for each case below for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; [2.8 ~ 5.0] dB; [2.3 ~ 4.0] dB; [3.3 ~ 5.5] dB. Band n79, [n104]; [3.6 ~ 6.0] dB; [3.0 ~ 5.5] dB; [4.2 ~ 6.5] dB. Note: Maximum level for t1r6 corrected for OPPO proposals per OPPO. - Option 3: Other options are not precluded.

In this contribution, we will share our views on the values of  ΔTRxSRS for the three SRS antenna switching configurations t1r6, t2r6, and t1r6-t2r6 and for the NR bands n41, n77, n78, n79, and n104 as highlighted in the WF summary above.

# Discussion

For SRS antenna switching, RAN4 defines ΔTRxSRS as the maximum allowed relaxation of the maximum configured transmitted power PCMAX,f,c due to the additional insertion loss differences between the main TX path and a given diversity branch. As was highlighted in [3], the main UE integration factors that contribute to these insertion loss differences are the losses on antenna switches, PCB traces, and the losses of the RF antenna cable located between the RF front-end ( with the integrated PA) on the main board and the antenna connector of the ports being sounded located on a separate RF board. The insertion losses of each component within any SRS branch vary depending on the suppliers. Also, these losses are frequency-dependent. As was previously used for 8Rx, we can split the bands into two frequency ranges: Bands whose FUL_high is lower than the FUL_low of n79 (n41, n77, n78) , and bands whose FUL_high is higher than the FUL_low of n79 (n79, n104).

- Antenna switches: Depending on the antenna architecture, different RF switch topologies ( SPST, SPDT, SP3T, SP4T, DP3T, SP6T etc.) could be used on different RX diversity branches. As depicted in Figure 2.1-1 below for example, the architecture shown on the left uses an SP6T switch while the one on the right uses an SP4T and an SP3T. Typically, a switch with a higher number of throws exhibits higher insertion loss compared to the one with lower number of throws, when the same process technology is used. Table 2-1 shows the assumed worst case insertion losses on several antenna switch topologies, based on a typical high-frequency SOI process.

- PCB trace losses: PCB trace is used on the main board for the connections between the Tx signals and to antennas.  These losses will be different between the main antenna path and the Rx diversity paths, due to their relative positions within the UE. We assume a trace length of 60mm to derive the worst-case losses on the PCB traces.

- RF cable losses: An RF coaxial cable is used from main board to antennas on the sub-board. We will assume that the antennas will be fed by semi rigid coax cables with length up to 200mm worst case and with an average cable jacket diameter of 1.32mm as was assumed in [4].

Table 2-1 shown below summarizes the insertion loss assumptions that we will use to derive the values of ΔTRxSRS.

| Parameter | 3.5GHz | 4.9GHz | [7.1GHz] |
| --- | --- | --- | --- |
| SP2T IL | 0.6dB | 0.9dB | 1.3dB |
| SP3T IL | 0.7dB | 1.0dB | 1.4dB |
| SP4T IL | 0.8dB | 1.1dB | 1.5dB |
| SP6T IL | 0.9dB | 1.2dB | 1.7dB |
| DP3T IL | 0.9dB | 1.2dB | 1.5dB |
| 60-mm PCB trace IL (SRS path) | 1.8dB | 2.4dB | 3.0dB |
| RF cable loss | 0.8dB | 1.1dB | 1.4dB |

Table 2-1: IL loss assumptions for the main contributors on ΔTRxSRS

  - 1T6R SRS antenna switching configuration

Below are shown two possible antenna configurations for the 1T6R SRS antenna switching. We will consider the configuration on the right (with SP4T + SP3T) to derive the insertion loss link budget

Using the assumptions shown in Table 2-1, the ΔTRxSRS can be computed as follows for the 5 antenna ports 2, 3, 4, 5, 6:

- For bands n41, n77, and n78: ΔTRxSRS = SP3T_IL + SP2T_IL + PCB_IL  + IL_Cable = 0.7 + 0.6 + 1.8 + 0.8 = 3.9dB

- For bands n79 and n104: ΔTRxSRS = SP3T_IL + SP2T_IL + PCB_IL + IL_Cable = 1.0 + 0.9 + 2.4 + 1.1 = 5.4dB

Proposal 1: For 1T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS = 3.9dB

- For n79 and n104, ΔTRxSRS = 5.4dB

Figure 2.1-1: 1T6R antenna configurations using an SP6T (left figure) and SP4T+SP3T (right figure)

  - 2T6R SRS antenna switching configuration

The diagram shown in Figure 2.2-1 can be used for the 2T6R RSR antenna switching configuration.

Figure 2.2-1: 2T6R antenna configuration for SRS antenna switching

Using the assumptions shown in Table 2-1, the ΔTRxSRS can be computed as follows for the 5 antenna ports 2, 3, 4, 5, 6:

- For bands n41, n77, and n78: ΔTRxSRS = SP2T_IL + PCB_IL + IL_Cable = 0.6 + 1.8 + 0.8 = 3.2dB

- For bands n79 and n104: ΔTRxSRS = SP2T_IL + PCB_IL + IL_Cable = 0.9 + 2.4 + 1.1= 4.4dB

Proposal 2: For 2T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS =3.2dB

- For n79 and n104, ΔTRxSRS =4.4dB

  - 1T6R-2T6R SRS antenna switching configuration

The diagram shown in Figure 2.3-1 can be used for the 1T6R-2T6R RSR antenna switching configuration.

Figure 2.3-1: 1T6R-2T6R antenna configuration for SRS antenna switching

Using the assumptions shown in Table 2-1, the ΔTRxSRS can be computed as follows for the 5 antenna ports 2, 3, 4, 5, 6:

- For bands n41, n77, and n78: ΔTRxSRS = DP3T_IL + SP2T_IL + PCB_IL + IL_Cable = 0.7 + 0.9 + 1.8 + 0.8 = 4.2dB

- For bands n79 and n104: ΔTRxSRS = DP3T_IL + SP2T_IL + PCB_IL  + IL_Cable = 1.0 + 1.2 + 2.4 + 1.1 = 5.7dB

Proposal 3: For 1T6R-2T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS =4.2dB

- For n79 and n104, ΔTRxSRS =5.7dB

# 4	Conclusion

Proposal 1: For 1T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS = 3.9dB

- For n79 and n104, ΔTRxSRS = 5.4dB

Proposal 2: For 2T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS =3.2dB

- For n79 and n104, ΔTRxSRS =4.4dB

Proposal 3: For 1T6R-2T6R SRS antenna switching configuration, the following values can be used for ΔTRxSRS:

- For n41, n77, and n78, ΔTRxSRS =4.2dB

- For n79 and n104, ΔTRxSRS =5.7dB

# 4	References

R4-2403716 “New WID: UE RF enhancements for FR1/FR2 and EN-DC , Phase 4”, by RAN4 chair (Huawei),  3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

R4-2410751, “WF on requirements for 6Rx”, by AT&T, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Japan, May 20-24, 2024

R4-2407178, “Views on 6Rx for handheld UEs”, by Apple, 3GPP TSG-RAN WG4 Meeting #110Bis, Changsha, China, 15th Apr 2024 - 19th Apr 2024

R4-2400157, “On 2Rx for XR”, by Apple, 3GPP TSG-RAN WG1 Meeting #110, Athens, Greece, February 26 – March 1, 2024