3GPP TSG-RAN WG4 Meeting #112															R4-2411153

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.1.1.2.1

Source:	Apple

Title:	On Rel-19 power domain enhancement

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

Power domain enhancement is a re-occurring topic and enters the 4th phase when starting the Rel-19 work. The objectives are covered in the WID [1]. This contribution focuses on the high-level aspects of MPR reduction. The WID lists those goals as:

# 2  Discussion

Over the past releases RAN4 discussed various features allowing to increase the output power and consequentially improve UL coverage. In Rel-19 a new angle is approached by evaluating various power domain improvements. One direction is to convert outer RB allocations into inner RB allocations which allows to considerably improve output power due to lower MPR. The WF [2] captures the following objective:

The objective is to study the approach of converting full RB allocations from a UE channel to inner RB allocations by extending the UE channel equally on each side. This can be achieved by using a larger BS channel and relating the emission requirements not on UE but on the BS channel. To determine the maximum UE channel, it is necessary to check the number of RBs which can fit inside the inner allocation region of the larger BS channel. To identify the maximum UE channel, it is required to compare the number of inner/outer RBs of the various channel sizes. Table 1 provides an overview of the maximum RB inner allocation size with respect to the channel. For example, a 20MHz channel at 15kHz SCS can have a maximum LCRB allocation of 53 RBs which are still part of the inner allocation region. Comparing these RB sizes with smaller channels allows to determine the UE maximum channel which can fit inside a BS channel under the condition that all RBs are part of the inner allocation region from BS perspective. This relation is provided in Table 2.

Table 1: Maximum RB size of inner allocation with respect to CBW

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max Inner RB | Max Inner RB | Max Inner RB | Max Inner RB | Max Inner RB | Max Inner RB | Max Inner RB | Max Inner RB |
| 15 | 7 | 13 | 26 | 39 | 53 | 67 | 80 | 94 |
| 30 | N/A | 5 | 12 | 19 | 25 | 33 | 39 | 46 |
| 60 | N/A | N/A | 5 | 9 | 12 | 15 | 19 | 22 |

| SCS (kHz) | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max Inner RB | Max Inner RB | Max Inner RB | Max Inner RB | Max Inner RB | Max Inner RB | Max Inner RB | Max Inner RB |
| 15 | 108 | 121 | 135 | N/A | N/A | N/A | N/A | N/A |
| 30 | 53 | 59 | 67 | 81 | 95 | 109 | 123 | 137 |
| 60 | 25 | 29 | 33 | 39 | 47 | 53 | 61 | 67 |

Table 2: Maximum UE CBW where all RBs could be inner RB allocations from BS channel point of view

| BS CBW | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS (kHz) | Max UE CBW | Max UE CBW | Max UE CBW | Max UE CBW | Max UE CBW | Max UE CBW | Max UE CBW | Max UE CBW |
| 15 | N/A | N/A | 5 | 5 | 10 | 10 | 15 | 15 |
| 30 | N/A | N/A | 5 | 5 | 10 | 10 | 15 | 15 |
| 60 | N/A | N/A | N/A | N/A | 10 | 10 | 15 | 15 |

| BS CBW | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS (kHz) | Max UE CBW | Max UE CBW | Max UE CBW | Max UE CBW | Max UE CBW | Max UE CBW | Max UE CBW | Max UE CBW |
| 15 | 20 | 20 | 25 | N/A | N/A | N/A | N/A | N/A |
| 30 | 20 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| 60 | 20 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |

Observation 1: If the goal is to convert all outer RB allocations into inner RB allocations, then the minimum BS channel needs to be at least twice the size of the UE channel.

As an alternative to deploying a UE with half of BS channel size the UE channel could be slightly larger. In this scenario some of the RB allocations would be part of outer allocations. Those allocations are limited by SEM and ACLR. However, it is expected that the resulting power back-off is lower than the specified outer MPR. One option is to set the UE channel to roughly two-thirds of the BS channel. Table 3 captures a scenario where the UE channel is 15MHz while the BS channel is 20MHz. All inner allocations of the UE channel have the same MPR as always. Outer allocations could benefit by 0.5 to 1.0dB increased output power. According to the initial simulations this could improve MPR for modulations of 16QAM and lower. Higher order modulations are limited by EVM/IBE and do not seem to benefit from this approach.

Table 3: UE channel inside larger BS channel (PC3)

| Modulation | DFT-s-OFDM |  | CP-OFDM |  |
| --- | --- | --- | --- | --- |
|  | Power Gain of Outer RB | Gating Factor | Power Gain of Outer RB | Gating Factor |
| PI/2 BPSK | 0.5 | PC |  |  |
| QPSK | 0.5-1.0 | PC/ACLR | 0.5-1.0 | ACLR |
| 16QAM | 0.5-1.0 | ACLR | 0.5-1.0 | ACLR |
| 64QAM | - | EVM/IBE | - | EVM/IBE |
| 256QAM | - | EVM/IBE | - | EVM/IBE |
| Details | Note 1: BS CBW is 20MHz. The UE CBW is centred and 15MHz Note 2: Tx impairments as agreed in [3] Note 3: Inner RB allocation MPR is not listed as it is the same as regular inner MPR Note 4: MPR of 64QAM and 256QAM is EVM/IBE limited and no improvements possible |  |  |  |

Observation 2: If the sole goal is to obtain output power improvements, then it is possible to have a more convenient ratio between UE and BS channel such as two-third. Initial analysis indicates that for 15MHz UE channel and 20MHz BS channel a power output improvement of 0.5 to 1.0dB seems to be possible.

# 4	Conclusions

This contribution provides certain considerations to the discussion on the MPR reduction which his part of the power domain enhancement WID. The following observations are made:

Observation 1: If the goal is to convert all outer RB allocations into inner RB allocations, then the minimum BS channel needs to be at least twice the size of the UE channel.

Observation 2: If the sole goal is to obtain output power improvements, then it is possible to have a more convenient ratio between UE and BS channel such as two-third. Initial analysis indicates that for 15MHz UE channel and 20MHz BS channel a power output improvement of 0.5 to 1.0dB seems to be possible.

# 4	References

- RP-240828, “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, 3GPP TSG RAN Meeting #103

- R4-2410749, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #111