3GPP TSG-RAN WG4 Meeting #112															R4-2411154

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	7.20.2

Source:	Apple

Title:	On PC2 for n28

WI/SI:	NR_n28_PC2_40MHz_CBW-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The objectives of the Rel-19 WI [2] are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28. It is encouraged to evolve the existing dual duplexer architecture assumption to meet the current industry demand. This contribution proposes PC2 requirements for NS_17 considering a full-band duplexer.

# 2  Discussion

##### 2.1 On PC2 NS_17

The situation for NS_17 for a 10MHz channel with lower edge at 718MHz is depicted below. The in-band emission limit is applicable up to frequency 710MHz since no exception for FOOB applies.

The simulation results for emission compliance and A-MPR were provided in [2]. During the Rel-18 discussion an intermediate agreement was made in PC2 A-MPR and captured in WF [3]. It is proposed to define the A-MPR regions and values as provided in Table 1 and Table 2.

Table 1: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | > 5.4 | A2 |

Table 2: A-MPR for NS_17 for PC2

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 3 | ≤ 4 |
|  | QPSK | ≤ 3 | ≤ 4 |
|  | 16 QAM | ≤ 3.5 | ≤ 4 |
|  | 64 QAM | ≤ 4 | ≤ 4.5 |
|  | 256 QAM |  | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 5 | ≤ 5.5 |
|  | 16 QAM | ≤ 5 | ≤ 5.5 |
|  | 64 QAM | ≤ 5 | ≤ 5.5 |
|  | 256 QAM |  |  |

Proposal 1: Specify NS_17 A-MPR for PC2 and define the additional power back-off according to Table 1 and Table 2.

# 4	Conclusions

This contribution provides considerations on n28 full-band duplexer architecture. The following proposal is made:

Proposal 1: Specify NS_17 A-MPR for PC2 and define the additional power back-off according to Table 1 and Table 2.

# 4	References

- RP-241205, “Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28“, CMCC, CBN, 3GPP TSG-RAN Meeting #104

- R4-2400170, “On n28 with full band duplexer”, Apple, 3GPP RAN WG4 Meeting #110

- R4-2321715, “WF on HPUE for FDD bands”, China Unicom, 3GPP TSG-RAN WG4 Meeting # 109