3GPP TSG-RAN WG4 Meeting #112															R4-2411155

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	7.20.3

Source:	Apple

Title:	On 40MHz CBW for n28

WI/SI:	NR_n28_PC2_40MHz_CBW-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The objectives of the Rel-19 WI [2] are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28. It is encouraged to evolve the existing dual duplexer architecture assumption to meet the current industry demand. This contribution discusses the introduction of 40MHz channel bandwidth.

# 2  Discussion

##### 2.1 40MHz CBW and ∆MPR

The WID objective for 40MHz is described as follows:

Special relaxation for the output power is defined for FDD bands where the relative bandwidth is larger than 3%. The requirement is defined in clause 6.2.2. In case of band n28 a delta value of 0.5dB is allowed for 30MHz channel as the relative bandwidth is 4.1%. Introducing 40MHz channel the relative bandwidth grows to 5.5%. It is encouraged to discuss the ∆MPR value for 30MHz and define corresponding relaxation.

Proposal 1: As ∆MPR is already defined for 30MHz channel bandwidth it is encouraged to further discuss delta value for 40MHz channel.

##### 2.2 On MSD

The RB setup for MSD is defined in Table 7.3.2-3 of TS 38.101-1 and copied below for n28. From CBW of 10MHz to 30MHz 25 RBs are defined for SCS=15kHz and 10 RBs for SCS=30kHz. The RBs are placed at the upper most possible location. When introducing 40MHz CBW it is proposed to use the same setup as for 30MHz channels.

Proposal 2: When introducing 40MHz CBW it is proposed to use 25 RBs for SCS=15kHz and 10 RBs for SCS=30kHz placed as close as possible to the downlink operating band.

When introducing a new channel bandwidth, the definition of MSD is a critical step to ensure UE receiver performance and for operators to plan their network deployment. Among several parameters that define MSD the duplex filter isolation plays a significant role. It has an impact on the final value and should therefore be discussed for n28 full-band duplexer due to the small frequency separation between UL and DL. In fact, the small separation was one of the reasons for introducing the split duplexer approach. As shown by many vendors a split duplexer can achieve the desired 50dB isolation. For full-band duplexer the isolation should be verified before proceeding with MSD calculations. RAN4 should discuss the isolation of a full-band duplexer when defining MSD for 40MHz bandwidth so that all companies have an aligned understanding on the performance.

Proposal 3: The small duplex distance makes it challenging to duplexer covering the whole band and RAN4 should discuss the duplexer isolation of full-band duplexer before defining MSD.

# 4	Conclusions

This contribution provides considerations on n28 full-band duplexer architecture. The following proposals are made:

Proposal 1: As ∆MPR is already defined for 30MHz channel bandwidth it is encouraged to further discuss delta value for 40MHz channel.

Proposal 2: When introducing 40MHz CBW it is proposed to use 25 RBs for SCS=15kHz and 10 RBs for SCS=30kHz placed as close as possible to the downlink operating band.

Proposal 3: The small duplex distance makes it challenging to duplexer covering the whole band and RAN4 should discuss the duplexer isolation of full-band duplexer before defining MSD.

# 4	References

- RP-241205, “Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28“, CMCC, CBN, 3GPP TSG-RAN Meeting #104

- R4-2400170, “On n28 with full band duplexer”, Apple, 3GPP RAN WG4 Meeting #110

- R4-2321715, “WF on HPUE for FDD bands”, China Unicom, 3GPP TSG-RAN WG4 Meeting # 109