3GPP TSG-RAN WG4 Meeting #111														R4-2411156

Maastricht, Netherlands, 19th – 23th August 2024

Agenda item:	5.21.1

Source:	Apple

Title:	(NR_cov_enh2-Core) On remaining issues with Rel-18 coverage enhancement

WI/SI:	NR_cov_enh2-Core

Release:	Rel-18

Document for:	Discussion

# 1	Introduction

The WF [1] captures the last open issues for Rel-18 coverage enhancement. In RAN4#111 agreements were made on the applicability on CA/DC band combinations. The remaining open issues are discussed, and proposals are made on how to capture that no test requirements are applicable for power boosting.

# 2	Discussion

During RAN4#111 it was agreed that no new MSD test cases shall be introduced for Rel-18 power boosting.

The assumption is that the receiver performance is already tested under normal conditions when boosting is not enabled. Consequently, RAN4 decided that there is no need for additional RAN5 test procedures with boosting. To capture the agreement in TS 38.101-1/3 it is proposed to modify the clause 7.1 with an additional statement. The addition is proposed below and clarifies that no receiver requirements apply if ΔPPowerBoost assumes a positive value. The CRs [2] and [3] capture the proposed implementation for TS 38.101-1 and TS 38.101-3.

## 7.1 General Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity). […] Unless otherwise stated, the receiver requirements of single carrier or CA operation are applicable to UE with one Tx antenna connector or multiple Tx antenna connectors with UL MIMO or Tx diversity operation in the UL band(s). Unless otherwise stated, the receiver requirements of single carrier or CA operation are not applicable if ΔPPowerBoost assumes a positive value.

Proposal 1: Capture the RAN4 agreement on receiver requirements and testing by agreeing the CRs for TS 38.101-1 and 38.101-3.

RAN2 does not currently have a dedicated signalling to inform the network of different capabilities with parent and fallback combinations. The handling of different capabilities can be achieved by reporting fallback band combinations along with parent combinations. Those reports for parents and fallbacks can advertise different capabilities. For RAN2 being able to correctly implement the RAN4 understanding it is recommended that RAN4 specifies ‘per-FS’ and explicitly states that fallbacks can have different capabilities. Then RAN2 would be able to decide on the best signalling option.

Observation 1: RAN2 does not currently have a dedicated signalling to inform the network of different capabilities with parent and fallback combinations. However, the handling different capabilities can be achieved by reporting fallback band combinations along with parent combinations.

Proposal 2: It is proposed to set the feature as ‘per-FS’ and explicitly state in the comment part of the feature list that RAN4 intends a UE to be able to support different capabilities for parent and fallback combinations.

# 3	Conclusions

This contribution discussed two of the remaining items for the coverage enhancement feature and makes the following proposals and observations:

Proposal 1: Capture the RAN4 agreement on receiver requirements and testing by agreeing the CRs for TS 38.101-1 and 38.101-3.

Observation 1: RAN2 does not currently have a dedicated signalling to inform the network of different capabilities with parent and fallback combinations. However, the handling different capabilities can be achieved by reporting fallback band combinations along with parent combinations.

Proposal 2: It is proposed to set the feature as ‘per-FS’ and explicitly state in the comment part of the feature list that RAN4 intends a UE to be able to support different capabilities for parent and fallback combinations.

# 4	References

- R4-2410611, “WF on NR_cov_enh2_R18“, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #111

- R4-2411266, “(NR_cov_enh2-Core) CR to 38.101-1: Clarification on receiver requirements for coverage enhancement”, Apple, 3GPP TSG-RAN WG4 Meeting #111

- R4-2411267, “(NR_cov_enh2-Core) CR to 38.101-3: Clarification on receiver requirements for coverage enhancement”, Apple, 3GPP TSG-RAN WG4 Meeting #111