3GPP TSG-RAN WG4 Meeting #112															R4-2411157

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	7.11.2

Source:	Apple

Title:	On simultaneous Rx/Tx

WI/SI:	LTE_NR_R19_Simult_RxTx

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

A new work item [1] on simultaneous Rx/Tx has been agreed in RAN#104. One of its objectives is to clarify and improve existing mandatory simultaneous Rx/Tx requirements. This discussion paper discusses two main topics. The first topic is about the Rel-17 agreement on FR1+FR1 FDD-TDD combinations and the missing requirement in TS 38.101-1/3. The second topic is about CA_n40A-n41A and the requirements when supporting simultaneous Rx/Tx.

# 2  Discussion

#### 2.1 Applicability of simultaneous Rx/Tx

In Rel-17 RAN4 discussed the applicability of mandatory simultaneous Rx/Tx for FDD-TDD and TDD-TDD combinations. The discussion differentiated between FR1+FR1 and FR1+FR2 combinations. It was determined that simultaneous Rx/Tx shall be the default capability for FR1+FR1 FDD-TDD. This agreement is captured in TR [2] in clause 5.2.1 and provided below:

The RAN4 agreement impacts network behaviour as it allows simultaneous Rx/Tx without further UE capability indication. Therefore, UEs need to meet 3GPP spec compliance for all FR1+FR1 FDD-TDD band combination under simultaneous Rx/Tx. While the TR [2] captures the RAN4 agreement there is no equivalent statement in TS 38.101-1 and TS 38.101-3. The issue is that a TR does not provide any specification value. The objective of a TR is to capture the background discussion in a detailed manner and to provide in-depth explanation for certain decisions. It seems an error that a default capability which requires careful engineering is not captured at least in one of the TS. To obtain binding technical requirements the agreement on simultaneous Rx/Tx needs to be added to TS 38.101-1 and TS 38.101-3. The draft CRs [3] and [4] provide a showcase on how the implementation could look like.

Proposal 1: Capture the Rel-17 agreement on mandatory simultaneous Rx/Tx for FR1+FR1 FDD-TDD in TS 38.101-1 and TS 38.101-3.

#### 2.3 Relaxation for CA_n40A-n41A with simultaneous Rx/Tx

The objective for CA_n40A-n41A is captured in [1] as shown below:

According to TS38.101-1 clause 7.2 a UE is required to feature a minimum of four Rx antenna ports for band n41. This requirement applies if n41 is configured as standalone band or if n41 is part of a band combination. Due to near proximity of n40 and n41, those bands are typically grouped together using the same antennas to reduce implementation complexity and number of required antennas.

In case of non-simultaneous Rx-Tx, it is possible to share antennas between n40 and n41 as uplink and downlink transmission never occurs at the same time. However, in case of simultaneous Rx-Tx between n40 and n41 it is challenging to share antennas for uplink and downlink as filter isolation is marginal due to the close frequency separation of the two bands. Additionally, it is challenging to optimize and achieve the required performance for all the required antennas over the entire frequency range of n40 and n41. If an implementation is done for simultaneous Rx/Tx then separate antennas are needed to cover the frequency range. Integrating additional antennas for this frequency range might not be an issue for FWA/CPE type devices but it is a challenging task for handhelds due to the small form factor.

Dependent on the RF front-end implementation a UE might not be able to support simultaneous Rx/Tx for CA_n40A-n41A together with MIMO operation. In this case the UE might either only indicate support of MIMO or only indicate support of simultaneous Rx/Tx for CA_n40A-n41A but not both capabilities. For such devices it might be beneficial to allow relaxation from 4Rx on band n41. This could be implemented by introducing a new note to Table 7.3.2-1b. The note specifies the relaxation for handheld devices only and only if it does not support 6Rx. The proposal can be found below. As alternative a new signalling could be introduced to indicate that a UE supporting MIMO and simultaneous Rx/Tx cannot support both features at the same time for a specific combination. To reduce signalling overhead the indication would be optional and only needed if a UE does not support MIMO and simultaneous Rx/Tx at the same time.

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands Operating band / SCS / Channel bandwidth / REFSENS. Operating band; SCS kHz; Channel bandwidth (MHz); REFSENS (dBm)8; Duplex Mode. …. n40; 15; 5, 10, 15, 20, 25, 30, 40, 50; -100 + 10log10(NRB/25); TDD. 30; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.5 + 10log10(NRB/11). n411,11, n901,11; 15; 5, 10, 15, 20, 25, 30, 35, 40, 45, 50; -94.8 + 10log10(NRB/52); TDD. 30; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.5 + 10log10(NRB/11). …. NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. Note 11: Requirements for four Rx antenna ports and 4 MIMO layers do not apply for this band when carrier aggregation with band n40 and simultaneous Rx-Tx operation is configured. This relaxation does not apply to FWA form factor devices and UEs indicating IE [TBD] for 6Rx capability..

Observation: Due to close frequency distance of n40 and n41 these bands are typically grouped together to reduce implementation complexity and number of required antennas. In case of simultaneous Rx-Tx between n40 and n41 it is challenging to share antennas for uplink and downlink as filter isolation is marginal due to the close frequency separation of the two bands. The challenge increases if support for MIMO and its related features e.g. antenna switching are considered in addition. If an implementation is done for simultaneous Rx/Tx and MIMIO then additional antennas might be needed to cover the frequency range. Integrating additional antennas for this frequency range might not be an issue for FWA/CPE type devices but it is a challenging task for handhelds due to the small form factor.

Proposal 2: Consider relaxing four Rx requirement for n41 when carrier aggregation with band n40 and simultaneous Rx-Tx operation is configured. As alternative a new signalling could be introduced to indicate that a UE supporting MIMO and simultaneous Rx/Tx cannot support both features at the same time for a specific combination.

# 4	Conclusions

This contribution provides certain considerations on simultaneous Rx/Tx. The following observations and proposals are made:

Proposal 1: Capture the Rel-17 agreement on mandatory simultaneous Rx/Tx for FR1+FR1 FDD-TDD in TS 38.101-1 and TS 38.101-3.

Observation: Due to close frequency distance of n40 and n41 these bands are typically grouped together to reduce implementation complexity and number of required antennas. In case of simultaneous Rx-Tx between n40 and n41 it is challenging to share antennas for uplink and downlink as filter isolation is marginal due to the close frequency separation of the two bands. The challenge increases if support for MIMO and its related features e.g. antenna switching are considered in addition. If an implementation is done for simultaneous Rx/Tx and MIMIO then additional antennas might be needed to cover the frequency range. Integrating additional antennas for this frequency range might not be an issue for FWA/CPE type devices but it is a challenging task for handhelds due to the small form factor.

Proposal 2: Consider relaxing four Rx requirement for n41 when carrier aggregation with band n40 and simultaneous Rx-Tx operation is configured. As alternative a new signalling could be introduced to indicate that a UE supporting MIMO and simultaneous Rx/Tx cannot support both features at the same time for a specific combination.

# 4	References

- RP-241682, “Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19”, Huawei, 3GPP TSG RAN Meeting #104

- TR 38.839, “Principles and requirements for simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC”, LTE_NR_Simult_RxTx-Core, Rel-17

- R4-2411158, “Draft CR to 38.101-1 to clarify simultaneous Rx/Tx applicability for band combinations”, Apple, 3GPP TSG-RAN WG4 Meeting #112

- R4-2411159, “Draft CR to 38.101-3 to clarify simultaneous Rx/Tx applicability for band combinations” Apple, 3GPP TSG-RAN WG4 Meeting #112