3GPP RAN WG4 Meeting #112		R4-2411166

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	7.2.2

Source:	Apple

Title:	On DC_3C_n7B with UL DC_3C_n7B

WI/SI:	DC_R19_xBLTE_yBNR-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The suite of the Rel-19 basket work items has been approved in RAN #104 meeting. In the new basket WID “Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (x<6, y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands” [1], it seems like contiguous UL CA in both UL bands with a total of 4 UL CCs would be allowed which in our view should be considered as a new RF feature and subject to further technical discussions before the feature can be included in the basket WID, as prior to Rel-19, only up to 3 UL CCs is allowed. The 4 UL CCs feature support in the basket WID is also evident by the proposed EN-DC band combination DC_3C_n7B with UL DC_3C_n7B. In this contribution, we share our views on how the band combination with 4 UL CCs may be handled in Rel-19.

# Discussion

In the newly approved Rel-19 basket WID “Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (x<6, y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands” [1], we have seen the very first band combination with 4 UL CCs being proposed which is DC_3C_n7B with UL DC_3C_n7B. Since prior to Rel-19, only up to 3 UL CCs is allowed for any band combination within FR1, in our view, 4 UL CCs within FR1 should be considered as a new RF feature and subject to further technical discussions before the feature can be included in the basket WIDs.

Observation 1: Prior to Rel-19, only up to 3 UL CCs is allowed for any band combination within FR1.

Proposal 1: 4 UL CCs within FR1 should be considered as a new RF feature and subject to further technical discussions before the feature can be included in the basket WIDs.

In Rel-18, for inter-band UL CA or EN-DC with intra-band contiguous UL CA in one of the two UL bands, a new type of MSD requirement due to UL triple-beat inter-modulation interference was introduced. Though the concern on the practicality of the current MSD requirements with intra-band contiguous UL CA being specified with relatively small UL RB allocations non-contiguously distributed between the two contiguous UL carriers has been brought up in the past two RAN4 meetings, it was not agreed to remove those MSD requirements. And whether to continue specifying such MSD requirements would be further discussed in Rel-19. If RAN4 would maintain the MSD analysis with (1RB + 1RB) UL configuration in contiguous UL CA in Rel-19, a potential quadruple-beat inter-modulation interference analysis may need to be carried out for band combination with 4 UL CCs such as for DC_3C_n7B with UL DC_3C_n7B where the framework has never been developed in the past and is expected to incur substantial technical efforts.

Observation 2: In Rel-18, for inter-band UL CA or EN-DC with intra-band contiguous UL CA in one of the two UL bands, a new type of MSD requirement due to UL triple-beat inter-modulation interference was introduced.

Observation 3: If RAN4 would maintain the MSD analysis with (1RB + 1RB) UL configuration in contiguous UL CA in Rel-19, a potential quadruple-beat inter-modulation interference analysis may need to be carried out for band combination with 4 UL CCs.

Based on the above observations, there can be two options on how to handle the band combination with 4 UL CCs in Rel-19. The first option is to postpone the band combination with 4 UL CCs till the necessity of the potential quadruple-beat MSD analysis is clarified. The second option is to stop the MSD analysis with (1RB + 1RB) UL configuration in contiguous UL CA in Rel-19 and do not define any new MSD framework for band combination with 4 UL CCs such that the combination can be directly proposed for block approval.

Proposal 2: RAN4 to consider the following two options on how to handle the band combination with 4 UL CCs in Rel-19.

- Option 1: Postpone the band combination with 4 UL CCs till the necessity of the potential quadruple-beat MSD analysis is clarified.

- Option 2: Stop the MSD analysis with (1RB + 1RB) UL configuration in contiguous UL CA in Rel-19 and do not define any new MSD framework for band combination with 4 UL CCs such that the combination can be directly proposed for block approval.

# 3	Conclusion

In this contribution, we share our views on how the band combination with 4 UL CCs may be handled in Rel-19.

Observation 1: Prior to Rel-19, only up to 3 UL CCs is allowed for any band combination within FR1.

Proposal 1: 4 UL CCs within FR1 should be considered as a new RF feature and subject to further technical discussions before the feature can be included in the basket WIDs.

Observation 2: In Rel-18, for inter-band UL CA or EN-DC with intra-band contiguous UL CA in one of the two UL bands, a new type of MSD requirement due to UL triple-beat inter-modulation interference was introduced.

Observation 3: If RAN4 would maintain the MSD analysis with (1RB + 1RB) UL configuration in contiguous UL CA in Rel-19, a potential quadruple-beat inter-modulation interference analysis may need to be carried out for band combination with 4 UL CCs.

Proposal 2: RAN4 to consider the following two options on how to handle the band combination with 4 UL CCs in Rel-19.

- Option 1: Postpone the band combination with 4 UL CCs till the necessity of the potential quadruple-beat MSD analysis is clarified.

- Option 2: Stop the MSD analysis with (1RB + 1RB) UL configuration in contiguous UL CA in Rel-19 and do not define any new MSD framework for band combination with 4 UL CCs such that the combination can be directly proposed for block approval.

# 4	Reference

RP-241673 “Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (x<=6) and single or two NR Supplementary Uplink (SUL) bands”, RAN4 Chair, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17th – 20th, 2024