3GPP RAN WG4 Meeting #112		R4-2411167

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	7.13.2

Source:	Apple

Title:	On NR new 1.4GHz band REFSENS

WI/SI:	NR_FDD_1400MHz-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

A new spectrum work item on NR 1.4GHz band in US was approved in RAN #104 meeting [1]. The WID objectives include,

- Specify a new NR FDD operating band for the US 1.4 GHz allocation spanning 1390-1395 MHz for UL and 1432-1435 MHz for DL.

- Specify band numbering and RF characteristics of the 1.4 GHz band.

- Specify 3 MHz channel BW for DL and UL in the 1.4 GHz band, with 15 kHz SCS.

In this contribution, we share our views on the REFSENS requirement for this new NR band.

# Discussion

Figure 2-1 illustrates the spectrum allocation of the new NR 1.4GHz band.

Figure 2-1 Spectrum allocation for the new NR 1.4GHz band

Though the UL of this new NR band may potentially support up to 5MHz channel BW while DL can only support 3MHz channel BW, considering that 3GPP has never defined asymmetric UL/DL channel BW where UL channel BW is wider than DL channel BW, the UE RF requirements shall be specified based on symmetric UL/DL channel BW at 3MHz.

Observation 1: For the new NR 1.4GHz band, the UE RF requirements shall be specified based on symmetric UL/DL channel BW at 3MHz.

On the other hand, the 3MHz UL channel BW technically can be allocated on any channel raster within the 5MHz band range. That would mean the UL/DL duplex spacing could vary from 40MHz to 42MHz. The WID objective however does not clarify whether the UL/DL duplex spacing is fixed or can be variable. We think it is necessary for the proponent company to clarify the intended UL/DL duplex spacing such that there would be no ambiguity on how the UL channel is to be allocated.

Proposal 1: For the new NR 1.4GHz band, the intended UL/DL duplex spacing shall be clarified such that there would be no ambiguity on how the UL channel is to be allocated.

On the UE REFSENS side, considering that the smallest UL/DL gap is already more than 12 channel BW apart, the UL induced noise impact to the DL is expected to be rather negligible except for the UL second order inter-modulation (IM2) noise caused by Rx down-conversion mixer. The IM2 noise could be more pronounced at channel BW less than 5MHz. As a result, the 3MHz REFSENS can be derived based on Rx noise figure (including duplexer insertion loss) or referenced to the REFSENS requirements for the frequency bands of the similar frequency range, such as LTE B21, NR bands n50/n51, and taking into account the impact of UL IM2.

Observation 2: For the new NR 1.4GHz band, as the smallest UL/DL gap is already more than 12 channel BW apart, the UL induced noise impact to the DL is expected to be rather negligible except for the UL IM2 noise caused by Rx down-conversion mixer.

In this contribution, we derive the 3MHz REFSENS requirement by referencing to the 5MHz REFSENS for LTE B21, NR bands n50/n51 which has been specified as -100dBm for 15kHz SCS. A 0.5dB degradation is also added to account for the IM2 noise impact. The 3MHz REFSENS can then be calculated as,

REFSENS_3MHz =

REFSENS_5MHz + 10log10(NRB_3MHz/NRB_5MHz)dB + 0.5dB = -100dBm + 10log10(15/25)dB + 0.5dB = -101.7dBm

Owing to that smallest UL/DL gap is already more than 12 channel BW apart where the fully allocated UL induced noise impact to the DL is expected to be rather negligible except for UL IM2, the UL configuration for REFSENS shall be specified as 15RB and the REFSENS is expected to be the same over the possible duplex spacing range.

Observation 3: For the new NR 1.4GHz band, the 3MHz REFSENS is expected to be the same over the possible duplex spacing range.

Proposal 2: For the new NR 1.4GHz band, the REFSENS requirement is specified as below:

| Operating Band | SCS (kHz) | 3MHz |  |
| --- | --- | --- | --- |
| nxxx | 15 | REFSENS (dBm) | UL Configuration (RB) |
|  |  | -101.7 | 15 |

# 3	Conclusion

In this contribution, we share our views on the REFSENS requirement for the new NR 1.4GHz band.

Observation 1: For the new NR 1.4GHz band, the UE RF requirements shall be specified based on symmetric UL/DL channel BW at 3MHz.

Proposal 1: For the new NR 1.4GHz band, the intended UL/DL duplex spacing shall be clarified such that there would be no ambiguity on how the UL channel is to be allocated.

Observation 2: For the new NR 1.4GHz band, as the smallest UL/DL gap is already more than 12 channel BW apart, the UL induced noise impact to the DL is expected to be rather negligible except for the UL IM2 noise caused by Rx down-conversion mixer.

Observation 3: For the new NR 1.4GHz band, the 3MHz REFSENS is expected to be the same over the possible duplex spacing range.

Proposal 2: For the new NR 1.4GHz band, the REFSENS requirement is specified as below:

| Operating Band | SCS (kHz) | 3MHz |  |
| --- | --- | --- | --- |
| nxxx | 15 | REFSENS (dBm) | UL Configuration (RB) |
|  |  | -101.7 | 15 |

# 4	Reference

RP-241648 “Introduction of the 1.4 GHz Band”, MidWave Wireless, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17th – 20th, 2024