3GPP RAN WG4 Meeting #112		R4-2411168

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.1.1.1.1

Source:	Apple

Title:	MPR for PC1.5 intra-band contiguous UL CA

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the discussions were concluded with an approved “WF on higher power UE” [2]. For PC1.5 intra-band UL CA, RAN4 has agreed to evaluate MPR/A-MPR for contiguous UL CA first then work on non-contiguous UL CA afterwards. In this contribution, we provide our initial MPR analysis for PC1.5 intra-band contiguous UL CA where MPR values were derived based on the PC2 MPR difference between single carrier and contiguous UL CA with dual Tx.

# Discussion

MPR/A-MPR evaluations usually involve a rather time-consuming simulation and/or measurement campaign. For dual Tx architecture, the evaluation process is of particular challenging as the Reverse Inter-Modulation Distortion (RIMD) between two power amplifiers (PAs) via finite antenna isolation is not straightforward to model nor is easy to emulate on measurement bench. Therefore, the MPR inaccuracy is expected to be higher than for single Tx architecture.

For PC1.5 intra-band contiguous UL CA, RAN4 has agreed to prioritize the dual Tx (26dBm+26dBm) single LO architecture for MPR/A-MPR evaluations. In principle, we can go through another laborious round of simulation and/or measurement champaign as was carried out for PC2 MPR/A-MPR with dual Tx. However, considering the aforementioned inaccuracy over this evaluation process, it may not be worth the effort to repeat the similar process as has been done for PC2 MPR evaluation with dual Tx. Instead, we could leverage the existing PC2 MPR requirements with dual Tx between single carrier and intra-band contiguous UL CA to derive the PC1.5 MPR requirements as in our view it is reasonable to assume the PC2 MPR difference between single carrier and intra-band contiguous UL CA would be similar to that of PC1.5 based on the commonality of Tx RF architecture between them.

Figure 2-1 illustrates the concept on how to derive the MPR values for PC1.5 intra-band contiguous UL CA.

Figure 2-1 Derivation of PC1.5 contiguous UL CA MPR

Table 2-1 summarizes the dual-Tx PC2 MPR difference between single carrier and contiguous UL CA (UL CA MPR – single CC MPR) which are calculated based on the specified dual-Tx PC2 MPR values for single carrier and contiguous UL CA in Table 2-2 and Table 2-3 respectively as recaptured from the latest UE RF specifications [3].

| Modulation |  | MPR for bandwidth class B (dB) |  | MPR for bandwidth class C (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 | 4.0 | 3.5 | 7.0 |
|  | QPSK | 2.5 | 3.0 | 3.0 | 6.0 |
|  | 16QAM | 2.0 | 2.5 | 2.0 | 5.5 |
|  | 64QAM | 1.0 | 2.5 | 3.0 | 5.0 |
|  | 256QAM | 1.0 | 1.5 | 2.5 | 3.0 |
| CP-OFDM | QPSK | 1.0 | 2.0 | 2.0 | 5.0 |
|  | 16QAM | 1.0 | 2.0 | 1.5 | 5.0 |
|  | 64QAM | -0.5 | 1.0 | 1.0 | 4.0 |
|  | 256QAM | -1.0 | -1.0 | -0.5 | 0.5 |

Table 2-1 Dual-Tx PC2 MPR difference between single carrier and contiguous UL CA

Table 2-2 Dual-Tx PC2 MPR for single carrier

Table 2-3 Dual-Tx PC2 MPR for contiguous UL CA

Table 2-4 and Table 2-5 summarize the derived PC1.5 MPR values for intra-band contiguous UL CA with 10dB and 20dB antenna isolation respectively which are calculated by summing the dual-Tx PC2 MPR difference in Table 2-1 and the specified PC1.5 MPR for single carrier in Table 2-6 and Table 2-7 as also recaptured from the latest UE RF specifications [3].

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.5 | 6.0 | 4.0 | 9.0 |
|  | QPSK | 3.0 | 5.5 | 3.5 | 8.5 |
|  | 16QAM | 3.5 | 6.0 | 3.5 | 9.0 |
|  | 64QAM | 4.5 | 6.5 | 6.5 | 9.0 |
|  | 256QAM | 7.5 | 8.0 | 9.0 | 9.5 |
| CP-OFDM | QPSK | 3.0 | 6.5 | 4.0 | 9.5 |
|  | 16QAM | 3.5 | 6.5 | 4.0 | 9.5 |
|  | 64QAM | 4.0 | 6.0 | 5.5 | 9.0 |
|  | 256QAM | 7.5 | 7.5 | 8.0 | 9.0 |

Table 2-4 Derived PC1.5 MPR values for intra-band contiguous UL CA with 10dB antenna isolation

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 | 5.5 | 3.5 | 8.5 |
|  | QPSK | 2.5 | 5.0 | 3.0 | 8.0 |
|  | 16QAM | 3.0 | 5.5 | 3.0 | 8.5 |
|  | 64QAM | 4.0 | 6.0 | 6.0 | 8.5 |
|  | 256QAM | 6.5 | 7.0 | 8.0 | 8.5 |
| CP-OFDM | QPSK | 2.5 | 6.0 | 3.5 | 9.0 |
|  | 16QAM | 3.0 | 6.0 | 3.5 | 9.0 |
|  | 64QAM | 3.5 | 5.5 | 5.0 | 8.5 |
|  | 256QAM | 6.5 | 6.5 | 7.0 | 8.0 |

Table 2-5 Derived PC1.5 MPR values for intra-band contiguous UL CA with 20dB antenna isolation

Table 2-6 Specified PC1.5 MPR values for single carrier with 10dB antenna isolation

Table 2-7 Specified PC1.5 MPR values for single carrier with 20dB antenna isolation

# 3	Conclusion

In this contribution, we provide our initial MPR analysis for PC1.5 intra-band contiguous UL CA where MPR values were derived based on the PC2 MPR difference between single carrier and contiguous UL CA with dual Tx.

Proposal: RAN4 to take the following MPR values for PC1.5 intra-band contiguous UL CA into consideration.

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.5 | 6.0 | 4.0 | 9.0 |
|  | QPSK | 3.0 | 5.5 | 3.5 | 8.5 |
|  | 16QAM | 3.5 | 6.0 | 3.5 | 9.0 |
|  | 64QAM | 4.5 | 6.5 | 6.5 | 9.0 |
|  | 256QAM | 7.5 | 8.0 | 9.0 | 9.5 |
| CP-OFDM | QPSK | 3.0 | 6.5 | 4.0 | 9.5 |
|  | 16QAM | 3.5 | 6.5 | 4.0 | 9.5 |
|  | 64QAM | 4.0 | 6.0 | 5.5 | 9.0 |
|  | 256QAM | 7.5 | 7.5 | 8.0 | 9.0 |

Derived PC1.5 MPR values for intra-band contiguous UL CA with 10dB antenna isolation

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 | 5.5 | 3.5 | 8.5 |
|  | QPSK | 2.5 | 5.0 | 3.0 | 8.0 |
|  | 16QAM | 3.0 | 5.5 | 3.0 | 8.5 |
|  | 64QAM | 4.0 | 6.0 | 6.0 | 8.5 |
|  | 256QAM | 6.5 | 7.0 | 8.0 | 8.5 |
| CP-OFDM | QPSK | 2.5 | 6.0 | 3.5 | 9.0 |
|  | 16QAM | 3.0 | 6.0 | 3.5 | 9.0 |
|  | 64QAM | 3.5 | 5.5 | 5.0 | 8.5 |
|  | 256QAM | 6.5 | 6.5 | 7.0 | 8.0 |

Derived PC1.5 MPR values for intra-band contiguous UL CA with 20dB antenna isolation

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2410750 “WF on higher power UE”, Samsung, 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

3GPP TS 38.101-1 V18.6.0 (2024-06)