3GPP RAN WG4 Meeting #112		R4-2411169

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.1.1.1.2

Source:	Apple

Title:	Rel-19 HPUE for inter-band UL CA/EN-DC with 2Tx or 3Tx

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the discussions were concluded with an approved “WF on higher power UE” [2]. For HPUE inter-band UL CA/EN-DC with 2Tx or 3Tx, it was agreed to define only one set of requirements for both handheld UE and FWA. The remaining open issues are whether to collect configurations to derive general requirements and whether to introduce 3UL CC with 3Tx in Rel-19. In this contribution, we share our views on the general requirement aspect for HPUE inter-band UL CA/EN-DC with 2Tx or 3Tx and clarify the scope on 3UL CC with 3Tx.

# Discussion

On the new general requirement aspect for HPUE inter-band UL CA/EN-DC with 2Tx or 3Tx, we have provided our analysis in last RAN4 meeting [3] which can be summarized in the following observations.

Observation 1: For PC1.5 inter-band UL CA with 2Tx or 3Tx configuration, there would be no new Tx requirement to be developed except for “configured transmitted power for Inter-band CA” with 2Tx where ΔPPowerClass definition may need to be revised to accommodate PC1.5.

Observation 2: For PC1.5 inter-band UL CA with 2Tx or 3Tx configuration, new MSD framework may be required only for UL configurations consisting of PC2 FDD band(s).

Observation 3: For PC2 2-band inter-band EN-DC with 2Tx or 3Tx configuration, no new general requirement needs to be developed.

Observation 4: For PC1.5 inter-band EN-DC with 2Tx or 3Tx configuration, there would be no new Tx requirement to be developed except for “configured output power level for Inter-band EN-DC within FR1” where ΔPPowerClass definition may need to be revised to accommodate PC1.5.

Observation 5: For PC1.5 inter-band EN-DC with 2Tx or 3Tx configuration, new MSD framework may be required only for UL configurations consisting of PC2 FDD band(s).

For PC1.5 inter-band UL CA/EN-DC with 2Tx or 3Tx configuration, although the ΔPPowerClass definition in configured transmitted power requirement may need to be revised to accommodate PC1.5 in order to maintain the specifications consistency, considering that practically ΔPPowerClass has never been used by any UE in the field to mitigate SAR issue and there is no test case in conformance tests to verify the requirement, it may not be worth the time and efforts in RAN4 to introduce such specifications changes which in our view could be a rather nontrivial task.

Observation 6: ΔPPowerClass has never been used by any UE in the field to mitigate SAR issue and there is no test case in conformance tests to verify the requirement.

Proposal 1: Do not introduce ΔPPowerClass related specifications changes for PC1.5 inter-band UL CA/EN-DC with 2Tx or 3Tx configuration.

On the potential new MSD framework which may be required only for PC1.5 UL configurations consisting of PC2 FDD band(s), we propose RAN4 to decide whether PC2 FDD bands would be considered in PC1.5 inter-band UL CA/EN-DC at the early phase of the work item as new MSD framework could take long meeting cycles to develop. If RAN4 decides to consider PC2 FDD bands in PC1.5 inter-band UL CA/EN-DC configurations, a few exemplary band combinations would be required for companies to exercise the MSD analysis.

Proposal 2: RAN4 to decide whether PC2 FDD bands would be considered in PC1.5 inter-band UL CA/EN-DC at the early phase of the work item.

Proposal 3: If RAN4 decides to consider PC2 FDD bands in PC1.5 inter-band UL CA/EN-DC configurations, a few exemplary band combinations would be required for companies to exercise the MSD analysis.

On the aspect of 3UL CC with 3Tx, in our view, the UL configuration CA_nXA-nY(2A) which consists of 3 discrete spectrum blocks should be out of the scope for this work item as this is a new feature which may require general requirement development even for PC3. On the other hand, UL configuration CA_nXA-nYB has already been supported in Rel-18. If this configuration would be considered in this work item, the CA_nYB part of the UL shall be limited up to PC2 only as PC1.5 general requirement is still under development. To better manage RAN4 workload on the already busy and demanding objectives of this work item, we suggest to limit the UL configuration to one CC only in each of the UL bands.

Observation 7: UL configuration CA_nXA-nY(2A) which consists of 3 discrete spectrum blocks is out of the scope for this work item.

Proposal 4: For PC1.5 inter-band UL CA/EN-DC, limit the UL configuration to one CC only in each of the UL bands to avoid RAN4 workload upsurge on the already busy and demanding objectives of this work item.

# 3	Conclusion

In this contribution, we share our views on the general requirement aspect for HPUE inter-band UL CA/EN-DC with 2Tx or 3Tx and clarify the scope on 3UL CC with 3Tx.

Observation 1: For PC1.5 inter-band UL CA with 2Tx or 3Tx configuration, there would be no new Tx requirement to be developed except for “configured transmitted power for Inter-band CA” with 2Tx where ΔPPowerClass definition may need to be revised to accommodate PC1.5.

Observation 2: For PC1.5 inter-band UL CA with 2Tx or 3Tx configuration, new MSD framework may be required only for UL configurations consisting of PC2 FDD band(s).

Observation 3: For PC2 2-band inter-band EN-DC with 2Tx or 3Tx configuration, no new general requirement needs to be developed.

Observation 4: For PC1.5 inter-band EN-DC with 2Tx or 3Tx configuration, there would be no new Tx requirement to be developed except for “configured output power level for Inter-band EN-DC within FR1” where ΔPPowerClass definition may need to be revised to accommodate PC1.5.

Observation 5: For PC1.5 inter-band EN-DC with 2Tx or 3Tx configuration, new MSD framework may be required only for UL configurations consisting of PC2 FDD band(s).

Observation 6: ΔPPowerClass has never been used by any UE in the field to mitigate SAR issue and there is no test case in conformance tests to verify the requirement.

Proposal 1: Do not introduce ΔPPowerClass related specifications changes for PC1.5 inter-band UL CA/EN-DC with 2Tx or 3Tx configuration.

Proposal 2: RAN4 to decide whether PC2 FDD bands would be considered in PC1.5 inter-band UL CA/EN-DC at the early phase of the work item.

Proposal 3: If RAN4 decides to consider PC2 FDD bands in PC1.5 inter-band UL CA/EN-DC configurations, a few exemplary band combinations would be required for companies to exercise the MSD analysis.

Observation 7: UL configuration CA_nXA-nY(2A) which consists of 3 discrete spectrum blocks is out of the scope for this work item.

Proposal 4: For PC1.5 inter-band UL CA/EN-DC, limit the UL configuration to one CC only in each of the UL bands to avoid RAN4 workload upsurge on the already busy and demanding objectives of this work item.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2410750 “WF on higher power UE”, Samsung, 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

R4-2407092 “Views on inter-band UL NR-CA/EN-DC with 2 bands and 2Tx and/or 3Tx”, Apple, 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024