3GPP RAN WG4 Meeting #112		R4-2411170

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.1.1.1.3

Source:	Apple

Title:	On Rel-19 increasing UE transmission high power limit

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the discussions were concluded with an approved “WF on higher power UE” [2]. For increasing UE transmission high power limit, it was agreed to use higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability as the starting point, while the applicable scenarios still remained FFS. In this contribution, we share our further views on “increasing UE transmission high power limit for inter-band UL CA and EN-DC” and propose the UE capability IEs to support increasing UE transmission high power limit for inter-band UL CA and EN-DC as defined in Rel-17, i.e., higherPowerLimit-R17 and higherPowerLimtMRDC-R17, are also applicable for any other UL power compositions with different power classes between the two UL bands. We also share our proposed 2UL IMD MSD power compositions for the possible new UL configurations based on the principle that no additional MSD requirements are specified in addition to the MSD requirements of its legacy power classes as agreed in [2].

# Discussion

The “Increasing UE power high limit for CA and DC” feature was introduced in Rel-17 [3] with the intention to maximize the UE per-band power class capability while transmitting in two UL bands simultaneously which would have been underutilized without the feature due to the total maximum output power restriction in current 3GPP UE power class definition. In Rel-17, only PC2 UL configurations with (PC3 FDD/TDD + PC2 TDD) composition have been specified to support this feature. In Rel-18, the feature was further extended to include UL configurations with (PC3 FDD/TDD + PC5 TDD) composition. Since the UE capability IEs to support increasing UE transmission high power limit for inter-band UL CA and EN-DC as defined in Rel-17, i.e., higherPowerLimit-R17 and higherPowerLimtMRDC-R17, are applicable to both Rel-17 (PC3 + PC2) and Rel-18 (PC3 + PC5) UL compositions, the same IEs should also be applicable to any other UL power compositions with different power classes between the two UL bands. Therefore, it is not necessary to define a separate UE capability in Rel-19 to support increasing UE transmission high power limit for inter-band UL CA and EN-DC with other different UL power compositions.

Proposal 1: The UE capability IEs to support increasing UE transmission high power limit for inter-band UL CA and EN-DC as defined in Rel-17, i.e., higherPowerLimit-R17 and higherPowerLimtMRDC-R17, are also applicable for any other UL power compositions with different power classes between the two UL bands.

On the applicable scenarios, with the introduction of PC2 FDD bands and PC1.5 inter-band UL CA and EN-DC, it opens up the opportunity for many new possible UL configurations where the “increasing UE power high limit for CA and DC” feature can also be leveraged, as summarized in Table 2-1.

| Power Class | UL Configuration |
| --- | --- |
| PC2 | PC2 FDD + PC5 TDD |
| PC2 | PC2 FDD + PC3 TDD |
| PC2 | PC2 FDD + PC3 FDD |
| PC1.5 | PC1.5 TDD + PC5 TDD |
| PC1.5 | PC1.5 TDD + PC3 TDD |
| PC1.5 | PC1.5 TDD + PC3 FDD |
| PC1.5 | PC1.5 TDD + PC2 TDD |
| PC1.5 | PC1.5 TDD + PC2 FDD |

Table 2-1 Possible new UL configurations to support “increasing UE power high limit” feature

For UL configurations consist of at least one FDD band, it may be subject to 2UL IMD interference issue. If PCMAX for each constituent UL band would be applied to evaluate MSD requirements, new MSD framework would be needed for these UL configurations. However, this will not only create more fragmentations in MSD test configurations but also increase specifications complexity substantially. To avoid the unnecessary new MSD framework to support the “increasing UE power high limit for CA and DC” feature, it was agreed in last RAN4 meeting that no additional MSD requirements are specified in addition to the MSD requirements of its legacy power classes [2]. Based on this principle, we came up with the 2UL IMD MSD power compositions for the possible new UL configurations to support “increasing UE power high limit” feature which is summarized in Table 2-2.

| Power Class | UL Configuration | UL Power for 2UL IMD MSD |
| --- | --- | --- |
| PC2 | PC2 FDD + PC5 TDD | 20dBm + 20dBm |
| PC2 | PC2 FDD + PC3 TDD | 23dBm + 23dBm |
| PC2 | PC2 FDD + PC3 FDD | 23dBm + 23dBm |
| PC1.5 | PC1.5 TDD + PC5 TDD | N/A |
| PC1.5 | PC1.5 TDD + PC3 TDD | N/A |
| PC1.5 | PC1.5 TDD + PC3 FDD | 27.8dBm + 23dBm |
| PC1.5 | PC1.5 TDD + PC2 TDD | N/A |
| PC1.5 | PC1.5 TDD + PC2 FDD | 26dBm + 26dBm |

Table 2-2 Proposed 2UL IMD MSD power compositions for the possible new UL configurations

Proposal 2: Use the UL power compositions in Table 2-2 to define 2UL IMD MSD requirements for the corresponding new UL configurations to support “increasing UE transmission high power limit” feature.

# 3	Conclusion

In this contribution, we share our further views on “increasing UE transmission high power limit for inter-band UL CA and EN-DC” and propose the UE capability IEs to support the feature as defined in Rel-17, i.e., higherPowerLimit-R17 and higherPowerLimtMRDC-R17, are also applicable for any other UL power compositions with different power classes between the two UL bands. We also share our proposed 2UL IMD MSD power compositions for the possible new UL configurations based on the principle that no additional MSD requirements are specified in addition to the MSD requirements of its legacy power classes.

Proposal 1: The UE capability IEs to support increasing UE transmission high power limit for inter-band UL CA and EN-DC as defined in Rel-17, i.e., higherPowerLimit-R17 and higherPowerLimtMRDC-R17, are also applicable for any other UL power compositions with different power classes between the two UL bands.

Proposal 2: Use the UL power compositions in the table below to define 2UL IMD MSD requirements for the corresponding new UL configurations to support “increasing UE transmission high power limit” feature.

| Power Class | UL Configuration | UL Power for 2UL IMD MSD |
| --- | --- | --- |
| PC2 | PC2 FDD + PC5 TDD | 20dBm + 20dBm |
| PC2 | PC2 FDD + PC3 TDD | 23dBm + 23dBm |
| PC2 | PC2 FDD + PC3 FDD | 23dBm + 23dBm |
| PC1.5 | PC1.5 TDD + PC5 TDD | N/A |
| PC1.5 | PC1.5 TDD + PC3 TDD | N/A |
| PC1.5 | PC1.5 TDD + PC3 FDD | 27.8dBm + 23dBm |
| PC1.5 | PC1.5 TDD + PC2 TDD | N/A |
| PC1.5 | PC1.5 TDD + PC2 FDD | 26dBm + 26dBm |

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2410750 “WF on higher power UE”, Samsung, 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

RP-212163 “New WID: Increasing UE power high limit for CA and DC”, China Telecom, Qualcomm, 3GPP TSG-RAN Meeting #93-e, September 13th – 17th, 2021