3GPP RAN WG4 Meeting #112		R4-2411171

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.7.2

Source:	Apple

Title:	On PC2 RedCap UE in FR1

WI/SI:	NR_PC2_RedCap_UE-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

A new work item on NR power class 2 (PC2) RedCap UE was approved in RAN #104 meeting [1]. The main objective of the core part is to apply the PC2 RF requirements for non-RedCap UE to RedCap UE based on 1Tx architecture with 26 dBm PA and the following additional clarifications:

- The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

- For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way.

- Check at RAN #106 on whether to define PC2 RF requirements for FDD bands and the applicable form factors for FDD bands (if defined).

In this contribution, we share our initial view on NR PC2 RedCap UE in FR1 and propose to enable the feature for all TDD bands which have been specified to support PC2 for non-RedCap UE. We also provide an example specifications change text proposal on how to enable RedCap UE PC2 support for TDD bands in TS 38.101-1 for consideration.

# Discussion

When NR RedCap UE was first introduced in Rel-17 with only power class 3 (PC3) supported, it was generally agreed that all the single-carrier (non-CA) transmit requirements for NR non-RedCap UE in FR1 can be directly applied to NR RedCap UE. However, among all the transmit requirements, only the “maximum output power” was explicitly indicated which is simply referenced to the PC3 requirements specified in clause 6.2.1, as shown in the following excerpt recaptured from TS 38.101-1 [2].

Figure 2-1 Rel-17 RedCap UE maximum output power requirements

Observation 1: For the Rel-17 NR RedCap UE, all the PC3 single-carrier (non-CA) transmit requirements for NR non-RedCap UE in FR1 can be directly applied.

For PC2 RedCap UE in FR1, the same approach for specifying the PC3 RedCap UE transmit requirements can also be applied as all the PC2 transmit requirements for NR non-RedCap UE in FR1 as specified in clause 6.2 which is for 1Tx only can also be directly applied to PC2 RedCap UE.

Observation 2: For PC2 RedCap UE in FR1, all the PC2 transmit requirements for NR non-RedCap UE as specified in clause 6.2 which is for 1Tx only can also be directly applied.

Though the initial WID only considers a few requested example TDD bands (n77, n78, n41, n79) to support PC2 for RedCap UE in FR1, in our view, it would be more efficient to enable the feature for all TDD bands which have been specified to support PC2 for non-RedCap UE since there is no new band specific requirement to be developed for all TDD bands and the specifications can be written in a band agnostic way to support PC2. Otherwise, every time a TDD band would be requested to support PC2 for RedCap UE, the specifications would need to be revised to add a band number which is rather inefficient.

Observation 3: It would be more efficient to enable the RedCap UE PC2 feature for all TDD bands which have been specified to support PC2 for non-RedCap UE since there is no new band specific requirement to be developed for all TDD bands and the specifications can be written in a band agnostic way to support PC2.

Proposal 1: Enable RedCap UE PC2 feature for all TDD bands which have been specified to support PC2 for non-RedCap UE.

As the same approach for specifying the PC3 RedCap UE transmit requirements also applies to PC2 RedCap UE in FR1, it is rather straightforward to change the specifications to enable the RedCap UE PC2 feature for TDD bands. Below is an example specifications change text proposal in TS 38.101-1 for consideration.

### 6.2I.1	Maximum output power for RedCap

For Redcap UE, the requirements for power class 3 FDD and TDD bands and power class 2 TDD bands specified in clause 6.2.1 apply.

Proposal 2: Consider the specifications change text proposal above to enable the RedCap UE PC2 feature for TDD bands.

# 3	Conclusion

In this contribution, we share our initial view on NR PC2 RedCap UE in FR1 and propose to enable the feature for all TDD bands which have been specified to support PC2 for non-RedCap UE. We also provide an example specifications change text proposal on how to enable RedCap UE PC2 support for TDD bands in TS 38.101-1 for consideration.

Observation 1: For the Rel-17 NR RedCap UE, all the PC3 single-carrier (non-CA) transmit requirements for NR non-RedCap UE in FR1 can be directly applied.

Observation 2: For PC2 RedCap UE in FR1, all the PC2 transmit requirements for NR non-RedCap UE as specified in clause 6.2 which is for 1Tx only can also be directly applied.

Observation 3: It would be more efficient to enable the RedCap UE PC2 feature for all TDD bands which have been specified to support PC2 for non-RedCap UE since there is no new band specific requirement to be developed for all TDD bands and the specifications can be written in a band agnostic way to support PC2.

Proposal 1: Enable RedCap UE PC2 feature for all TDD bands which have been specified to support PC2 for non-RedCap UE.

Proposal 2: Consider the specifications change text proposal below to enable the RedCap UE PC2 feature for TDD bands.

### 6.2I.1	Maximum output power for RedCap

For Redcap UE, the requirements for power class 3 FDD and TDD bands and power class 2 TDD bands specified in clause 6.2.1 apply.

# 4	References

RP-241665 “New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1”, China Telecom, MediaTek, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17th – 20th, 2024

3GPP TS 38.101-1 V18.6.0 (2024-06)