3GPP RAN WG4 Meeting #112		R4-2411172

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.25.2.1

Source:	Apple

Title:	On NR NTN RedCap UE RF requirements

WI/SI:	NR_NTN_Ph3-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

Considering that the support of RedCap devices operating in FR1 band NR-NTN networks can offer enhanced service capabilities (wideband/broadband) compared to IoT-NTN while ensuring low-complexity devices and the global coverage would clearly benefit RedCap devices, an objective on the “support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands” is included in the Rel-19 new WID on “Non-Terrestrial Networks (NTN) for NR Phase 3” led by RAN2 [1]. In the past two RAN4 meetings, the majority of the UE RF requirements for NTN (e)RedCap UE have been agreed and captured in two corresponding WFs approved in each respective RAN4 meeting [2,3]. The only remaining open issues are the HD-FDD REFSENS tightening relative to FD-FDD REFSENS and the maximum output power. In this contribution, we share our views and proposals on these two remaining open issues for NR NTN RedCap UE RF requirements.

# Discussion

For 2Rx HD-FDD, the 5MHz REFSENS tightening relative to 2Rx FD-FDD 5MHz REFSENS can follow the previous agreement for RedCap UE [4] which is summarized in Table 2-1.

| 2Rx FD-FDD 5MHz REFSENS | 2Rx HD-FDD 5MHz REFSENS Tightening |
| --- | --- |
| ≤ -100 dBm | 0 dB |
| > -100 dBm and ≤ -99 dBm | 0.5 dB |
| > -99 dBm | 0.8 dB |

Table 2-1 2Rx HD-FDD 5MHz REFSENS tightening relative to 2Rx FD-FDD 5MHz REFSENS

For HD-FDD, since there is no UL self-interference to DL, the REFSENS for channel BW wider than 5MHz can be linearly scaled over 5MHz and calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n = 1 for 15kHz SCS, n = 2 for 30kHz SCS, and n = 4 for 60kHz SCS. Based on the above REFSENS tightening and the scaling over channel BW, the 2Rx HD-FDD REFSENS can be derived and specified as in Table 2-2.

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n256 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 |  |
| n254 | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

Table 2-2 Proposed NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements

On the other hand, UL RB restriction would not be needed for HD-FDD REFSENS as there is no UL self-interference to DL. Therefore, the UL configuration for HD-FDD REFSENS shall be specified with full RB allocation which has already been agreed in the approved WF of last RAN4 meeting [3]. Table 2-3 summarizes the uplink configurations for NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements.

| Operating Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 | 25 | 50 | 75 | 100 | HD-FDD |
| n256 | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | 100 | HD-FDD |
| n255 | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 |  | HD-FDD |
| n254 | 30 |  | 24 | 36 |  |  |
|  | 60 |  | 10 | 18 |  |  |

Table 2-3 Uplink configurations for NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements

Proposal 1: NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements and UL configurations are specified as in Table 2-2 and Table 2-3 respectively.

For NR NTN (e)RedCap UE maximum output power, the following two options were considered in last RAN4 meeting [3].

- Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band.

- Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE

Considering that both PC2 for NR NTN UE and PC2 for RedCap UE TDD bands are being developed in two separate ongoing work items [5,6], we propose to wait for the completion of these two features before the introduction of HPUE for NR NTN (e)RedCap UE.

Proposal 2: For NR NTN (e)RedCap UE maximum output power, wait for the completion of PC2 for NR NTN UE and PC2 for RedCap UE TDD bands before the introduction of HPUE.

# 3	Conclusion

In this contribution, we share our views and proposals on the two remaining open issues for NR NTN RedCap UE RF requirements.

Proposal 1: NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements and UL configurations are specified as in Table 2-2 and Table 2-3 respectively.

Proposal 2: For NR NTN (e)RedCap UE maximum output power, wait for the completion of PC2 for NR NTN UE and PC2 for RedCap UE TDD bands before the introduction of HPUE.

# 4	References

RP-234078 “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Huawei, 3GPP TSG-RAN Meeting #102, Edinburgh, Scotland, December 11th – 15th, 2023

R4-2406610 “WF on NR_NTN_Ph3_UERF”, Qualcomm, 3GPP TSG RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th, 2024

R4-2410574 “WF on Rel-19 phase 3 NR-NTN UE RF requirements”, Qualcomm Inc., 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

R4-2203692 “RedCap UE HD-FDD REFSENS requirements”, Apple, 3GPP TSG RAN WG4 Meeting #102-e, Online, February 21st – March 3rd, 2022

RP-240857 “Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 Chair, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18th -21st, 2024

RP-241665 “New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1”, China Telecom, MediaTek, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17th – 20th, 2024