3GPP RAN WG4 Meeting #112		R4-2411175

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	11

Source:	Apple

Title:	Motivation for UL Tx switching for FR1 intra-band non-contiguous UL CA in Rel-19

WI/SI:

Release:	Rel-19

Document for:	Information

# 1	Introduction

UL Tx switching was first introduced in 3GPP Rel-16 as a new feature to enhance inter-band EN-DC, inter-band UL CA, and SUL operations, where the switching is either between (1Tx + 1Tx) and (0Tx + 2Tx) or (1Tx + 0Tx) and (0Tx + 2Tx) for two UL carriers in two corresponding bands [1]. The feature provides the benefits of coverage improvement, UL throughput enhancement, and latency reduction. Considering that the same feature can very well be applied to FR1 intra-band non-contiguous UL CA where UE is typically limited with two Tx paths for simultaneous transmission, in this contribution, we provide the motivation on introducing the UL Tx switching feature for FR1 intra-band non-contiguous UL CA as part of the Rel-19 RAN4 objectives.

# Discussion

## 2.1	Motivation for UL Tx switching for FR1 intra-band NC UL CA

Similar to UL Tx switching for inter-band UL CA, the UL Tx switching for intra-band non-contiguous UL CA allows UE to dynamically switch between the two non-contiguous UL carriers without the RRC reconfiguration process. The switching can be based on the channel conditions of the two carriers where one carrier may have better propagation condition than the other due to frequency selective fading.

Observation 1: UL Tx switching for intra-band non-contiguous UL CA allows UE to dynamically switch between the two non-contiguous UL carriers without the RRC reconfiguration process.

On the other hand, the UL Tx switching avoids the potential high MPR/A-MPR for simultaneous non-contiguous UL CA transmission due to clustered out-of-band emissions caused by 2UL inter-modulations (IMD) as conceptually illustrated in Figure 2.1-1. As a result, the UL coverage can be improved yet without compromising UL throughput performance as compared to the non-switched UL CA which will be further elaborated below.

Figure 2.1-1 Potential high MPR for intra-band non-contiguous UL CA due to IMD emissions

Observation 2: The UL Tx switching avoids the potential high MPR/A-MPR for simultaneous non-contiguous UL CA transmission due to clustered out-of-band emissions caused by 2UL inter-modulations.

For simultaneous intra-band non-contiguous UL CA transmission, due to the UE limitation on number of Tx path at two, only non-MIMO operation can be supported for each carrier, as is shown in Figure 2.1-2. However, with UL Tx switching, each UL carrier can leverage the two Tx paths to support UL MIMO as conceptually illustrated in Figure 2.1-3, as a result, the UL throughput performance for UL Tx switching is not compromised as compared to non-switched UL CA.

Figure 2.1-2 Only non-MIMO operation can be supported for intra-band non-contiguous UL CA

Figure 2.1-3 UL MIMO can be supported with UL Tx switching

Observation 3: UL Tx switching does not compromise UL throughput performance as compared to UL CA as each switched carrier can support UL MIMO while UL CA can only support non-MIMO operation for each carrier due to the limitation of UL Tx path for simultaneous transmission.

Another potential benefit for intra-band non-contiguous UL CA with UL Tx switching is that only one frequency synthesizer (1 LO) is required to support the operation while for simultaneous transmission, it would require two frequency synthesizers (2 LOs) to support the operation when the frequency span between the two carriers is wider than 200 MHz.

Observation 4: For intra-band non-contiguous UL CA with UL Tx switching, only one frequency synthesizer (1 LO) is required to support the operation, while for simultaneous transmission it would require two frequency synthesizers (2 LOs) to support the operation when the frequency span between the two carriers is wider than 200 MHz.

## 2.2	Rel-19 RAN4 objectives for Intra-CA UL Tx switching

The following Rel-19 RAN4 objectives for UL Tx switching for FR1 intra-band non-contiguous UL CA are proposed for consideration:

- Introduce UL Tx switching feature for FR1 intra-band non-contiguous UL CA.

- Specify the requirement of switching period.

- Introduce new feature/capability signaling if necessary.

# 3	Conclusion

In this contribution, we provide the motivation on introducing the UL Tx switching feature for FR1 intra-band non-contiguous UL CA as part of the Rel-19 RAN4 objectives for consideration.

Observation 1: UL Tx switching for intra-band non-contiguous UL CA allows UE to dynamically switch between the two non-contiguous UL carriers without the RRC reconfiguration process.

Observation 2: The UL Tx switching avoids the potential high MPR/A-MPR for simultaneous non-contiguous UL CA transmission due to clustered out-of-band emissions caused by 2UL inter-modulations.

Observation 3: UL Tx switching does not compromise UL throughput performance as compared to UL CA as each switched carrier can support UL MIMO while UL CA can only support non-MIMO operation for each carrier due to the limitation of UL Tx path for simultaneous transmission.

Observation 4: For intra-band non-contiguous UL CA with UL Tx switching, only one frequency synthesizer (1 LO) is required to support the operation, while for simultaneous transmission it would require two frequency synthesizers (2 LOs) to support the operation when the frequency span between the two carriers is wider than 200 MHz.

The following Rel-19 RAN4 objectives for UL Tx switching for FR1 intra-band non-contiguous UL CA are proposed for consideration:

- Introduce UL Tx switching feature for FR1 intra-band non-contiguous UL CA.

- Specify the requirement of switching period.

- Introduce new feature/capability signaling if necessary.

# 4	Reference

RP-192282, "WID revision: RF requirements for NR frequency range 1 (FR1)", Huawei, HiSilicon, China Telecom, 3GPP TSG RAN Meeting #84, Newport Beach, USA, September 16th – 20th, 2019