3GPP RAN4 Meeting #112		R4-2411176

Maastricht, The Netherlands, August 19 – 23, 2024

Agenda item:	11

Source:	Apple, Skyworks Solutions Inc., BT plc, TELUS, Bell Mobility, Anterix, Southern Linc

Title:	Motivation for a new work item on LB+LB CA based on switching in Rel-19

Release:	Rel-19

Document for:	Information

# 1	Introduction

During the RAN #104 meeting an operator request for low band + low band (LB+LB) carrier aggregation based on switching was submitted and discussed [1].  Considering the practicalities of handset RF front end architectures available in the market today, the operators have requested 3GPP to consider specifying a solution which can potentially accommodate the related limitations.  This contribution describes the relevant LB+LB CA scenarios and provides the motivation for a new Rel-19 work item to specify a switching solution to enable their adoption into practical handset front end architectures.

# 2	Discussion

The proponents of [1] have identified the problem statement of aggregating bands n12 and n29 with the following justifications:

-	We are rich in mid-band spectrum (~300 MHz) which is effective closer to sites, however we have limited low-band spectrum (15 MHz) which propagates farther

-	As our customers move between sites (urban and rural), they’re spending more time in low-band coverage.

-	Low-band carries significant traffic volumes in urban and rural

-	Our customers spend 15% of their day on average in urban low-bands, and 50% in rural low-bands

-	Low-band capacity challenge has a huge impact on customer experience. Users will experience poor data speeds due to low-band congestion.

-	Low-Low band CA is one way of solving this problem, however OEM’s have challenges to support low-low band CA and this ecosystem does not exist

-	B29 is widely deployed in our network and is underutilized

-	Bell and TELUS together has nationwide access to 10 MHz of MBS – DL only (n29) in 700MHz spectrum.

-	Band 29 reaches 83% of Canada’s pop.

The frequency arrangement of this scenario is shown in Figure 1 below.

Figure 1: Aggregation of n12 and n29

Observation 1:	Band n29 falls within the duplex gap of band n12 and leaves 1 MHz for the filter stopband between n12 UL and n29 DL.  With current duplexer technology, a solution based on traditional carrier aggregation architecture does not seem feasible.

During the offline discussions in RAN #104, two additional operators (Anterix and Southern Linc) have identified another LB+LB scenario which carries very similar challenges as above.  The operators have provided the following problem statement:

-	Our issue in the US market is with band 26 and 106 in the 900 MHz range (FDD) and band 48 (TDD).

-	Our low bands provide the coverage layer and continuous connectivity but we cannot currently CA and have a need to do so as we're spectrum limited.

This scenario is illustrated in Figure 2 below.

Figure 2: Aggregation of n26 and n106

Observation 2:	Band n106 UL is separated from band n26 DL by 2 MHz.  With current duplexer technology, a solution based on traditional carrier aggregation architecture does not seem feasible.

Another scenario, suggested by BT, involves the band combination with the supplemental downlink band n67, as shown in Figure 3 below.

Figure 3: Aggregation of n28 and n67

Observation 3:	Band n67 DL overlaps with band n28 UL.  With current duplexer technology, a solution based on traditional carrier aggregation architecture does not seem feasible.

According to the proposed concept in [1], a switching scheme can be a feasible architecture to resolve the three scenarios, as shown in Figure 4 below.

a)  
b)  c)

Figure 4: Switching architectures for a) Scenario 1, b) Scenario 2, c) Scenario 3

An enhancement of the 3GPP specification is needed to enable the following solution:

-	Device needs to support inter-carrier scheduling: UE monitors PCell (n12 in case of Scenario 1, n26/n106 in case of Scenario 2, or n28 in case of Scenario 3) PDCCH DCI, which has scheduling information for both bands applicable to the scenario

-	Device needs to support TTI level switching; when the SCell is scheduled, UE needs to switch to the SCell filter, and during the scheduled period there is no simultaneous Tx/Rx between the PCell and the SCell

-	Device switches back to the PCell duplexer after the SCell scheduled period is finished

-	In the case of Scenario 1, switching between the PCell and the aggregation of DL carriers can also be considered

Considering that 3GPP has already specified the Tx Switching suite of features (with components and enhancements adopted in Rel-16 through Rel-18), it is useful for 3GPP to consider the application of this framework to the listed scenarios.

Proposal 1:	An enhancement of the 3GPP specification is needed to enable LB+LB carrier aggregation based on switching.

Proposal 2:	The following objectives are provided for information and further refinement:

-	Specify UE requirements, including at least switching gap (if needed), to allow switching for supplementary downlink between {case 1, case 2a} [RAN1, RAN4]

-	Case 1: 1 Tx/2 Rx on FDD carrier 1 and 0 Tx/0 Rx on carrier 2

-	Case 2a: 2 Rx on SDL carrier 2 and 0 Tx/0 Rx on carrier 1

-	Specify UE requirements, including at least switching gap (if needed), to allow switching for inter-band CA between {case 1, case 2b} [RAN1, RAN4]

-	Case 1: 1 Tx/2 Rx on FDD carrier 1 and 0 Tx/0 Rx on carrier 2

-	Case 2b: 1 Tx/2 Rx on FDD carrier 2 and 0 Tx/0 Rx on carrier 1

-	Specify UE requirements, including at least switching gap (if needed), to allow switching for inter-band CA between {case 1, case 3} [RAN1, RAN4]

-	Case 1: 1 Tx/2 Rx on FDD carrier 1 and 0 Tx/0 Rx on carrier 2

-	Case 3: 2 Rx on aggregated FDD carrier 1 and SDL carrier 2 and 0 Tx on carrier 1

-	The carrier frequency for all cases is <1 GHz

-	Consider the following deployment constraints:

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	Study the impact on interruption and delay requirements and specify the physical layer procedures and/or RRM requirements if needed [RAN1, RAN4]

-	Strive to reuse the existing Tx Switching framework as much as possible

-	Strive to minimize the RAN1 impact

-	Specify UE RF requirements [RAN4]

-	Define the corresponding UE capabilities [RAN1, RAN4]

# 3	Conclusions

This contribution describes the relevant LB+LB CA scenarios and provides the motivation for a new Rel-19 work item to specify a switching solution to enable their adoption into practical handset front end architectures.  The following observations and proposals are presented:

Observation 1:	Band n29 falls within the duplex gap of band n12 and leaves 1 MHz for the filter stopband between n12 UL and n29 DL.  With current duplexer technology, a solution based on traditional carrier aggregation architecture does not seem feasible.

Observation 2:	Band n106 UL is separated from band n26 DL by 2 MHz.  With current duplexer technology, a solution based on traditional carrier aggregation architecture does not seem feasible.

Observation 3:	Band n67 DL overlaps with band n28 UL.  With current duplexer technology, a solution based on traditional carrier aggregation architecture does not seem feasible.

Proposal 1:	An enhancement of the 3GPP specification is needed to enable LB+LB carrier aggregation based on switching.

Proposal 2:	The following objectives are provided for information and further refinement:

-	Specify UE requirements, including at least switching gap (if needed), to allow switching for supplementary downlink between {case 1, case 2a} [RAN1, RAN4]

-	Case 1: 1 Tx/2 Rx on FDD carrier 1 and 0 Tx/0 Rx on carrier 2

-	Case 2a: 2 Rx on SDL carrier 2 and 0 Tx/0 Rx on carrier 1

-	Specify UE requirements, including at least switching gap (if needed), to allow switching for inter-band CA between {case 1, case 2b} [RAN1, RAN4]

-	Case 1: 1 Tx/2 Rx on FDD carrier 1 and 0 Tx/0 Rx on carrier 2

-	Case 2b: 1 Tx/2 Rx on FDD carrier 2 and 0 Tx/0 Rx on carrier 1

-	Specify UE requirements, including at least switching gap (if needed), to allow switching for inter-band CA between {case 1, case 3} [RAN1, RAN4]

-	Case 1: 1 Tx/2 Rx on FDD carrier 1 and 0 Tx/0 Rx on carrier 2

-	Case 3: 2 Rx on aggregated FDD carrier 1 and SDL carrier 2 and 0 Tx on carrier 1

-	The carrier frequency for all cases is <1 GHz

-	Consider the following deployment constraints:

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	Study the impact on interruption and delay requirements and specify the physical layer procedures and/or RRM requirements if needed [RAN1, RAN4]

-	Strive to reuse the existing Tx Switching framework as much as possible

-	Strive to minimize the RAN1 impact

-	Specify UE RF requirements [RAN4]

-	Define the corresponding UE capabilities [RAN1, RAN4]

# 4	References

RP-241537, “Rx switched nX-n29 low-low carrier aggregation,” TELUS, Bell Mobility, 3GPP RAN #104, June 2024