3GPP TSG-RAN WG4 Meeting #112                                                             R4-2411177

Maastricht, The Netherlands, Aug 19 – Aug 23, 2024

Source:	Korea Testing Laboratory

Title:	Testability and interoperability issues for beam management

Agenda item:			8.17.2

Document for:	Discussion

# 1. Introduction

In this document, we offer a technical view of the AI/ML study conducted by WF on AI/ML [1] and the AI/ML ad-hoc meeting minutes[2]. Specifically, in addressing issue 2-5, we have outlined an efficiency testing approach that takes into account the virtual relationship between Set A and Set B as well as UE rotation. Concerning issue 2-6, we handle the technical aspects of LCM operation reflected on field deployment conditions and  two model identifications and the overfitting problem. Lastly, we summarize that achieving slightly better measurement accuracy than the current requirement can result in performance gains.

2. Discussion

2.1 Testability and interoperability issues for BM

Issue 2-5: Test setup needs [1]

Agreement: - Maximum number of set B Tx beams that test system should be able to emulate: [8-16] - Maximum number of set A Tx beams that test system should be able to emulate:[64-128] - FFS on AoAs - UE rotation during the test: FFS - UE rotation/repositioning between different tests: Yes

In actual deployment situations, Set A and Set B combinations can be changed in accordance with the gNB beamforming scheme. A fixed Set B or a variable Set B with pre-configured patterns was taken into consideration throughout the evaluation stage [2]. For example, the network provides and has the ability to modify the association between Set A and Set B. For the purpose of conformance testing, it is more practical to limit the combinations to just one or some of them because it is difficult to take into account every potential combination between Set A and Set B. Additionally, if we need to verify the performance with the variable Set B, we can check the performance by changing the association information with virtual relation information between them.

Table 1. Configuring Set B [2]

The following options are studied on the selection of Set B of beams (pairs): - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference) - Opt 2A: Set B is changed following a set of pre-configured patterns - Opt 2B: Set B is randomly changed among pre-configured patterns - Opt 2C: Set B is randomly changed among Set A beams (pairs) - Opt 2D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C - The number of beams(pairs) in Set B can be fixed or variable - Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B) - Note: BM-Case1 and BM-Case2 may be considered for different option. - Note: This does not preclude the alternative that Set B is different from Set A. For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), study the following options as AI/ML model inputs:

Observation 1: It costs a lot to consider all possible combinations between Set A and Set B for the conformance testing.

Proposal 1:  For the conformance testing, study and specify the reduced or a single combination(s)  between Set A and Set B.

In the evaluation phase [3], for BM-Case2, AI/ML shows the performance gain without and with UE rotation compared to the non-AI baseline, according to [1]. When considering UE rotation, it evaluated the speed of UE rotation and all three rotational axes under this modelling, with the rotational direction chosen uniformly at random among the three axes.

It could be practical and feasible to accept the existing FR2 OTA chamber for BM-Case2 of the AI-BM test if there is no effect on the inference running for the rotational direction. In the other case, we need to study whether or not the inference algorithm can be affected by the rotational direction of UE. So far, the baseline measurement setup of RRM characteristics does not support the rotation direction on uniform distribution movement among the three axes. Technically, the rotation direction is processed as the preconfigured sequential direction, as captured in Figure 2[3].

Figure 1 : Baseline measurement setup of RRM characteristics (from Fig. 6.2.1.1-1 [3])

##### 6.3.2.2.1 Performance when Set A = Set B For BM-Case2, when Set B = Set A, for DL Tx beam prediction with the measurements from the best Rx beam or Tx-Rx beam pair prediction, without considering generalization aspects, with the following assumptions: - UE speed: 30km/h (unless otherwise stated) - Prediction time: 80ms/160ms/320ms/640ms/800ms/others - With UE rotation and without UE rotation - Set B is the same as Set A in each time instance for measurement ~ sentences skipped ~ ##### 6.3.2.2.2 Performance when Set B is a subset of Set A For BM-Case2, when Set B patten is a subset of Set A in each time instance, for DL Tx beam prediction with the measurements from the best Rx beam or Tx-Rx beam pair prediction, without considering generalization aspects, with the following assumptions: - UE speed: 30km/h (unless otherwise stated) - Prediction time: 40ms/80ms/160ms/320ms/640ms/others - With and without UE rotation - Fixed Set B patterns or preconfigured Set B pattens in each measurement instances (unless otherwise stated) ~ sentences skipped ~ #### 6.3.2.3 Performance under different assumptions/scenarios for BM-Case1 and/or BM-Case2 (B) For Tx DL beam prediction with UE rotation, ~ sentences skipped ~ - Wherein, UE rotation is modelled every 40ms with constant 10 RPM rotation speed in all three rotational axes, with rotational direction chosen uniformly at random among the three axes. ~ sentences skipped [109 page] ~ (B) For Tx DL beam prediction, based on the evaluation from 2 sources, AI/ML can provide some beam prediction accuracy gain comparing with non-AI baseline (Option 2, sample-and-hold) with UE rotation and the performance of AI/ML compared to baseline (Option 2, sample-and-hold) improves with the increase of measurement periodicity: ~ sentences skipped ~ - In the evaluation, UE rotation is modelled every 40ms with constant 10 RPM rotation speed in all three rotational axes, with rotational direction chosen uniformly at random among the three axes.

In the evaluation [2], the UE direction is captured for BM-Case2 where both scenarios of Set A= Set B and Set B is a subset of Set A are evaluated under with UE rotation and without UE rotation.

Observation 2: In the evaluation, UE rotation is modelled for BM-Case2 with a rotation speed in all three rotational axes, with the rotational direction chosen uniformly at random among the three axes.

Proposal 2:  The following issues should be considered

- Study on the testing BM-Case2 whether or not the inference algorithm is independent of the rotational direction of UE.

  - Study the test setup on the existing FR2 OTA setup as baseline.

  - Otherwise, study a test environment considering the rotational direction of UE

Issue 2-6 : Datasets for training/testing [1]

Agreement: - Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - Sufficient test environment/conditions should be defined to enable vendors to create the data needed for training - FFS on proposals to augment training data to avoid overfitting of UE models to the test environment - RAN4 to strive to make the test conditions similar to field deployment conditions

In [3], there are two kinds of AI/ML model identification: AI/ML functionality identification and AI/ML model ID identification.  For AI/ML functionality identification, we shall consider several AI/ML models to support the specific functionality meaning that there could be multiple AI models in one UE each corresponding to specific network conditions, and configurations.  In other words, the UE can select, switch and activate the appropriate AI/ML model depending on the test environment/conditions based on the LCM. For AI/ML model ID identification, the exact AI/ML model is identified at the network and the UE. That identified AI/ML model can be tested under the test environment/conditions based on the LCM. If UE supports both model ID identifications, it is required the process to configure which model ID identification is setup between TE and UE for the test. When using the model ID identification setup, it is quite straightforward for us to verify which model is currently being tested, given the precondition that the model should not experience fallback or deactivation. In case of the AI/ML functionality identification setup, it is possible to check which AI/ML functionality is tested under the test case ensuring the model remains active and does not encounter fallback or deactivation. The last bullet of “RAN4 to strive to make the test conditions similar to field deployment conditions” shall be achieved as much as possible however RAN4 shall consider whether or not the LCM performance monitoring function shall be involved since the field deployment condition reflects the LCM function.

Observation 3: Two kinds of AI/ML model identification shall be considered for test setup.

Observation 4: The field deployment condition includes the LCM function.

Proposal 3:  RAN4 shall consider whether or not the LCM performance monitoring function shall be involved since the field deployment condition reflects the LCM.

Overfitting is a common problem in machine learning (ML) models, where the model performs well on training data but poorly on new, unseen data. To prevent overfitting, there are various methods that can be used, such as early stopping, data augmentation, regularization, and modifying the ML model with techniques like adding dropout layers or batch normalization. From the RAN4 perspective, it is emphasized that data augmentation is crucial for controlling test environments, while other approaches should be considered by UE vendors.  Required efforts are made to configure models to reflect real-field environments for AI/ML use cases, however, it's important to note that in the conformance testing, RAN4 sets minimum requirements for each test case. While data augmentation is a powerful tool for mitigating overfitting, it may not completely prevent the issue in all cases when solely relied upon. Consequently, it is necessary to validate whether test configurations, especially those considering data augmentation, can effectively prevent overfitting. If there is uncertainty, other approaches should be taken into account.

It's important to recognize that an AI/ML model could exhibit poor performance in a field deployment condition due to overfitting and/or data drift, among other factors. While the test environment may have been designed to mitigate overfitting, it can still be challenging to completely avoid overfitting (local minima) in field deployment conditions.

Observation 5: Required efforts are made to configure models to reflect real-field environments for AI/ML use cases, however, it's important to note that in the conformance testing, RAN4 sets minimum requirements for each test case.

Observation 6: While data augmentation is a powerful tool for mitigating overfitting, it may not completely prevent the issue in all cases when solely relied upon.

Proposal 4:  It is necessary to validate whether test configurations, especially those considering data augmentation, can effectively prevent overfitting. If there is uncertainty, other approaches should be taken into account.

Issue 2-8 : Measurement error impact [2]

- Multiple companies brought up the need to evaluate the impact that the UE measurement error and the error in the training data have on inference accuracy - Proposals - Option 1: RAN4 should study the impact of measurement error, companies should bring proposals for the next meeting on how to proceed with such a study - Option 2: RAN4 should postpone the discussion on the impact of the measurement error for now - Option 3: No need to evaluate the measurement error impact - Recommended WF - To be discussed

In [4], there are issue #2 regarding on whether/how to address the measurement error as follows;

Table 2 FL summary #0 for AI/ML in beam management [4]

#### Issue #2: Whether/how to address Measurement error - Ericsson [2] The number of samples and statistical metrics of the performance metrics needs to be addressed. - Intel [4] RAN1 should further discuss if one-shot L1 measurements are used for set B beams or if averaging of L1 measurements over time is needed. - GOOGLE [23] Support SSB/CSI-RS repetition to improve the measurement accuracy for SD beam prediction. Support SSB/CSI-RS repetition to improve the measurement accuracy for temporal beam prediction. - OPPO [9] For temporal domain beam prediction, suggest to study and evaluate the beam dwelling time prediction. - Fujitsu [20] Regarding training data collection, repetition of the reference signals could be considered to improve the measurement accuracy and the same UE Rx beam should be maintained during the measurement. - DoCoMo [32] Discuss how to handle measurement sensitivity issue in the measurements of Set B/C.

From the RAN4 perspective, both approaches can be implemented concurrently to address the measurement error issue. The first approach involves awaiting RAN1's discussion, while the second approach involves exploring the possibility of tightening requirements on L1-RSRP measurement accuracy.

As per Table 2 and Table 3, the measurement error significantly impacts the Top-1 beam prediction accuracy. As indicated in reference [3], it is important to notice that the presence of measurement error has a detrimental impact on both the efficacy of traditional beam management and the performance of AI/ML beam prediction. Therefore we can expect a performance gain for both of them if we can define a higher of accuracy than the current accuracy requirement. Also, the effectiveness of the first approach, using repetition or averaging, can be ensured from a higher level of accuracy than the current FR2’s L1-RSRP accuracy. It may be beneficial to consider using FR1’s accuracy of L1-RSRP measurement requirement as the upper bound for FR2’s requirement.

Table 3 For BM-Case 1 DL Tx beam prediction, without differentiating BB errors and RF errors [3]

| Measurement error [dB] | number of source company | For BM-Case 1 DL TX beam prediction accuracy degradation in terms of Top-1 beam prediction accuracy compared to the one without measurement error. | Note Set B is a subset(1/4 unless otherwise stated) of Set A |
| --- | --- | --- | --- |
| ± 2 | 3 | 6 %~10 % | 1 source : 95%ile of L1-RSRP diff can be about 1.4~2dB, 1 source : that average L1-RSRP diff can be lower than 1dB |
|  | 1 | 28.80% | average L1-RSRP diff can be about 7.3 dB |
|  |  | 2.40% | For TX-Rx beam pair prediction when Set B is 1/8 of Set A average L1-RSRP diff can be about 5.8 dB |
| ±3 or ±4 | 4 | 14 % (3 dB error) ~ 20 % ( 4dB error) |  |
| ±5 | 1 | 13.60% | when Set B is 1/8 of Set A |
| ±6 | 3 | 22 % ~ 30 % | the 95%ile of L1-RSRP diff can be about 3.1~7.5 dB |
|  |  |  | 1 source : the L1-RSRP diff in 90%ile degrade 7dB, compared to baseline 1 and 2 that degrades 3 dB respectively 1 dB ate the same percentile |
|  | 1 | 42 % ~ 48 % | for both DL Tx beam prediction and beam pair prediction the average L1-RSRP diff can be about 1.6 dB |

Table 4 For BM-Case 1 DL Tx beam prediction, with separately BB errors and RF errors [3]

| Measurement error [dB] | number of source company | For BM-Case 1 DL TX beam prediction accuracy degradation in terms of Top-1 beam prediction accuracy comparing to the one without measurement error. | Note Set B is a subset(1/4 unless otherwise stated) of Set A |
| --- | --- | --- | --- |
| ±3 for BB and RF respectively | 1 | 42 %, 38 % | average L1-RSRP diff is about 1.1 dB and 2.16 dB respectively |
|  |  | 4.3 %, 6.3 % | Set B is of Set A and beam pair prediction with Set B is 1/16 Set A average of L1-RSRP diff becomes 0.7 dB and 2.18 dB larger respectively |

Observation 7: The measurement error degrades the performance of the legacy and also AI/ML beam management.

Proposal 8:  Study the possibility of enhancing the area to have a higher level of accuracy than the current FR2’s L1-RSRP accuracy.

3. Conclusion

In this contribution, we provided our viewpoints on the testability and interoperability for beam management.

Observation 1: It costs a lot to consider all possible combinations between Set A and Set B for the conformance testing.

Proposal 1:  For the conformance testing, study and specify the reduced or a single combination(s)  between Set A and Set B.

Observation 2: In the evaluation, UE rotation is modelled for BM-Case2 with a rotation speed in all three rotational axes, with the rotational direction chosen uniformly at random among the three axes.

Proposal 2:  The following issues should be considered

- Study on the testing BM-Case2 whether or not the inference algorithm is independent of the rotational direction of UE.

  - Study the test setup on the existing FR2 OTA setup as baseline.

  - Otherwise, study a test environment considering the rotational direction of UE

Observation 3: Two kinds of AI/ML model identification shall be considered for test setup.

Observation 4: The field deployment condition includes the LCM function.

Proposal 3:  RAN4 shall consider whether or not the LCM performance monitoring function shall be involved since the field deployment condition reflects the LCM.

Observation 5: Required efforts are made to configure models to reflect real-field environments for AI/ML use cases, however, it's important to note that in the conformance testing, RAN4 sets minimum requirements for each test case.

Observation 6: While data augmentation is a powerful tool for mitigating overfitting, it may not completely prevent the issue in all cases when solely relied upon.

Proposal 4:  It is necessary to validate whether test configurations, especially those considering data augmentation, can effectively prevent overfitting. If there is uncertainty, other approaches should be taken into account.

Observation 7: The measurement error degrades the performance of the legacy and also AI/ML beam management.

Proposal 8:  Study the possibility of enhancing the area to have a higher level of accuracy than the current FR2’s L1-RSRP accuracy.

4. Reference

[1] R4-2410570, “WF on AI/ML”, Qualcomm

[2] R4-2410571, “AI/ML ad-hoc meeting minutes”, Qualcomm

[3] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[4] R1-2405427, “FL summary #0 for AI/ML in beam management”, Fukuoka, Japan, RAN#117