3GPP TSG-RAN WG4 Meeting #112                                                             R4-2411178

Maastricht, The Netherlands, Aug 19 – Aug 23, 2024

Source:	Korea Testing Laboratory

Title:	Testability and interoperability issues for CSI compression and CSI prediction

Agenda item:			8.17.4

Document for:	Discussion

# 1. Introduction

The issues 4-1 of “WF on AI/ML” are captured from the previous RAN4 #111 meeting. Regarding the huge AI/ML related data, the AI/ML Logistical issues were discussed in RAN #104, which will be discussed further in RAN4.  In this paper, we will share the reason to be documented (in TR, WF, Electrical files etc) in the form of specification format regarding the Reference encoder and Reference decoder.

Table 1. AI/ML ad-hoc meeting minutes [1]

Issue 4-1: Reference encoder/decoder Agreement: - Reference encoder: - The reference encoder is used in RAN4 discussions at least for simulation alignment/requirement derivation, test decoder derivation and/or test decoder verification. It could be documented (in TR, WF, etc) or captured in the specifications as necessary. - Reference decoder: - The reference decoder is used in RAN4 discussions at least for simulation alignment/requirement derivation and/or verification of the decoder implemented by the TE. It could be documented (in TR, WF, etc) or captured in the specifications as necessary. - For option 3, for each test, a test decoder needs to be captured in the specs, a reference encoder might be needed to derive the test decoder and/or requirements and/or to validate the test decoder implementation in the TE. the same decoder might be used in multiple tests or each test could have a difference decoder. - In option 3 the Reference decoder is the test decoder. - For option 4, there might be a need to have reference encoder and/or reference decoder

2. Discussion

2.1 ISO/IEC 17025:2017 Requirements

ISO/IEC 17025 is the international standard for testing and calibration laboratories. It's important to follow the requirements for Proficiency Testing (PT) and Inter-Laboratory Comparison (ILC) within the ISO/IEC 17025. This ensures that testing and calibration laboratories adhere to the relevant standards and guidelines for quality assurance and accuracy in their testing processes. According to ISO/IEC 17025, it is essential to ensure several critical elements in the AI/ML test environment, similar to other areas of conformance testing. The first requirement pertains to the interpretability or explainability of machine learning models, which is essential for debugging and fixing incorrect predictions or errors made by the model in all AI/ML use cases. The second requirement relates to the documentation and traceability needed to ensure reliable and consistent test environments, which are critical for compliance with ISO/IEC 17025.

These are,

- Explainable AI/ML: Implement techniques that make the AI/ML model’s decisions interpretable, which helps understand and verify the model’s outputs, ensuring they align with expected standards.

- Documentation and Traceability: Maintain thorough documentation of the training, validation, and testing for AI/ML model, which can ensure transparency and traceability, which are critical for ISO/IEC 17025 compliance.

It would be for long benefit for RAN4 to consider the detail parameters for the reference encoder and reference decoder providing “Explainability” and “Documentation and Traceability” as much as possible. For sure, it's important to note that for legacy test cases such as LTE, 3G and 5G, the test specifications do provide for Proficiency Testing (PT) and Inter-Laboratory Comparison (ILC) where the highest technical competence is met meaning that the Explainability, Documentation and Traceability of test environments. Therefore, like any other test cases, the tests for AI/ML use cases must deliver the identical high quality the PT and ILC that legacy tests provide.

Table 3 ISO/IEC 17025:2017 Requirements

3 Terms and definitions 3.3 interlaboratory comparison Organization, performance and evaluation of measurements or tests on the same or similar items by two or more laboratories in accordance with predetermined conditions [SOURCE:ISO/IEC 17043:2020, 3.4] 3.4 intralaboratory comparison Organization, performance and evaluation of measurements or tests on the same or similar items within the same laboratory (3.6) in accordance with predetermined conditions 3.5 proficiency testing evaluation of participant performance against pre-established criteria by means of interlaboratory comparisons (3.3) [SOURCE: ISO/IEC 17043:2010, 3.7, modified — Notes to entry have been deleted. 7.7 Ensuring the validity of results 7.7.1 The laboratory shall have a procedure for monitoring the validity of results. The resulting data shall be recorded in such a way that trends are detectable and, where practicable, statistical techniques shall be applied to review that results. This monitoring shall be planned and reviewed and shall include, where appropriate, but not be limited to: a) use of reference materials or quality control materials b) use of alternative instrumentation that has been calibrated to provide traceable results; c) functional check(s) of measuring and testing equipment; d) use of check or working standards with control charts, where applicable; e) intermediate checks on measuring equipment; f) replicate tests or calibrations using the same or different methods; g) retesting or recalibration of retained items; h) correlation of results for different characteristics of an item; i) review of reported results; j) intralaboratory comparisons; k) testing of blind sample(s); 7.7.2 The laboratory shall monitor its performance by comparison with results of other laboratories, where available and appropriate. This monitoring shall be planned and reviewed and shall include, but not be limited to, either or both of the following: a) participation in proficiency testing; NOTE ISO/IEC 17043 contains additional information on proficiency tests and proficiency testing providers. Proficiency testing providers that meet the requirements of ISO/IEC 17043 are considered to be competent. b) participation in interlaboratory comparisons other than proficiency testing

Observation 1: The interpretability, explainability, documentation, and traceability provided by legacy test cases should also be applied to AI/ML use cases, in accordance with the proficiency testing and interlaboratory comparison requirements outlined in ISO/IEC 17025.

Proposal 1:  The detail parameters for Reference encoder and Reference decoder shall be specified (in TR, WF, etc) or captured in the specifications. (Fully standardized reference model (structure + parameters))

2.1 Testability and interoperability for two-sided model

In RAN1#117 (May 2024), the following agreements were achieved on MI-Option2 for the two-sided CSI compression. For the reference encoder and reference decoder, a dataset needs to consider the various kind of environmental scenario since CSI is highly dependent on the propagation environment. It means TE transfer a dataset predefined by RAN4 to UE. With that reason, a dataset shall be specified (in TR, WF, etc) or captured in the specifications.

From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset

Observation 2: A dataset needs to consider the various kinds of environmental scenarios since CSI is highly dependent on the propagation environment.

Proposal 2:  For the conformance testing, a dataset shall be specified (in TR, WF, etc) or captured in the specifications.

3. Conclusion

In this contribution, we provided our viewpoints on the testability and interoperability for for CSI compression and CSI prediction.

Observation 1: The interpretability, explainability, documentation, and traceability provided by legacy test cases should also be applied to AI/ML use cases, in accordance with the proficiency testing and interlaboratory comparison requirements outlined in ISO/IEC 17025.

Observation 2: A dataset needs to consider the various kinds of environmental scenarios since CSI is highly dependent on the propagation environment.

Proposal 1:  The detail parameters for Reference encoder and Reference decoder shall be specified (in TR, WF, etc) or captured in the specifications. (Fully standardized reference model (structure + parameters))

Proposal 2:  For the conformance testing, a dataset shall be specified (in TR, WF, etc) or captured in the specifications.

4. Reference

[1] R4-2401571, “AI/ML ad-hoc meeting minutes”, Qualcomm

[2] ISO/IEC 17025:2017