3GPP TSG-RAN WG4 Meeting #112                         R4-2411183

Maastricht, NL, 19-23 Aug 2024

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda Item:	8.16.2

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved in [1].

The following objectives are approved for UE with 8Rx:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

RAN4#112 is the first meeting for the discussion on the above objectives. And our initial views are given in this contribution.

Discussion

In the following table, we have summarized the key parameters used for MMSE-IRC requirements defined for 2/4Rx UE in Rel-17:

Table 1. Key parameters used for Rel-17 MMSE-IRC requirements for 2/4Rx UE

- PDCSH requirements with inter cell interference - Receiver type: - MMSE-IRC - Network type: - Synchronized network only - Cover both HomNet and Hetnet scenarios with the following applicability rule [2] - ‘If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode’ - Interference profile: INR based inter cell interference modeling with the following interference level: - INR1 = 7.77dB and INR2 = 2.29dB for HomNet - INR1 = 7.58dB for HetNet - Random rank with 70% and 30% probability for rank 1 and rank 2 for the interference cell(s) - Test and PDSCH configuration - TDLC300-100, MCS 13, 2x2 ULA Low, rank1 for HomNet - TDLA-30-10, MCS 13, 2x2 ULA Low, rank1 for HetNet - PDCSH requirements with intra cell inter user interference - Receiver type: - MMSE-IRC and R-ML - 1 co-scheduled UE with random 16QAM symbols for MMSE-IRC test - Rank configuration: - Both rank 1+1 and rank 2+2 are covered - Precoding method: - Target UE: Random precoding - Co-scheduled UE: - Random precoding for MMSE-IRC tests with rank 1+1; - Orthogonal precoding with the target UE for MMSE-IRC tests with rank 2+2 and for R-ML tests - Test and PDSCH configuration for the target UE - TDLC300-100, MCS 13, 2x2 and 2x4 ULA Low for MMSE-IRC tests with rank 1+1 - TDLA30-10, MCS 13, 4x4 ULA Low for MMSE-IRC tests with rank 2+2 - DMRS port allocation between target and co-scheduled UEs: - Same DMRS group for rank 1+1 test, i.e., {1000} for target and {1001} for co-UE - Different DMRS groups for rank 2+2 test, i.e., {1000, 1001} for target and {1002, 1003} for co-UE - Wideband CQI reporting requirements with inter-cell interference - Receiver type: - MMSE-IRC - Test criteria: - the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ ; - when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02. - Configuration for the target cell: - TDLA30-5, 2x2 and 2x4 ULA Low - Configuration for the target cell: - AWGN 1x2 and 1x4, INR = 10.04dB - Tested SINR = -2dB

In general, we think most of the test settings and parameters can be reused for 8Rx UEs. At the same time, we would also like to share our views on the following parameters.

General

Test scope

For the test scope for 8Rx UE, PDSCH absolute physical layer throughput requirements are also mandatory requirements for 2/4Rx which is not covered in Rel-18 or the current objectives in the Rel-19 WID.

As there is still a check point in RAN#106 for potential scope expansion for RAN4-led WIs, we would like to bring this issue in the RAN4 meeting to see if companies are ok to also cover PDSCH absolute physical layer throughput requirements for 8Rx.

Proposal 1: RAN4 to discuss whether to cover PDSCH absolute physical layer throughput requirements for 8Rx.

Receiver assumptions

In Rel-18, 2 types of 8Rx receivers are defined in TS38.306:

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

Different implementations of 8Rx receivers have resulted in significant performance difference, e.g., the performance gap between Baseline SU-MIMO 8Rx receiver and simplified SU-MIMO 8Rx receiver is 2.2dB for FDD rank 2 requirements.

For requirement definition for this WI, it is proposed to define requirements for both 8Rx receiver types to ensure that all 8Rx UEs can be tested with a reasonable requirement value.

Proposal 2: UE performance requirements should cover both Baseline SU-MIMO 8Rx receiver and Simplified SU-MIMO 8Rx receiver.

Test applicability rule and release independent

MMSE-IRC is a mandatory without capability signaling feature for 2/4Rx UEs from Rel-17. As a new feature for Rel-18, we believe all 8Rx UEs should at least implement MMSE-IRC receiver. Therefore, the new requirements for 8Rx MMSE-IRC receiver should also be mandatory requirements for all 8Rx UEs without additional test applicability rule.

In addition, the 8Rx UE feature is introduced in Rel-18 and we have already discussed the possibility to cover MMSE-IRC requirements during the performance part discussion under WI ‘NR_ENDC_ RF_FR1_enh2’. Such requirements are not introduced in Rel-18 mainly because of the limited time and huge workload. Therefore, we think the new requirements for 8Rx should be release independent from Rel-18.

Proposal 3: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule.

Proposal 4: The new requirements for 8Rx should be release independent from Rel-18.

PDCSH requirements with inter cell interference

Interference profile

As this WI targets to define requirements for 8Rx CPE/FWA/vehicle/industrial devices, the actual interference power level may be different from the values we have agreed for Rel-17 handheld UEs due to changing of deployment scenarios.

However, since the purpose of these requirements targets to verify the MMSE-IRC processing itself, we are also ok to reuse the existing INR values for Rel-17 2/4Rx to avoid extra system level simulation work.

Proposal 5: Reuse the same interference power levels (i.e., INR values) defined for Rel-17 2/4Rx for 8Rx requirements with inter cell interference.

Antenna configuration and rank

The existing requirements with ICI for 4Rx rank1 are around 10dB for TDLC300-100 and 7dB for TDLA30-10.

By increasing the Rx number from 4 to 8, around 3dB performance gain is expected due to pure diversity gain. In addition, more performance gain of the MMSE-IRC receiver over the baseline receiver can be further expected due to more spatial domain freedom. For example, the requirements for MMSE-IRC with 4Rx has reduced to 7.4dB from 12.5dB with 2Rx.

In that case, under the same test configuration with 2/4Rx, the SNR requirement value for 8Rx may be reduced to around 2-3 dB, which is not a common SNR requirement for 8Rx.

Observation 1: Under the same test configuration with 2/4Rx, the SNR requirement value for 8Rx may be reduced to around 2-3dB due to larger diversity gain and more MMSE-IRC performance gain.

Considering the above, it is proposed to investigate simulation results for rank 1 (2x8), 2 (2x8) and 4 (4x8) and choose a reasonable requirement value.

Proposal 6: Investigate simulation results for rank 1 (2x8), 2 (2x8) and 4 (4x8) and choose a reasonable requirement value.

PDCSH requirements with intra cell inter user interference

Receiver Type

From the UE capability perspective, currently there is no capability for 8Rx UE to support R-ML receiver. Therefore, it is impossible for a Rel-18 UE supporting 8Rx to test R-ML receiver tests.

Observation 2: Currently there is no capability for 8Rx UE to support R-ML receiver.

advReceiver-MU-MIMO-r18 Indicates whether the UE supports R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2RX and 4RX antennas, when co-scheduled UE(s)' modulation order is explicitly signalled by DCI index 1-5 in Table 7.3.1.2.2-12 of TS 38.212 [10]. NOTE: UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA, NE-DC, NR-DC and EN-DC operation. A UE supporting this feature shall also support SU-MIMO Interference Mitigation advanced receiver in clause 5.; UE; No; No; FR1 only.

As for the receiver type for PDCSH requirements with intra cell inter user interference for 8Rx UE, we propose to consider MMSE-IRC.

At the same time, we would also like to check the possibility for 8Rx UE to support R-ML receiver for SU/MU-MIMO scenario. If so, we can consider introduce new UE capabilities in Rel-19 and have it tested.

Proposal 7: Consider MMSE-IRC for the receiver type for PDCSH requirements with intra cell inter user interference for 8Rx UE.

Proposal 8: RAN4 to check the possibility for 8Rx UE to support R-ML receiver for SU/MU-MIMO scenario. If so, new UE capabilities can be introduced in Rel-19 and have it tested.

Precoding method for the co-scheduled UE

For 2/4Rx MMSE-IRC tests, random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests. Same time, orthogonal precoding was used for all R-ML tests.

In our understanding, RAN4 has chosen to use only orthogonal precoding for R-ML tests because the test configurations for R-ML is stricter in terms of MCS and antenna correlation. Higher modulation order or higher antenna correlation makes it hard for the network to use random precoding for co-UE anymore.

Therefore, if we consider using the same test setting and PDSCH configuration as MMSE-IRC, we propose to reuse the same precoding method as MMSE-IRC, i.e., random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests.

Proposal 9: Reuse the same precoding method as MMSE-IRC, i.e., random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests, if we consider using the same test setting and PDSCH configuration as MMSE-IRC.

Wideband CQI reporting requirements with inter-cell interference

Tested SINR value

In Rel-17, the tested SINR value is chosen based on the simulation results with reasonable performance gain. Therefore, we propose to decide the detailed SINR value based on simulation results.

Proposal 10: The tested SINR value should be decided based on simulation results with reasonable performance gain.

Besides the above proposal, for the other parameters for 8Rx UE new requirements, we propose to reuse the same configuration for Rel-17 2/4Rx as summarized in Table 1.

Proposal 11: For the other parameters for 8Rx UE new requirements, we propose to reuse the same configuration for Rel-17 2/4Rx as summarized in Table 1.

Conclusion

Proposal 1: RAN4 to discuss whether to cover PDSCH absolute physical layer throughput requirements for 8Rx.

Proposal 2: UE performance requirements under this WI should cover both Baseline SU-MIMO 8Rx receiver and Simplified SU-MIMO 8Rx receiver.

Proposal 3: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule.

Proposal 4: The new requirements for 8Rx should be release independent from Rel-18.

Proposal 5: Reuse the same interference power levels (i.e., INR values) defined for Rel-17 2/4Rx for 8Rx requirements with inter cell interference.

Observation 1: Under the same test configuration with 2/4Rx, the SNR requirement value for 8Rx may be reduced to around 2-3dB due to larger diversity gain and more MMSE-IRC performance gain.

Proposal 6: Investigate simulation results for rank 1 (2x8), 2 (2x8) and 4 (4x8) and choose a reasonable requirement value.

Observation 2: Currently there is no capability for 8Rx UE to support R-ML receiver.

Proposal 7: Consider MMSE-IRC for the receiver type for PDCSH requirements with intra cell inter user interference for 8Rx UE.

Proposal 8: RAN4 to check the possibility for 8Rx UE to support R-ML receiver for SU/MU-MIMO scenario. If so, new UE capabilities can be introduced in Rel-19 and have it tested.

Proposal 9: Reuse the same precoding method as MMSE-IRC, i.e., random precoding was used with rank 1+1 and orthogonal precoding was used for rank 2+2 tests, if we consider using the same test setting and PDSCH configuration as MMSE-IRC.

Proposal 10: The tested SINR value should be decided based on simulation results with reasonable performance gain.

Proposal 11: For the other parameters for 8Rx UE new requirements, we propose to reuse the same configuration for Rel-17 2/4Rx as summarized in Table 1.

Reference

RP-241297, Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO, RAN #104, June 2024.

R4-2120707, WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021