3GPP TSG-RAN WG4 Meeting #112                         R4-2411184

Maastricht, NL, 19-23 Aug 2024

Source:	China Telecom

Title:	Discussion on BS demodulation performance requirements for MMSE-IRC

Agenda Item:	8.16.3

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved in [1].

The following objectives are approved for BS requirements:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions:

  - Receiver type: MMSE-IRC receiver

  - Scenarios:

    - FDD synchronous deployments

    - TDD synchronous deployments with aligned UL:DL configurations among cells

    - Slot-based transmission and aligned SCS among cells

  - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

RAN4#112 is the first meeting for the discussion on the above objectives. And our initial views are given in this contribution.

Discussion

Receiver assumption

As the agreed receiver type, it is proposed to use the same receiver assumption for MMSE-IRC in TR36.884 as below:

The MMSE-IRC receiver weight matrix is usually defined as where and denote the estimated channel matrix and covariance matrix, respectively. To obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. The covariance matrix is estimated at DM-RS REs by following equations: , , where the DM-RS symbols are used to estimate the covariance matrix, and Nsp is the number of sampling DM-RS REs. The estimation of interference covariance matrix is performed at per PRB and per TTI basis.

Proposal 1: Reuse the same receiver assumption for MMSE-IRC in TR36.884.

Test scope

The BS MMSE-IRC feature is a baseband feature which should be applicable for all SCS, TDD patterns, CHBW, PUSCH mapping type if supported.

Therefore, as for the test scope, it is proposed to align with the same test scope for Rel-15 PUSCH requirements with transform precoding disabled:

- Cover the following SCS and TDD patterns:

- 15kHz SCS with TDD pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS with TDD pattern 7D1S2U, S=6D:4G:4U

- Cover both PUSCH mapping Type A and Type B.

- Channel bandwidth:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz channel bandwidth

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz channel bandwidth

Proposal 2: Align with the same test scope for Rel-15 PUSCH requirements with transform precoding disabled:

- Cover the following SCS and TDD patterns:

- 15kHz SCS with TDD pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS with TDD pattern 7D1S2U, S=6D:4G:4U

- Cover both PUSCH mapping Type A and Type B.

- Channel bandwidth:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz channel bandwidth

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz channel bandwidth

Test scenarios

The LTE BS MMSE-IRC test requirements have covered both HomNet and HetNet scenarios. In our understanding, both scenarios are also typical scenarios for 5G NR deployments and thus we propose to also cover HomNet and HetNet scenarios for 5G BS MMSE-IRC test requirements.

Proposal 3: Cover HomNet and HetNet scenarios for 5G BS MMSE-IRC test requirements.

In addition, in our field test with different TDD UL/DL configurations between neighboring cells (celled dynamic frame structure in our field tests), heavy gNB-to-gNB co-channel CLI is observed, which can also be mitigated by BS MMSE-IRC. Our field test shows that without MMSE-IRC processing, the UL TP of the victim cell with DSUUU pattern has decreased by 37% when the aggressor gNB with 7D3U is on.

Although we have agreed to use aligned TDD pattern among cells, since we are not excluding other interference profiles for requirement definition in [1], we would still like to propose this possible scenario in our deployment and discuss how to model the corresponding interference profile.

Proposal 4: Consider the following scenario for BS MMSE-IRC requirement definition and discuss how to model the corresponding interference profile:

- Serving cell: DSUUU

- Neighbour cell: 7D3U

Interference modeling

As agreed in the WID, we are generally fine to reuse the same interference modeling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbor cell PUSCH. And to avoid duplicate burden for new system level simulation, we propose to also reuse the same interference power level as in LTE, i.e., DIP -1.11dB and -10.91dB for HomNet scenario and DIP -0.43dB and -13.78dB for HetNet scenario.

Same time, as Rel-17 UE MMSE-IRC requirement definition has agreed to use INR-based modeling + SNR-based requirement definition criteria. It is also proposed to use the same approach for the BS side and transfer the DIP value into INR value.

Proposal 5: Reuse the same interference modeling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbor cell PUSCH.

Proposal 6: Reuse the same interference power level as in LTE, i.e., DIP -1.11dB and -10.91dB for HomNet scenario and DIP -0.43dB and -13.78dB for HetNet scenario.

Proposal 7: Use INR-based modeling + SNR-based requirement definition criteria and transfer the DIP value into INR value.

Antenna configuration

The LTE BS MMSE-IRC test requirements have covered 2/4/8 Rx with 1Tx. Based on the test purpose to verify the BS MMSE-IRC processing itself, we propose to reuse the same antenna configuration.

Proposal 8: Cover 2/4/8 Rx with 1Tx.

Channel condition

The LTE BS MMSE-IRC test requirements use EPA 5 and EVA 70 channel model for the serving cell for HetNet and HomNet respectively. And ETU channel model is used for the interference cell(s).

Number of TX antennas (Note 1); Number of RX antennas (Note 1); Propagation conditions and correlation matrix (Annex B) (Note 2); DIP set. Tested signal; Interferer 1; Interferer 2. 1; 2; EPA 5 Low; ETU 5 Low; N/A; Set 2. EVA 70 Low; ETU 70 Low; N/A; Set 1*. 4; EPA 5 Low; ETU 5 Low; ETU 5 Low; Set 2. EVA 70 Low; ETU 70 Low; ETU 70 Low; Set 1*. 8; EPA 5 Low; ETU 5 Low; ETU 5 Low; Set 2. EVA 70 Low; ETU 70 Low; ETU 70 Low; Set 1*.

Since we have proposed to reuse the same interference power level (i.e., DIPs) for the interference cell, we should also use channel models with similar delay spread and Doppler frequency. Therefore, we propose to use TDLA30-10 and TDLC300-100 propagation channel model for the serving cell for HetNet and HomNet respectively.

As for the nonboring cell, currently TS38.104 has not yet modeled a propagation channel with delay spread as large as ETU model (i.e., 991ns) and with maximum excess tap delay as 5000ns. It is proposed to introduce new propagation channel for testing with desired delay spread 1000ns, i.e., TDLC1000.

Proposal 9: Use TDLA30-10 and TDLC300-100 propagation channel model for the serving cell for HetNet and HomNet respectively.

Proposal 10: Introduce new propagation channel for testing with desired delay spread 1000ns for the nonboring cell, i.e., TDLC1000.

Conclusion

Proposal 1: Reuse the same receiver assumption for MMSE-IRC in TR36.884.

Proposal 2: Align with the same test scope for Rel-15 PUSCH requirements with transform precoding disabled:

- Cover the following SCS and TDD patterns:

- 15kHz SCS with TDD pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS with TDD pattern 7D1S2U, S=6D:4G:4U

- Cover both PUSCH mapping Type A and Type B.

- Channel bandwidth:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz channel bandwidth

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz channel bandwidth

Proposal 4: Consider the following scenario for BS MMSE-IRC requirement definition and discuss how to model the corresponding interference profile:

- Serving cell: DSUUU

- Neighbour cell: 7D3U

Proposal 5: Reuse the same interference modeling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbor cell PUSCH.

Proposal 6: Reuse the same interference power level as in LTE, i.e., DIP -1.11dB and -10.91dB for HomNet scenario and DIP -0.43dB and -13.78dB for HetNet scenario.

Proposal 7: Use INR-based modeling + SNR-based requirement definition criteria and transfer the DIP value into INR value.

Proposal 8: Cover 2/4/8 Rx with 1Tx.

Proposal 9: Use TDLA30-10 and TDLC300-100 propagation channel model for the serving cell for HetNet and HomNet respectively.

Proposal 10: Introduce new propagation channel for testing with desired delay spread 1000ns for the nonboring cell, i.e., TDLC1000.

Reference

RP-241297, Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO, RAN #104, June 2024.