3GPP TSG-RAN4 Meeting #112	R4-2411185

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0103 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_enh-Core) CR to TS 38.101-5: clarification of the additional requirements for n512 + additional fixes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For band n512, NS signalling was introduced, listing requirements which are specific to Region 1. But the corresponding requirements were not explicitely identified as additional requirement and the corresponding NS was not mentioned in each requirement sub-clause. Some [] are also remaining. Some notes’ format is not correct in tables. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify which are the additional regional requirements in each sub-clause and the corresponding NS. Remove [] Change some notes’ format. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When reading the specifications, reader would be confused if requirements applied to band n512 in all Regions or only in Region 1. Undefined requirements will remain. The editorial on notes’ format remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.3, 9.5.2.3, 9.5.3.2.1, 9.5.3.3.1, 9.5.3.3.4, 9.6.1.1.1, 9.6.1.2.1, 10.3.2, 10.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present Release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future Releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NTN satellite can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 7125 MHz |
| FR2-NTN (Note 2) | 17300 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

<End of the change>

<Start of the change>

### 5.2.3	Operating bands with radiated requirements

NTN satellite is designed to operate in the operating bands defined in Table 5.2.3-1.

Table 5.2.3-1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512(Note 1) | 27500 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n511(Note 2) | 28350 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n510(Note 3) | 27500 MHz - 28350 MHz | 17300 MHz - 20200 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). |  |  |  |

<End of the change>

<Start of the change>

#### 9.2.2.3	Additional regional minimum requirement for band n512 (NS_201N)

##### 9.2.2.3.1	Fixed VSAT

For co-polarized transmissions, the requirements specified in Table 9.2.2.3.1-1 apply to Fixed VSAT type 1 or 2 when transmitting towards satellites in geostationary orbit.

Table 9.2.2.3.1-1: Off-axis EIRP density limits for co-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.8° ≤ θ ≤ 7° | 49 – 25log(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 28 – K | 40 |
| 9.2° ≤ θ ≤ 48° | 52 – 25log(θ) – K | 40 |
| 48° < θ | 20 – K | 40 |
| NOTE: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Fixed VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.4.2 in [18]. The manufacturer shall declare the value of N. |  |  |

For cross-polarized transmissions, the requirements specified in Table 9.2.2.3.1-2 apply to Fixed VSAT type 1 or 2 when transmitting towards satellites in geostationary orbit.

Table 9.2.2.3.1-2: Off-axis EIRP density limits for cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.8° ≤ θ ≤ 7° | 39 – 25log(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 18 – K | 40 |
| NOTE: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Fixed VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.4.2 in [18]. The manufacturer shall declare the value of N. |  |  |

##### 9.2.2.3.2	Mobile VSAT

For co-polarized transmissions, the requirements specified in Table 9.2.2.3.2-1 apply to Mobile VSAT.

Table 9.2.2.3.2-1: Off-axis EIRP density limits for co-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 49 – 25log(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 28 – K | 40 |
| 9.2° ≤ θ ≤ 48° | 52 – 25log(θ) – K | 40 |
| 48° < θ ≤ 180° | 20 – K | 40 |
| NOTE1: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17]. The manufacturer shall declare the value of N. NOTE2: The manufacturer shall declare the operational conditions of the system e.g. motion of the platform with 6 degrees of freedom and the duration for which the limits will not be exceeded for more than 0,01% of the time. |  |  |

Mobile VSAT with low elevation angles may exceed the limits specified in Table 9.2.2.3.2-1 by the amount specified in Table 9.2.2.3.2-2.

Table 9.2.2.3.2-2: Off-axis EIRP density limits for co-polarized transmissions

| Elevation angle to Satellite (ε) | Increase in EIRP density (dB) |
| --- | --- |
| ε < 5o | 2.5 |
| 5o < ε < 30o | 3 – 0.1 * ε |

For cross-polarized transmissions, the requirements specified in Table 9.2.2.3.2-3 apply to Mobile VSAT.

Table 9.2.2.3.2-3: Off-axis EIRP density limits for cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.0° ≤ θ ≤ 7° | 39 – 25log(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 18 – K | 40 |
| NOTE 1: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one Mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17]. The manufacturer shall declare the value of N. NOTE 2: The manufacturer shall declare the operational conditions of the system e.g. motion of the platform with 6 degrees of freedom and the duration for which the requirement will not be exceeded for more than 0,01% of the time. |  |  |

##### 9.2.2.3.3	Additional Off-axis EIRP density requirements for protection of fixed services

For NTN VSAT, the requirements specified in Table 9.2.2.3.3-1 apply.

Table 9.2.2.3.3-1: Off-axis EIRP density limits for protection of fixed services

| Frequency Range (GHz) | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 27.8285—28.4445 |  |  |
| 28.8365 – 28.9485 (Note 1) | -5 | 1 |
| 28.9485 – 29.4525 |  |  |
| NOTE 1: When applicable, if this frequency range is allocated to fixed service. |  |  |

<End of the change>

<Start of the change>

#### 9.5.2.3	Adjacent channel leakage ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. ACLR requirement is specified for a scenario in which adjacent carrier is another NR channel.

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 9.5.2.3-1 for FR2-NTN.

If the measured adjacent channel power is greater than –35 dBm then the NRACLR shall be higher than the value specified in Table 9.5.2.3-1. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 9.5.2.3-1: General requirements for NRACLR for FR2-NTN

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n512, n511, n510 | 14 dB | 14 dB | 14 dB | 14 dB |
| NR channel measurement bandwidth (MHz) | 47.58 | 95.16 | 190.20 | 380.28 |
| Adjacent channel centre frequency offset (MHz) | +50 / -50 | +100 / -100 | +200 / -200 | +400 / -400 |

<End of the change>

<Start of the change>

#### 9.5.3.2	On-axis spurious requirement

##### 9.5.3.2.1	Applicability

The additional regional On-axis spurious requirement is applicable to NTN VSAT operating in band n512 (NS_201N and NS_202N). The On-axis spurious emissions are measured as EIRP.

These limits are applicable to the complete NTN VSAT equipment, including cabling between the units.

<End of the change>

<Start of the change>

#### 9.5.3.3	Off-axis spurious requirement

##### 9.5.3.3.1	Applicability

The additional regional Off-axis spurious requirement is applicable to NTN VSAT operating in band n512 (NS_201N and NS_202N). The Off-axis spurious emissions are measured as EIRP.

These limits are applicable to the complete NTN VSAT equipment, including cabling between the units.

<End of the change>

<Start of the change>

##### 9.5.3.3.4	“Carrier-on” and “Carrier-off” states

The requirements specified in table 9.5.3.3.4-1 apply to NTN VSAT in “Carrier-on” and “Carrier-off” states for all off-axis angles greater than 7° or greater than the minimum elevation angle supported, whichever is lower.

Table 9.5.3.3.4-1: Off-axis spurious limits in “Carrier-on” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 1.00 – 3.40 | -41 | 0.1 |
| 3.40 – 10.70 | -35 | 0.1 |
| 10.70 – 21.20 | -29 | 0.1 |
| 21.20 – 27.35 | -23 | 0.1 |
| 27.35 – 27.50 | -5 (Note 1) | 1 |
|  | -23 (Note 2) | 0.1 |
| 27.50 – 29.35 | -5 (Note 1) | 1 |
|  | -23 (Note 2) | 0.1 |
| 29.35 – 29.50 | -5 | 1 |
| 30.00 – 30.15 | -5 | 1 |
| 30.15 – 60.00 | -23 | 0.1 |
| NOTE 1: For mobile VSAT transmitting in the frequency range 29.5 – 30.0 GHz NOTE 2: For mobile VSAT transmitting in the frequency range 27.5 – 29.5 GHz |  |  |

<End of the change>

<Start of the change>

#### 9.6.1.1	Minimum requirements for NTN VSAT

##### 9.6.1.1.1	Applicability

Except if otherwise stated, the following additional regional requirements are applicable to NTN VSAT types 1, 2, 3, 4 or 5 operating in band n512 (NS_201N and NS_202N).

<End of the change>

<Start of the change>

#### 9.6.1.2	Minimum requirement for Fixed VSAT types 1 or 2

##### 9.6.1.2.1	Applicability

The following additional regional requirements are applicable to Fixed VSAT types 1 or 2 operating in band n512 when connected to Geostationary Satellite Orbit (GSO) SAN (NS_201N).

<End of the change>

<Start of the change>

### 10.3.2	Minimum requirement

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 10.3.2-1. EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

<End of the change>

<Start of the change>

## 10.8	Receiver antenna off-axis performance

The following additional regional requirements are applicable to NTN VSAT operating in band n512 (NS_201N).

The receiver antenna off-axis gain of each co-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8-1.

Table 10.8-1: Off-axis Co-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φmin ≤ φ ≤ 48° | 32 – 25logφ |
| 48° ≤ φ ≤ 85° | -10 |
| 85° ≤ φ ≤ 180° | 0 |
| NOTE: φmin = 1o or 100λ/D (degrees) whichever is the greater, for D/λ≥ 50. φmin = 2o or 114(D/ λ)-1.09 (degrees) whichever is the greater, for D/λ 50. where D is the nominal diameter of the antenna |  |

The receiver antenna off-axis gain of each cross-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8-2.

Table 10.8-2: Off-axis Cross-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φr ≤ φ ≤ 7° | 23 – 20logφ |
| NOTE: φr = 1o or 100λ/D (degrees) whichever is the greater where D is the nominal diameter of the antenna |  |

<End of the change>