3GPP TSG-RAN WG4 Meeting # 112	R4-2411188

# Maastricht, Netherland, August 19 – 23, 2024

Source:			Ericsson

Title:			NTN Ku-band – coexistence study

Agenda item:		8.9.2

Document for:		Discussion

# Introduction

In last RAN#104, a new WI ([1]) has been approved to specify new NTN Ku band(s). One of the WI objectives is to conduct a coexistence study, reusing as much as possible the assumptions of the coexistence study for the NTN Ka-band.

This contribution is triggering initial discussion on the coexistence study for the NTN Ku-band, identifying the missing assumptions for such study.

# Discussion

## WI objectives

The NTN Ku band WI lists the following objectives which are major input for this coexistence study:

When specifying new RF requirements for SAN/UE in a new frequency range, as it has been done for NTN and TN, RAN4 determines the SAN/UE ACLR/ACS requirements based on the outcomes of a coexistence study. Such study is assuming adjacent channel networks. RAN4 shall run such study in the context of this WI as it’s a new frequency range for which no NTN band has been specified so far. This is aligned with the second bullet of WI objectives.

Proposal1: As done for any TN/NTN band previously, and as also mentioned in the WI objectives, RAN4 shall conduct a coexistence study assuming adjacent TN/NTN networks.

## General

### New scenario

Compared to the study for the Ka-band, a new scenario has been proposed: ESIM connected to NGSO.

Observation1: ESIM connected to NGSO SAN is a new additional scenario to consider for the coexistence study for the NTN Ku band.

### Reference frequencies

The coexistence simulations should be based on reference frequencies, one for UL (Earth to Space) and one for DL (Space to Earth). For the Ka-band, RAN4 used respectively 27 and 17 GHz.

Similarly, using one frequency in the lowest UL and DL range, we would propose to use the following reference frequencies for the Ku-band coexistence study:  14 GHz for UL and 11 GHz for DL.

Proposal2: The reference frequencies to be considered in the coexistence study should be 14 GHz for UL (Earth to Space) and 11 GHz for DL (Space to Earth).

### Channel BW

According to the WI ([1]), the new NTN KU band(s) should belong either to FR1-NTN or FR2-NTN, pending on RAN4 decision. For the coexistence study, to make sure the results would be relevant for both frequency ranges, we propose to use 100 MHz channel BW for both TN and NTN as 100 MHz channel BW has been specified for both FR1-NTN and FR2-NTN. And this even if 100 MHz channel BW is not specified today for FR1-NTN.

Proposal3: Use 100 MHz channel BW for both NTN and TN network.

## Missing NTN assumptions

### Parameters

For NTN Ka band and NTN FR1 (2 GHz) coexistence studies, RAN4 used the parameters (set 1) from the NTN TR 38.821. This information is available for the reference frequencies used for the coexistence study, i.e. 2 GHz, 17 GHz and 27 GHz.

But for the Ku band, such information is not available in this TR 38.821, this is missing to run the coexistence study. Hopefully, satellite manufacturers and/or operators could provide representative values that could be used for the study.

A least, the following list of parameters is missing:

- For LEO600, LEO1200 and GEO SAN:

  - EIRP density.

  - Tx antenna max gain.

  - Rx antenna max gain.

  - Satellite noise figure (G/T)

- For VSAT UE :

  - Antenna type

  - Noise figure

  - Tx antenna max gain.

  - Rx antenna max gain.

  - UE max Tx power.

Proposal4 : RAN4 shall collect the following missing information on:

- SAN: EIRP density, Tx antenna max gain, Rx antenna max gain, G/T figure.

- VSAT UE: antenna type, noise figure, Tx antenna max gain, Rx antenna max gain, UE max Tx power.

### Propagation model

For NTN Ka band and NTN FR1 (2 GHz) coexistence studies, RAN4 used the propagation mode information from the NTN TR 38.811. This information is available for the reference frequencies used for the coexistence study, i.e. 2 GHz, 17 GHz and 27 GHz.

Again, for the Ku band, such information is not available in the TR 38.811, this is missing to run the coexistence study. Hopefully, satellite manufacturers and/or operators could provide the representative values that could be used for the study.

A least, the following information are missing:

- Shadow fading and clutter loss tables for the reference frequencies.

- Tropospheric scintillation.

Moreover, to minimize any issue during calibration, RAN4 should also align on the atmospheric absorption (based on the method of Annex 2 in ITU-R P.676 ).

Proposal5: RAN4 shall collect the following missing information on NTN propagation model to progress on the coexistence study:

- Shadow fading and clutter loss tables for the reference frequencies

- Tropospheric scintillation.

Observation2: To minimize potential issues during calibration, RAN4 should also align on the atmospheric absorption (based on the method of Annex 2 in ITU-R P.676 ).

## Missing TN assumptions

### Parameters

For TN, the following assumptions would be missing:

- Antenna parameters.

- BS/UE ACLR/ACS for the reference frequencies.

Nevertheless, RAN4 already made a study for the 10.0-10.5 GHz frequency range ([2]) and is currently investigating parameters for the 14.8-15.35 GHz frequency range to respond to ITU-R LS ([3]). We would then propose to take, as starting point, the parameters agreed for those 2 studies. Further discussion would be needed to fine tune and adapt those parameters to the 2 reference frequencies that will be used for this NTN Ku band coexistence study.

Proposal6: Use as starting point the antenna parameters and BS/UE ACLR/ACS agreed for:

- The 10.0-10.5 GHz frequency range ([2]) for the reference frequency of 11 GHz

- The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

### Channel model

RAN1 is currently running a study updating the channel model for the frequency range 7-24 GHz.

Nevertheless, to not delay the NTN Ku band coexistence study waiting for RAN1 conclusions, we would propose to use the channel models currently specified in TR 38.901 and encourage RAN4 delegates to check with their RAN1 colleagues if any major impact is expected for the 10-15 GHz frequency range.

Proposal7: For TN channel model, reuse existing version of TR 38.901.

# Conclusion

In this contribution, we initiated the discussion on NTN Ku-band coexistence study and listed the missing information to conduct such study. We made the following proposals and observations:

Proposal1: As done for any TN/NTN band previously, and as also mentioned in the WI objectives, RAN4 shall conduct a coexistence study assuming adjacent TN/NTN networks.

Observation1: ESIM connected to NGSO SAN is a new additional scenario to consider for the coexistence study for the NTN Ku band.

Proposal2: The reference frequencies to be considered in the coexistence study should be 14 GHz for UL (Earth to Space) and 11 GHz for DL (Space to Earth).

Proposal3: Use 100 MHz channel BW for both NTN and TN network.

Proposal4 : RAN4 shall collect the following missing information on:

- SAN: EIRP density, Tx antenna max gain, Rx antenna max gain, G/T figure.

- VSAT UE: antenna type, noise figure, Tx antenna max gain, Rx antenna max gain, UE max Tx power.

Proposal5: RAN4 shall collect the following missing information on NTN propagation model to progress on the coexistence study:

- Shadow fading and clutter loss tables for the reference frequencies

- Tropospheric scintillation.

Observation2: To minimize potential issues during calibration, RAN4 should also align on the atmospheric absorption (based on the method of Annex 2 in ITU-R P.676 ).

Proposal6: Use as starting point the antenna parameters and BS/UE ACLR/ACS agreed for:

- The 10.0-10.5 GHz frequency range ([2]) for the reference frequency of 11 GHz

- The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

Proposal7: For TN channel model, reuse existing version of TR 38.901.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

- TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz

- RP-240787, New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, Ericsson.

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz