3GPP TSG-RAN WG4 Meeting # 112	R4-2411189

# Maastricht, Netherland, August 19 – 23, 2024

Source:			Ericsson

Title:			NTN Ku-band – FR1-TN vs FR2-TN

Agenda item:		8.9.3

Document for:		Discussion

# Introduction

In last RAN#104, a new WI ([1]) has been approved to specify new NTN Ku band(s). One of the WI objectives is to enable 125 MHz in UL and 250 MHz, considering either FR1-TN range or FR2-TN range.

This contribution is discussing the different approaches.

# Discussion

## WI objectives

The NTN Ku band WI lists the following objectives which are major input for this discussion on FR1-NTN vs FR2-NTN:

## FR1-NTN range

### Maximum aggregated channel BW to be supported

First, it should be noted that, if 125 MHz UL should be supported via CA, it would be better to add 25 MHz to the list of channel BW to be specified for the Ku-band(s), this is actually missing from the list proposed for FR1-NTN in the WI.

Observation1: To better support 125 MHz UL aggregated, 25 MHz channel BW should also be supported in the Ku-band(s), if FR1-NTN is the frequency range supposed to host this Ku-band(s).

If 25 MHz channel BW is added, the targeted 125 MHz BW could be supported by aggregated 100 and 25 MHz channel BW. The targeted 250 MHz BW could be supported by aggregated 2 times 100 and 1 time 50 MHz channel BW.

Observation2: The targeted 125 MHz BW could be supported by aggregated 100 and 25 MHz channel BW. The targeted 250 MHz BW could be supported by aggregated 2 times 100 and 1 time 50 MHz channel BW.

### Specified channel BWs in FR1-NTN

So far, only 5, 10, 15 and 20 MHz channel BWs have been specified in the FR1-NTN range. 30 MHz was added lately but no RF requirement nor demodulation requirement has been specified for this channel BW.

If the Ku-band(s) should be considered as a FR1-NTN band(s), to address the WI objectives, RF and demodulation requirements for 35, 50, 70 and 100 MHz channel BW would have to be specified in both TS 38.108 and TS 38.101-5.

Observation3: If the Ku-band(s) is part of the FR1-NTN frequency range, RF and demodulation requirements shall be specified at least for 35, 50, 70 and 100 MHz channel BW to address the WI objective.

### NTN UE type operating in the Ku-band(s)

According to the WI objectives (2.1), the type of NTN UEs considered for the Ku-band(s) are NTN UE types 1-5, as specified in 38.101-5, handheld UEs are not considered for this band(s).

Per today, NTN UE types 1-5 are supported in bands belonging to FR2-NTN, not FR1-NTN. If the Ku-band(s) should be considered as a FR1-NTN band(s), further study would be needed to evaluate the exact impacts on existing requirements for NTN UE types 1-5.

Observation4: RF and demodulation requirements are specified for NTN UE types 1-5 when operating in bands belonging to FR2-NTN.

### SCS

It should be noted that 60kHz SCS was specified in RF specifications but it’s not fully supported in RRM, some requirements are missing for this SCS (e.g. timing error limit). If the NTN Ku-band(s) should be part of FR1-NTN, 60 kHz should be an interesting SCS value for such frequency range but this would require some RRM specifications update.

Observation5: 60kHz SCS is not supported in RRM specifications.

### FR1-NTN range extension

If the Ku-band(s) should be considered as a FR1-NTN band(s), FR1-NTN upper limit should be extended from 7125 MHz up to 14.5 GHz.

Observation6: If the Ku-band(s) should be considered as a FR1-NTN band(s), FR1-NTN upper limit should be extended from 7125 MHz up to 14.5 GHz.

## FR2-NTN range

### Maximum aggregated channel BW to be supported

If the Ku-band(s) is part of the FR2-NTN frequency range, only 50, 100, 200 and 400 MHz channel BWs are specified for that frequency range, meaning 125 MHz channel BW support, even aggregated, is not immediately possible.

A study was done in RAN4 to support “irregular” channel bandwidth, and the outcomes are captured in [2]. This conclusion of this study might be reused to address this 125 MHZ channel BW in UL, further discussion would be needed to identify the exact procedure and related impacts when applied to the Ku-band(s).

Observation7: The targeted 125 MHz aggregated channel BW might be challenging if Ku-band(s) is considered as a FR2-NTN band(s).

The targeted 250 MHz BW could be supported by aggregated 200 and 50 MHz channel BW.

Observation8: The targeted 250 MHz BW could be supported by aggregated 200 and 50 MHz channel BW.

### FR2-NTN range extension

If the Ku-band(s) should be considered as a FR2-NTN band(s), FR2-NTN lower limit should be extended from 17300 MHz down to 10700 MHz.

Observation9: If the Ku-band(s) should be considered as a FR2-NTN band(s), FR2-NTN lower limit should be extended from 17300 MHz down to 10700 MHz.

# Conclusion

In this contribution, we analyzed the impacts of considering the NTN Ku-band(s) as part of FR1-NTN or FR2-NTN. We made the following proposals and observations:

Observation1: To better support 125 MHz UL aggregated, 25 MHz channel BW should also be supported in the Ku-band(s), if FR1-NTN is the frequency range supposed to host this Ku-band(s).

Observation2: The targeted 125 MHz BW could be supported by aggregated 100 and 25 MHz channel BW. The targeted 250 MHz BW could be supported by aggregated 2 times 100 and 1 time 50 MHz channel BW.

Observation3: If the Ku-band(s) is part of the FR1-NTN frequency range, RF and demodulation requirements shall be specified at least for 35, 50, 70 and 100 MHz channel BW to address the WI objective.

Observation4: RF and demodulation requirements are specified for NTN UE types 1-5 when operating in bands belonging to FR2-NTN.

Observation5: 60kHz SCS is not supported in RRM specifications.

Observation6: If the Ku-band(s) should be considered as a FR1-NTN band(s), FR1-NTN upper limit should be extended from 7125 MHz up to 14.5 GHz.

Observation7: The targeted 125 MHz aggregated channel BW might be challenging if Ku-band(s) is considered as a FR2-NTN band.

Observation8: The targeted 250 MHz BW could be supported by aggregated 200 and 50 MHz channel BW.

Observation9: If the Ku-band(s) should be considered as a FR2-NTN band(s), FR2-NTN lower limit should be extended from 17300 MHz down to 10700 MHz.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

- TR 38.844, Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths