3GPP TSG-RAN WG4 Meeting # 112	R4-2411190

# Maastricht, Netherland, August 19 – 23, 2024

Source:			Ericsson

Title:			NTN Ku-band – Regulatory aspects and band(s) definition

Agenda item:		8.9.1

Document for:		Discussion

# Introduction

In last RAN#104, a new WI ([1]) has been approved to specify new NTN Ku band(s). One of the WI objectives is to define NTN bands to cover the different frequency ranges proposed in the WI.

This contribution is discussing the regulatory aspects related to NTN Ku-band, proposing some bands definition.

# Discussion

## WI objectives

The NTN Ku band WI lists the following objectives related to Ku-band definition:

## Region 1

In the Ku-band frequency ranges targeted in the WI, the following relevant ECC Decisions apply:

| ECC Decision | Frequency Range | Title |
| --- | --- | --- |
| ECC Decision(18)04 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use, exemption from individual licensing and free circulation and use of land based Earth Stations In-Motion (ESIM) operating with GSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0- 14.5 GHz |
| ECC Decision(18)05 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use, exemption from individual licensing and free circulation and use of Earth Stations In-Motion (ESIM) operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz |
| ECC Decision(19)04 | 10.7 - 12.75 GHz (S-to-E) 12.75 – 13.25 GHz (E-to-S) | The harmonised use of spectrum, free circulation and use of earth stations on-board aircraft operating with GSO FSS networks and NGSO FSS systems in the frequency bands 12.75-13.25 GHz (Earth-to-space) and 10.7-12.75 GHz (space-to-Earth) |
| ECC Decision(05)10 | 10.7 – 11.7 GHz and 12.5 – 12.75 GHz (S-to-E) 14.0 - 14.5 GHz (E-to-S) | The free circulation and use of Earth Stations on board Vessels operating in fixed satellite service networks in the frequency bands 14-14.5 GHz |
| ECC Decision(17)04 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use and exemption from individual licensing of fixed earth stations operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz |
| ECC Decision(03)04 | 10.7 – 11.7 GHz (S-to-E) 14.25 – 14.5 GHz (E-to-S) | Exemption from Individual Licensing of Very Small Aperture Terminals (VSAT) operating in the frequency bands 14.25-14.50 GHz Earth-to-space and 10.70-11.70 GHz space-to-Earth |
| ECC Decision(06)03 | 10.7 – 12.75 GHz (S-to-E) 14.0 – 14.25 GHz (E-to-S) | Exemption from Individual Licensing of high e.i.r.p. satellite terminals (HEST) operating with geostationary satellites and in the frequency bands 10.70-12.75 GHz or 19.70-20.20 GHz space-to-Earth and 14.00-14.25 GHz or 29.50-30.00 GHz Earth-to-space |
| ERC Decision(00)08 | 10.7 – 12.5 GHz (S-to-E) | ERC Decision of 19 October 2000 on the use of the band 10.7 – 12.5 GHz by the fixed service and Earth stations of the broadcasting-satellite and fixed-satellite service (space-to-Earth) |

It should be noted that:

- There is no ECC Decision / ERC Decision covering the frequency range 13.75-14.0 GHz: VSAT devices might operate but under license only (e.g. transmit power limit is defined according to each country license), they are not license exempt. Also, according to ITU RR (5.502), following restrictions apply:

- ESIM operations are regulated (license exempt) in the following frequency ranges:

  - 10.7 - 12.75 GHz (S-to-E) / 14.0 - 14.5 GHz (E-to-S).

  - 10.7 - 12.75 GHz (S-to-E) / 12.75 – 13.25 GHz (E-to-S) but for ESIM on board aircraft.

- The frequency ranges 12.5 – 12.75 GHz (S-to-E) / 14.0 – 14.25 GHz (E-to-S) are allocated to FSS only.

- The frequency ranges 10.7 – 11.7 GHz (S-to-E) / 14.25 – 14.5 GHz (E-to-S) are allocated to FSS and Fixed service.

Looking at the ETSI Harmonized Standards’ landscape:

- Most of the active ENs are covering 14.0 - 14.5 GHz (E-to-S).

- The only Harmonized Standard covering 12.75-13.25 GHz (E-to-S) is for news gathering transportable Earth Station. Further discussion would be needed to understand if this type of UE should be covered by RAN4.

Proposal1: For Region 1, consider a Ku-band covering 10.7 – 12.75 GHz (E-to-S) and 14.0 - 14.5 GHz (E-to-S) as starting point.

Proposal2: Consider and further discuss if this band (S-to-E) should be split in 2 bands actually, splitting the 10.7 – 12.75 GHz frequency range in two: one covering 10.7 – 11.7 GHz where FSS only is allocated and one covering 11.7 – 12.75 GHz where both FSS and Fixed are allocated.

Observation1: Any band covering the 12.75 – 13.25 GHz (E-to-S) should be further discussed, this frequency range has been regulated for free circulation and use of earth stations on-board aircraft, and it seems the only Harmonized Standard covering this frequency range is for “news gathering transportable earth station”.

## Region 2

In the following, we have focused the regulatory investigation only to Canada but this should be extended to other countries in Region 2 before finalizing the NTN Ku-bands definition.

Technical Requirements for Fixed Earth Stations operating above 1 GHz in Space Radiocommunication Services and Earth Stations in Motion (ESIMs) Operating in the Fixed-Satellite Service are specified in SRSP-101.

ISED also published its Decision on Updates to the Licensing and Fee Framework for Earth Stations and Space Stations in Canada (SMSE-008-22) in which the list of bands eligible for generic licensing is specified in table D1.

As it can be observed, Earth Station shall not create interference nor claim any protection to/from the other licensed services in the frequency range 10.7 – 10.95 GHz (S-to-E). The requirements for this frequency range would then be very stringent. To not impact the other frequency ranges, it’s most likely better to exclude this frequency range or consider it separately.

Proposal3: Do not consider the 10.7 – 10.95 GHz frequency range (S-to-E) for NTN Ku-band in Region 2 (Canada).

Also, land ESIM is not allowed in that 10.7 – 10.95 GHz frequency range, nor in the 11.2-11.45 GHz frequency range. It should be noted that land ESIM is the type operation which is covered in the coexistence study, similar to what was done for bands n511 and n510.

From those observations, for Canada at least, the following bands should be considered.

Proposal4: For Canada at least, consider 2 NTN Ku-bands:

- One continuous band covering 11.45 – 12.7 GHz (S-to-E) and 13.75 – 14.0 GHz (E-to-S) supporting fixed and mobile NTN UE.

- One continuous band covering 10.95 – 11.45 GHz (S-to-E) and 13.75 – 14.0 GHz (E-to-S) supporting fixed NTN UE only.

As mentioned earlier, other Regulations should be studied before finalizing the NTN Ku-bands definition in Region 2.

## Region 3

To be further investigated.

# Conclusion

In this contribution, we investigated the Regulations in Region 1 and in Region 2 (Canada) related to the NTN Ku-band. We made the following proposals and observations:

Proposal1: For Region 1, consider a Ku-band covering 10.7 – 12.75 GHz (E-to-S) and 14.0 - 14.5 GHz (E-to-S) as starting point.

Proposal2: Consider and further discuss if this band (S-to-E) should be split in 2 bands actually, splitting the 10.7 – 12.75 GHz frequency range in two: one covering 10.7 – 11.7 GHz where FSS only is allocated and one covering 11.7 – 12.75 GHz where both FSS and Fixed are allocated.

Observation1: Any band covering the 12.75 – 13.25 GHz (E-to-S) should be further discussed, this frequency range has been regulated for free circulation and use of earth stations on-board aircraft, and it seems the only Harmonized Standard covering this frequency range is for “news gathering transportable earth station”.

Proposal3: Do not consider the 10.7 – 10.95 GHz frequency range (S-to-E) for NTN Ku-band in Region 2 (Canada).

Proposal4: For Canada at least, consider 2 NTN Ku-bands:

- One continuous band covering 11.45 – 12.7 GHz (S-to-E) and 13.75 – 14.0 GHz (E-to-S) supporting fixed and mobile NTN UE.

- One continuous band covering 10.95 – 11.45 GHz (S-to-E) and 13.75 – 14.0 GHz (E-to-S) supporting fixed NTN UE only.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

- TR 38.844, Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths