3GPP TSG-RAN WG4 Meeting # 112	R4-2411192

# Maastricht, Netherland, August 19 – 23, 2024

Source:			Ericsson

Title:			NTN Ku-band – UE RF requirements overview

Agenda item:		8.9.4

Document for:		Discussion

# Introduction

In last RAN#104, a new WI ([1]) has been approved to specify new NTN Ku band(s).

This contribution is giving an overview of the impact on the existing UE RF requirements as specified in TS 38.101-5.

# Discussion

## General requirements

The following table list the updates needed on general requirements when introducing NTN Ku-band(s) support in TS 38.101-5.

| Requirements | Expected impact |
| --- | --- |
| 4. General |  |
| 4.1 Relationship between minimum requirements and test requirements | No impact |
| 4.2 Applicability of minimum requirements | No impact |
| 4.3 Specification suffix information | No impact |
| 4.4 Relationship with other core specifications | No impact |
| 5. Operating bands and channel arrangement |  |
| 5.1 General | One for the frequency range should be updated to consider the new NTN Ku-band(s) |
| 5.2 Operating bands | One for the 2 bands tables should be updated to consider the new NTN Ku-band(s) |
| 5.3 UE channel bandwidth |  |
| 5.3.1 General | No impact |
| 5.3.2 Transmission bandwidth configuration | Depending on which channel BWs are introduced and which frequency range is selected, one of the FR1-NTN transmission bandwidth table should be updated. |
| 5.3.3 Minimum guardband and transmission bandwidth configuration | Depending on which channel BWs are introduced and which frequency range is selected, one of the FR1-NTN minimum guard band table should be updated. |
| 5.3.4 RB alignment | No impact |
| 5.3.5 UE channel bandwidth per operating band | Depending on which channel BWs are introduced and which frequency range is selected, one of the corresponding SAN channel bandwidth table should be updated. |
| 5.3.6 Asymmetric channel bandwidths | T |
| 5.4 Channel arrangement |  |
| 5.4.1 Channel spacing | No impact |
| 5.4.2 Channel raster | The channel raster ranges should be added for the new NTN Ku-band(s) in subclause 5.4.2.3 |
| 5.4.3 Synchronization raster | The sync raster ranges should be added for the new NTN Ku-band(s) in subclause 5.4.3.3 |
| 5.4.4 TX–RX frequency separation | The UE Tx-Rx separation table should be updated of the new NTN Ku-band(s) are part for FR1-NTN. |

## Tx requirements

The WI explicitly mentions that the NTN UE operating in the Ku-band will be of type 1-5 as specified in TS 38.101-5, and not handheld UEs. The following table list the updates needed on Tx requirements when introducing NTN Ku-band(s) support in TS 38.101-5 with these types of NTN UEs.

| Requirements | Expected impacts |
| --- | --- |
| 9.1 General | No impact |
| 9.2 Transmitted power |  |
| 9.2.1 NTN VSAT maximum output power | The minimum peak EIRP and maximum output power limits should be re-evaluated for the NTN Ku-band(s). |
| 9.2.2 Off-axis EIRP emission density limit within the operating band | Those requirements shall be re-evaluated depending on the considered Regulations. |
| 9.2.3 Configured transmitted power | No impact |
| 9.3 Output power dynamics | The ON/Off time mask transient period length might be reconsidered if the NTN Ku-band(s) are part of FR1-NTN |
| 9.4 Transmitter signal quality | No impact |
| 9.5 Output RF spectrum emissions |  |
| 9.5.1 Occupied bandwidth | No impact if NTN Ku-band(s) is part of FR2-NTN |
| 9.5.2 Out of Band Emissions |  |
| 9.5.2.1 General | Some editorials if FR1-NTN, no impact if FR2-NTN |
| 9.5.2.2 Spectrum emission mask | Some editorials if FR1-NTN, no impact if FR2-NTN on the general requirement. The additional spectrum mask requirements shall be re-evaluated depending on the considered Regulations. |
| 9.5.2.3 Adjacent channel leakage ratio | The ACLR value shall be updated for the new NTN Ku-band(s) based on the ACLR value agreed in the coexistence study. |
| 9.5.3 Spurious Emissions | No impact if NTN Ku-band(s) is part of FR2-NTN |
| 9.6 Antenna pointing accuracy and performance | Those requirements shall be re-evaluated depending on the considered Regulations. |
| 9.7 Additional regional requirements indicated by NS | Those requirements shall be re-evaluated depending on the considered Regulations. |

## Rx requirements

The WI explicitly mentions that the NTN UE operating in the Ku-band will be of type 1-5 as specified in TS 38.101-5, and not handheld UEs. The following table list the updates needed on Tx requirements when introducing NTN Ku-band(s) support in TS 38.101-5 with these types of NTN UEs.

| Requirements | Expected impacts |
| --- | --- |
| 10.1 General | No impact |
| 10.2 Polarization characteristics | No impact |
| 10.3 OTA reference sensitivity level | The OTA reference sensitivity should be re-evaluated for the NTN Ku-band(s). |
| 10.4 Maximum input level | The maximum input level should be re-evaluated for the NTN Ku-band(s). |
| 10.5 Adjacent channel selectivity | The ACS value shall be updated for the new NTN Ku-band(s) based on the ACS value agreed in the coexistence study. |
| 10.6 Blocking characteristics | To be re-evaluated |
| 10.7 Spurious emissions | No impact |
| 10.8 Receiver antenna off-axis performance | Those requirements shall be re-evaluated depending on the considered Regulations. |

## Relevant regulations and Harmonized Standards for Region 1

As also listed in our other contribution [2], the following Regulations shall be taken as input for the TS 38.101-5 updates when introducing the NTN Ku-band(s):

- ECC Decision(18)04: The harmonised use, exemption from individual licensing and free circulation and use of land based Earth Stations In-Motion (ESIM) operating with GSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0- 14.5 GHz

- ECC Decision(18)05:  The harmonised use, exemption from individual licensing and free circulation and use of Earth Stations In-Motion (ESIM) operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz

- ECC Decision(19)04: The harmonised use of spectrum, free circulation and use of earth stations on-board aircraft operating with GSO FSS networks and NGSO FSS systems in the frequency bands 12.75-13.25 GHz (Earth-to-space) and 10.7-12.75 GHz (space-to-Earth)

- ECC Decision(05)10: The free circulation and use of Earth Stations on board Vessels operating in fixed satellite service networks in the frequency bands 14-14.5 GHz

- ECC Decision(17)04: The harmonised use and exemption from individual licensing of fixed earth stations operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz

- ECC Decision(03)04: Exemption from Individual Licensing of Very Small Aperture Terminals (VSAT) operating in the frequency bands 14.25-14.50 GHz Earth-to-space and 10.70-11.70 GHz space-to-Earth

- ECC Decision(06)03: Exemption from Individual Licensing of high e.i.r.p. satellite terminals (HEST) operating with geostationary satellites and in the frequency bands 10.70-12.75 GHz or 19.70-20.20 GHz space-to-Earth and 14.00-14.25 GHz or 29.50-30.00 GHz Earth-to-space

- ECC Decision(00)08: ERC Decision of 19 October 2000 on the use of the band 10.7 – 12.5 GHz by the fixed service and Earth stations of the broadcasting-satellite and fixed-satellite service (space-to-Earth

The following ETSI Harmonized Standards should also be taken into account:

- EN 301 428: Harmonised Standard for Very Small Aperture Terminal (VSAT);
Transmit-only, transmit/receive or receive-only satellite earth stations operating in the 11/12/14 GHz frequency bands covering the essential requirements of article 3.2 of Directive 2014/53/EU

- EN 302 448: Harmonised Standard for tracking Earth Stations on Trains (ESTs) operating in the 14/12 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU

- EN 302 977: Harmonised Standard for Vehicle-Mounted Earth Stations (VMES) operating in the 14/12 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU

- EN 303 980: Fixed and in-motion Earth Stations communicating with non-geostationary satellite systems (NEST) in the 11 GHz to 14 GHz frequency bands;
Harmonised Standard for access to radio spectrum

- EN 303 981: Fixed and in-motion Wide Band Earth Stations communicating with non-geostationary satellite systems (WBES) in the 11 GHz to 14 GHz frequency bands;
Harmonised Standard for access to radio spectrum

- EN 302 340: Harmonised Standard for satellite Earth Stations on board Vessels (ESVs) operating in the 11/12/14 GHz frequency bands allocated to the Fixed Satellite Service (FSS) covering the essential requirements of article 3.2 of the Directive 2014/53/EU

And the following ETSI Harmonized Standards should be checked if relevant:

- EN 301 430:  Harmonised Standard for Satellite News Gathering Transportable Earth Stations (SNG TES) operating in the 11 GHz to 12 GHz/13 GHz to 14 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU

# Conclusion

In this contribution, we made a first evaluation of the impacts on the UE RF requirements when introducing NTN Ku band in TS 38.101-5.

Proposal: Agree with the 3 above tables listing UE RF impacts when introducing new NTN Ku-band(s) support in TS 38.101-5.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

- R4-2411190, NTN Ku-band – Regulatory aspects and band(s) definition, Ericsson